3GPP TSG-RAN WG4 Meeting # 106	R4-2302170

# Athens, Greece, February 27 – March 3, 2023

Title: 	Discussions on CSI requirements for 8RX

Source: 	Huawei, HiSilicon

Agenda item:	9.6.6.1

Document for:	Discussion

Background

As per [1], many agreements for CSI requirements are reached while some issues are still left to be discussed. In this paper we provide our views on these open issues.

Discussions

CQI requirements

Following initial agreements are reached for CQI requirements:

Whether define CQI requirements for 8RX - Introduce CQI requirements with rank 4 under static channel Test setup for CQI requirements for static conditions - 40 MHz/30 kHz, 106 RBs. - CQI index for up to 64QAM. - Rank 4.

However, detailed simulation assumptions has not been discussed. We will provide our views on detailed simulation assumptions in this section.

CodebookSubsetRestriction

CQI calculation is based on selected PMI from given codebook which has always been limited to single precoding matrix by configuring RRC configuration “CodebookSubsetRestriction” in legacy CQI test. The intention is to avoid to mix CQI calculation and PMI selection. The same logic can be applied for 8RX CQI test.

Firstly, since Rank 4 has been agreed, XP 4T8R can be used for antenna configuration. That means (N1,N2)=(2,1), (O1,O2)=(4,1) can be used. If RRC parameter reportQuantity is set to “cri-RI-PMI-CQI” which is usual configuration for legacy CSI test, no restriction on i2 can be applied according to RAN 1’s agreements:

typeI-SinglePanel-codebookSubsetRestriction-i2 i2 codebook subset restriction for Type I Single-panel codebook used when reportQuantity is CRI/Ri/i1/CQI (see TS 38.214 [19] clause 5.2.2.2.1).

That means tested UE should calculate CQI based on two Precoding matrixes (i2=0,1) which mixes the purposes of CQI testing and PMI testing. Therefore, we propose to replace “cri-RI-PMI-CQI” by “cri-RI-i1-CQI” and set “two-one-TypeI-SinglePanel-Restriction” to “0b00000001” and “typeI-SinglePanel-codebookSubsetRestriction-i2” to “0x0000000000000001”. Under this condition, the PMI can be fixed to one and CQI calculation can be tested purely.

Observation 1: Restriction on i2 is only applied to “reportQuantity” setting to “CRI/Ri/i1/CQI”, which mixes the purpose of CQI testing and PMI testing if “reportQuantity” is set to “cri-RI-PMI-CQI”

Proposal 1: Consider following simulation assumptions:

- Antenna configuration: XP 4T8R

- (N1,N2)=(2,1); (O1,O2)=(4,1)

- ReportQuantity: CRI/Ri/i1/CQI

- Two-one-TypeI-SinglePanel-Restriction:  0b00000001

- TypeI-SinglePanel-codebookSubsetRestriction-i2:  0x0000000000000001

We summarize the proposed simulation assumptions in Table 2-1:

Table 2-1: Proposed simulation assumptions for 8RX CQI test

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-i1CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | TBD |  |  |  |

Proposal 2: Use simulation assumptions listed in Table 2-1 for CQI testing.

In last meeting, RAN4 agreed to introduce CQI requirements up to 64QAM. We provide our simulation results in Figure 2-1

Figure 2-1: Median CQI distribution simulation results

We propose to set lower SNR to [4,5] dB for 16QAM and higher SNR to  [10,11] dB for 64QAM for 8RX CQI testing.

Proposal 3: Set lower SNR to [4,5] dB for 16QAM and higher SNR to [10,11] dB for 64QAM for 8RX CQI testing.

PMI requirements

In last meeting, RAN4 discussed a lot on whether to introduce PMI requirements for 8RX. Based on our understanding, no enhancement for codebook is introduced for 8RX. The only difference compared to legacy PMI testing is verifying UE’s capability of PMI selection for higher rank codebook. However, it is noted that performance gain of follow PMI vs random PMI can only be observed with high antenna correlation which is not a typical configuration for high rank due to large inter-layer interference.

Proposal 4: Don’t introduce PMI requirements

RI requirements

Some companies argue that 8RX capable UE supports higher rank reporting. However, RAN4 didn’t define any dedicated RI test for 4RX UE. I.e. Only Rank 2 defined. Follow the same logic, we propose to not define RI requirements for 8RX UE.

Proposal 5: Don’t define RI requirements

Conclusion

In this paper, we provide our views on CSI requirements for 8RX UE. The proposals and observations are:

Observation 1: Restriction on i2 is only applied to “reportQuantity” setting to “CRI/Ri/i1/CQI”, which mixes the purpose of CQI testing and PMI testing if “reportQuantity” is set to “cri-RI-PMI-CQI”

Proposal 1: Consider following simulation assumptions:

- Antenna configuration: XP 4T8R

- (N1,N2)=(2,1); (O1,O2)=(4,1)

- ReportQuantity: CRI/Ri/i1/CQI

- Two-one-TypeI-SinglePanel-Restriction:  0b00000001

- TypeI-SinglePanel-codebookSubsetRestriction-i2:  0x0000000000000001

Proposal 2: Use simulation assumptions listed in Table 2-1 for CQI testing.

Proposal 3: Set lower SNR to [4,5] dB for 16QAM and higher SNR to [10,11] dB for 64QAM for 8RX CQI testing.

Proposal 4: Don’t introduce PMI requirements

Proposal 5: Don’t define RI requirements

- 4   Reference

- RP-220613 WF on 8RX demodulation and CSI requirements.  Huawei, HiSilicon