3GPP TSG-RAN WG4 Meeting #106	R4-2302326

# Athens,  February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR | 0027 | rev |  | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.175: Corrections in clause 1 Scope and clause 9 Immunity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | These corrections were discussed and approved for the draft specifications TS 38.114 NR Repeater (R4-2117585). This CR proposes similar changes in TS 38.175. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a statement in the scope to clarify that technical specifications related to the antenna and TAB connectors are not considered in the present document. - Rewrite the references to EU EMC regulations to show where the level of 10 V/m (instead of spatial exclusion) is from. - Revise a note about “intrusive method” as this term is not defined and in fact the IEC spec mention “clamp injection” instead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusing statements in the specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 9.2.2, 9.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------- start of changes----------

# 1	Scope

The present document covers the assessment for NR Integrated access and backhaul (IAB) node and associated ancillary equipment in respect of Electromagnetic Compatibility (EMC).

The present document specifies the applicable test conditions, performance assessment and performance criteria for NR Integrated access and backhaul (IAB) node and associated ancillary equipment.

Technical requirements related to the antenna and TAB connectors are not included in the present document. These are found in the relevant product standards [2, 24, 25].

The environment classification used in the present document refers to the residential, commercial and light industrial environment classification used in IEC 61000-6-1 [4] and IEC 61000-6-3 [5].

The EMC requirements have been selected to ensure an adequate level of compatibility for apparatus at residential, commercial and light industrial environments. The levels, however, do not cover extreme cases which may occur in any location but with low probability of occurrence.

# 2	References

---------- end of changes----------

---------- start of changes----------

### 9.2.2	Test method and level

The test method shall be in accordance with IEC 61000-4-3 [13], which specifies test methodology based on anechoic chamber. The use of reverberation chamber test method according to IEC 61000-4-21 [18], clause 6.1 and Annex D as alternative method is allowed.

The following requirements shall apply:

-	The test level shall be 3 V/m amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 1 % of the momentary frequency;

-	The test shall be performed over the frequency range 80 MHz  - 6000 MHz; with the exception of the exclusion band for receivers (see clause 4.4);

-	Responses in stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies selected during the test shall be recorded in the test report.

-	For the test method in accordance with IEC 61000-4-3[13], the spatial exclusion zone can be chosen to protect the IAB node receiver(s). For the frequency range above 690 MHz (according to ETSI EN 301 489-50 [28]), the EMC RF electromagnetic field immunity requirement with a level of 10 V/m applies on the non-radiating faces of the IAB type 1-O, or IAB type 2-O, as depicted on figures 9.2.2-1.

NOTE:	Depending on the IAB implementation, application of the spatial exclusion to all radiating faces of the IAB may not allow proper execution of the RI testing. In such cases as depicted on figures 9.2.2-2, to protect the IAB node receiver(s), exclusion bands shall be considered, as in table 4.4.1-1.

Figure 9.2.2-1: Example of the spatial exclusion application in case of a single instance of the IAB radiating face (horizontal plane depicted)

Figure 9.2.2-2: Example of the spatial exclusion application in case of a 3-panel IAB node (horizontal plane depicted) – proper execution of the RI test may not be feasible

---------- end of changes----------

---------- start of changes----------

## 9.5	RF common mode (0.15 MHz - 80 MHz)

The test shall be performed on AC mains power input/output ports.

This test shall be performed on signal ports, telecommunication ports, control and DC power input/output ports, which may have cables longer than 3 m.

Where this test is not carried out on a port or any other ports because the manufacturer declares that it is not intended to be used with cables longer than stated above, a list of ports which were not tested shall be included in the test report.

This test shall be performed on a representative configuration of the equipment, the associated ancillary equipment, or representative configuration of the combination of radio and ancillary equipment.

NOTE:	This test can also be performed using the clamp injection method, where appropriate, see IEC 61000-4-6 [16].

----------end of changes----------