3GPP TSG-RAN WG4 Meeting #106 	R4-2302242

Athens, Greece, February 27 – March 3, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on LTM general aspects and scenarios

Agenda item:	9.23.3.1

Document for:	Discussion

Introduction

This paper discusses RAN4 aspects of L1/L2 Triggered mobility. Since the #105 in Toulouse, there are a number of open issues (R4-2220403)  around LTM which are being discussed by RAN4.

Discussion

LTM Interruption delay scenarios

Currently LTM delay discussion has been delay components based on L3 Handover. As one of the purposes of LTM is to support lower layer mobility, the scenarios should also consider beam switch delay, in addition to L3 delay components. Therefore, we propose to consider two main scenarios from which one is addressing the lower end of the delay, and other one is addressing the higher end of the delay. Both of the main scenarios can be further refined with details of the delay components. Scenario 1 considers Active TCI state switch delay for LTM, and the second one is scenario where the interruption delay is less than L3 HO.  LTM delay components should be defined in a way that allows requirements to be specified flexibly in between these two scenarios. Therefore, at least the following LTM scenarios should be supported.

- Scenario 1: Active TCI state switch for LTM

- Scenario 2: Interruption time less than L3 HO

RAN4 to define at least two interruption delay scenarios. Scenario 1: Active TCI state switch for LTM; Scenario 2: Interruption time less than L3 HO

Simultaneous Data Rx/Tx

Issue 1-2-1: Whether to consider simultaneous data Rx/Tx with both source cell and target cell during cell switch delay.

Supporting simultaneous data Rx/Tx would need UE to either support DAPS or multi-Rx / (multi-Tx) capabilities. As discussed in RAN4 #104e and #104e-bis, DAPS-based solution for simultaneous data Rx/Tx would come with the price of high UE complexity. Furthermore, it is not recommended to mix ongoing RAN4 work items (NR_Mob_Enh2 and Multi-Rx). Therefore, we propose that within the context of Rel-18, LTM work can focus on the scenario where source and target cell data transmission have an interruption delay starting from LTM Cell switch command.

Deprioritize DAPS based simultaneous data Rx/Tx with both source cell and target cell during LTM switch.

LTM Intra-frequency and Inter-frequency support

Issue 1-3-1: Whether to cover inter-frequency cell switch

In our last meeting, we have clarified the definition of intra/inter-frequency measurement. In RAN1, it is also agreed that:

RAN1 agreement: For candidate cell measurement for Rel-18 LTM, SSB based L1-RSRP is supported for intra-frequency measurement SSB based L1-RSRP is supported for inter-frequency measurement from RAN1 point of view FFS: L1-SINR, CSI-RS based L1-RSRP

Inter-frequency measurements are important scenario, and it should be supported. However, to progress the discussion on various open design aspects of LTM, we should first focus on resolving the LTM requirements for intra-frequency scenario. Therefore, for now RAN4 should prioritize the discussion of intra-frequency related topic and move to inter-frequency after intra-frequency is clear.

One of the key objectives of LTM is to minimise the interruption delay (in specific scenarios).

Intra-frequency LTM interruption delay is expected to be lower than inter-frequency delay. Therefore, RAN4 should focus on defining scenarios and conditions where interruption delay can be minimised.

RAN4 to focus first on intra-frequency LTM scenarios and subsequently define inter-frequency LTM scenarios.

Issue 1-3-2: Definition of inter-frequency cell switch

Even if intra-frequency design is prioritized over inter-frequency design, in order to be consistent with the definition of inter-frequency measurements, LTM should follow the existing RAN4 definitions. We propose the definition of inter-frequency switch as the SSB of the target cell is of different frequency layer and of different sub carrier spacing as that of the source cell.

Inter-frequency cell switch is defined as the switch from the source cell to target cell in which the SSB of the source cell is of different frequency layer and of different sub carrier spacing as that of the target cell.

Synchronous vs. non-synchronous definition

The last meeting way forward (R4-2220403) captures the following issues:

Issue 1-4-1: Whether to consider RTD of serving cell and neighbour cell larger than one CP for intra-frequencyL1-RSRP measurement < Wayforward >: FFS the following options - Option 1 (MTK, Xiaomi, OPPO, vivo): Start the discussion from RTD of serving cell and neighbour cell within one CP for SSB based L1-RSRP measurement. FFS impact to UE complexity, measurement delay and interruptions for RTD>CP. - Option 2 (Intel, Ericsson, QC): No need to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement - Option 3 (Apple): FFS after L1 measurement procedure become clearer and more stable. - Option 4 (CATT, vivo): depends on UE implementation - FFS: Whether to relax the RTD< CP restriction can be an optional capability of UE. - Option 5(Huawei): For SSB based L1-RSRP, discuss whether Rx time difference between serving cell and non serving cell is larger than [x]us. Whether UE supports out of [x]us depends on UE capability.

In the last meeting, it was discussed whether “RTD of serving cell and neighbour cell larger than one CP” is restricted to a case where RTD is smaller than one CP. In FR2 scenarios, the propagation timing difference can be very short and this type of scenario would limit the LTM deployment scenarios. Therefore, from network point of view it is not desirable to have this limitation.

No need to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement

Conclusion

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 to define at least two interruption delay scenarios. Scenario 1: Active TCI state switch for LTM; Scenario 2: Interruption time less than L3 HO

Proposal 2: Deprioritize DAPS based simultaneous data Rx/Tx with both source cell and target cell during LTM switch.

Observation 1: One of the key objectives of LTM is to minimise the interruption delay (in specific scenarios).

Observation 2: Intra-frequency LTM interruption delay is expected to be lower than inter-frequency delay. Therefore, RAN4 should focus on defining scenarios and conditions where interruption delay can be minimised.

Proposal 3: RAN4 to focus first on intra-frequency LTM scenarios and subsequently define inter-frequency LTM scenarios.

Proposal 4: Inter-frequency cell switch is defined as the switch from the source cell to target cell in which the SSB of the source cell is of different frequency layer and of different sub carrier spacing as that of the target cell.

Proposal 5: No need to restrict the RTD between serving cell and neighbour cell to be within CP for SSB-based L1-RSRP measurement