3GPP TSG-RAN WG4 Meeting#106	R4-2302351

Athen, Greece,  27th Feb – 3rd Mar, 2023

Source: 	Qualcomm Incorporated

Title: 	NTN NB-IoT/eMTC demodulation performance requirements

Agenda item:	 10.5.7.1

Document for:	Discussion

- Introduction

In this paper, the demodulation performance requirements in NTN mode of both NBIoT and eMTC are discussed based on discussion in [1].

- NBIoT NTN Scenarios

Scenarios in the following table were considered in [3]. All numbers provided here are with zero frequency error.

Table 2.1

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR in dB @ 70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 (Table 8.12.1.1.2-2 Test 1) | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | -8.8 |
| 1 (Table 8.12.1.1.2-2 Test 1) | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-30 | 1x1 | 8ms | -8.6 |

Proposal 1: Propose SNR numbers in the Table 2.1 for alignment purpose

Proposal 2: Repetition factor 64 shall be discussed for scenario in Table 2.2 for simulation purposes

Table 2.2

| 2 (Table 8.12.1.1.2-2 Test 2) | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 256 64 | NTN-TDLA100-10 | 1x1 | 8ms |
| --- | --- | --- | --- | --- | --- | --- | --- |

Repetition factor of 256 is likely to yield very optimistic results in the very low SNR regime. Moreover, with limited duration of satellite horizon in LEO, and  we are of the view to have a lower repetition factor.

- eMTC Scenarios

Proposal 3: Propose SNR numbers in the Table 3.1 for alignment purpose

Table 3.1

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | Offset | SNR in dB @ 70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (Table 8.11.1.1.2.1-1 Test 1) | FDD 1.4MHz | QPSK 1/3 (Reuse R.80 FDD) | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | -4.5 |
| (Table 8.11.1.1.2.1-1 Test 1) | FDD 1.4MHz | QPSK 1/3 (Reuse R.80 FDD) | TM1 | 8 | NTN-TDLA100-10 | 1x1 | 8ms | -2.8 |

- General Assumption

Proposal 4: RAN4 to discuss the frequency drift mode

- Conclusions

Following is the summary of the proposals.

Proposal 1: Proposed SNR numbers in Table 2.1 for alignment purpose

Proposal 2: Repetition factor 64 shall be discussed for scenario in Table 2.2 for simulation purposes

Proposal 3: Proposed SNR numbers in Table 3.1 for alignment purpose

# References

[1] R4-2220278, “WF for IoT over NTN UE demodulation requirements”.

[2] 3GPP WID “New WID on NB-IoT/eMTC core & performance requirements for NTN,” 3GPP TSG RAN meeting#95e, online, March 17 – 23, 2022.

[3] 3GPP TS 36.101 version 17.6.0 “LTE; E-UTRA User Equipment (UE) radio transmission and reception”.