3GPP TSG-RAN WG4 Meeting #106	R4-2302356

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.24.2.1

Source:	MediaTek Inc.

Title:	Discussion on the general aspects of MUSIM gaps

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. Based on which, UE can be configured with up to four MUSIM gaps. However, the current design of MUSIM gaps does not include any priority. In the last meeting, RAN4 has made the agreement on the necessity of introducing priority levels for MUSIM gaps to handle the collisions as captured in the WF [1]. In addition, RAN4 agreed on the necessity of allowing the UE to assist NW A in configuring these priority levels of MUSIM gaps since only the UE knows what activities to be monitored in NW B within these gaps. Therefore, in this paper, we discuss sending an LS to RAN2 on the achieved agreements in RAN4.

# Discussion

In the last meeting, RAN4 agreed to postpone the LS to RAN2 on MUSIM gap agreements and to send it in February meeting instead. Therefore, in this paper we suggest sending the LS regarding latest RAN4 agreements which will give sufficient time for RAN2 to apply these agreements on MUSIM gaps design.

Proposal 1: RAN4 to agree on the following LS to RAN2:

- Overall Description:

RAN4 has discussed and achieved the following agreements on priority for MUSIM gaps:

- Introduction of priority levels for MUSIM gaps

  - Each periodic MUSIM gap shall be assigned with a unique priority via gapPriority

  - FSS for aperiodic MUSIM gap

- When requesting MUSIM gap(s), UE can provide assistance-information for MUSIM gap priorities

  - UE can indicate gapPriority of MSUIM gap(s) to NW A.

- After receiving UE’s assistance information for MUSIM gap priorities, NW A may modify the priority levels of MUSIM gaps (e.g., for joint consideration of measurement gap and MUSUM gaps), but is not expected to change the relative priority order among MUSIM gaps

- Action:

RAN4 respectfully asks RAN2 to take the above information into account and design corresponding signalling in their future work.

# Summary

In this contribution, the following proposal is concluded:

Proposal 1: RAN4 to agree on the following LS to RAN2:

- Overall Description:

RAN4 has discussed and achieved the following agreements on priority for MUSIM gaps:

- Introduction of priority levels for MUSIM gaps

  - Each periodic MUSIM gap shall be assigned with a unique priority via gapPriority

  - FSS for aperiodic MUSIM gap

- When requesting MUSIM gap(s), UE can provide assistance-information for MUSIM gap priorities

  - UE can indicate gapPriority of MSUIM gap(s) to NW A.

- After receiving UE’s assistance information for MUSIM gap priorities, NW A may modify the priority levels of MUSIM gaps (e.g., for joint consideration of measurement gap and MUSUM gaps), but is not expected to change the relative priority order among MUSIM gaps

- Action:

RAN4 respectfully asks RAN2 to take the above information into account and design corresponding signalling in their future work.

# References

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo.