# 3GPP TSG-RAN WG4 Meeting # 106						R4-2302431

# Athens, 27th Feb –3rd March, 2023

Source:		Ericsson

Title:			A-MPR simulation for NS-23 for B256

Agenda item:		10.5.4

Document for:		Approval

# Introduction

In this contribution, we present our simulation on the A-MPR for NS_24 for protection of B34.

# Discussion

## Specification

Currently the A-MPR for Cat-M1 UE is specified in TS 36.102 as below.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | [Table 5.3A-1] | [3.5] |

The requirement is quoted below

##### 6.5A.4.4.3 Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3-1.

Table 6.5A.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

This NS-24 is also required within a frequency range greater than 1980 according to note 4 in TS 36.101:

NOTE 4:	Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz.

So it is not clear if the 3.5 A-MPR would be needed at the whole frequency range > 1980 MHz or not. Thus there is a need to make some simulation.

## Simulation assumption

A-MPR assumptions are listed below.

- 0 dBi UE antenna gain

- 4 dB post-PA losses

- PA calibrated at 1 dB MPR for -30 dBc EUTRA ACLR with 5 MHz DFT-s-OFDM 100 RB0 waveform

- Image/carrier leakage = -25 dBc for LTE

- -60 dBc CIM3 and -70 dBc CIM5

Figure 1: Backoff power needed to meet NS-24 for Cat-M1 for integer allocation.

Figure 1 illustrates the A-MPR needed for a Cat-M1 device to meet the NS-24 requirement. The edge Fc is set 2010 – 5 – 1.4/2 = 2004.3 MHz considering the 5 MHz guard band to the B34 lower frequency edge (2010MHz). It can be observed that at Fc =2003.5MHz, there is no A-MPR needed. Thus a A-MPR table is proposed as follows:

Table x.y.z: A-MPR for "NS_24" for Cat-M1

| Channel Bandwidth [MHz] | Parameters |  |
| --- | --- | --- |
| 1.4 | Fc [MHz] | Fc > [2003.5] and Fc <= [2004.3] |
|  | RBstart | 0 -6 |
|  | LCRB [RBs] | 0 - 1 |
|  | A-MPR [dB] | ≤ 1 |

Consider the above new A-MPR in TS 36.102 for Cat-M1 UE for NS-24

# Conclusions

In this contribution, we present our simulation on the A-MPR for NS_24N for protection of B34 with below proposal:

- Consider the above new A-MPR in TS 36.102 for Cat-M1 UE for NS-24

# References

TS 36.102