3GPP TSG-RAN WG4 Meeting #106                           		R4-2302471

# Athens, Greece, February 27 – March 3, 2022

Source: 	Huawei, HiSilicon

Title: 	Initial discussion on the regulatory requirements for cab-radio operating in RMR bands n100 and n101

Agenda Item:	8.17.2

Document for:	Discussion

# Introduction

CEPT/ECC [2] agreed on regulatory requirements which allocate the spectrum ranges in 900MHz and 1900MHz for Rail Mobile Radio. This regulation was based on the technical studies and co-existence evaluations for 900MHz and 1900MHz frequency bands [3, 4], as well as compatibility studies among the RMR and MFCN [5].

During Rel-17, n100 and n101 bands were defined based on this regulation.

In this contribution we provide initial discussion on the regulatory requirements for CAB-radio operating in RMR n100/n101 bands.

# Discussion

Referring to the ECC Decision (20)02 [2], the CAB-radio is defined as an RMR terminal installed on-board the train capable of supporting voice and data applications (e.g. European Train Control System).

Figure 1 FRMCS base station and CAB-radio on-board equipment [4]

Based on the WID in [1], the objective is to introduce the use of UE output power of 31 dBm for the CAB-radio type terminals, applicable for frequency band n100 (FDD) and n101 (TDD), which were already specified in Rel-17 based on the above referred ECC Decision (20)02 [2].

| n100 | 874.4 MHz – 880 MHz | 919.4 MHz – 925 MHz | FDD |
| --- | --- | --- | --- |
| n101 | 1900 MHz – 1910 MHz | 1900 MHz – 1910 MHz | TDD |

For the use of CAB-radios in a HPUE context, ECC Decision (20)02 allows a maximum output power of up to 31dBm, while Rel-17 work has specified PC3 for smartphone device power class already:

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n100 |  |  |  |  |  |  | 23 | ±2 |
| n101 |  |  |  |  |  |  | 23 | ±2 |

# CAB-radio regulations for n100

Referring to annex A2.2 of [2], the following technical conditions were defined for RMR CAB-radios operating in band n100:

Table 1: CAB-radio regulations for n100 [2]

Below we provide brief analysis:

- Maximum output power

The n100 CAB-radio maximum output power of up to 31 dBm limit in [2] is aligned with the PC1 class output power limit, as well as PC1 intended use (i.e. not a smartphone form factor; at least some PC1 terminals intended for public safety type applications).

The above regulation defines the power limit as “higher than 23dBm and up to 31dBm” (as comparison, please refer to the n101 regulation which simply defined the output power limit as “31dBm” without any range). While the first approach may be to simply associate it with the PC1, one may also think if the other power classes shall be also specified (e.g. PC2 with 26dBm limit). According to our understanding, the intention of the source regulation is to define PC1 only.

- ACLR

The n100 CAB-radio ACLR requirement in [2] is aligned with the PC1 ACLR currently specified in NR specification:

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB1 | 31 dB | 31 dB | 30 dB |

It shall be noted that there is n100 related work under separate work item on “NR support for dedicated spectrum less than 5MHz for FR1” [6]. Related objective are listed below:

In case new narrower channel bandwidth is introduced for n100, the ACLR requirement section in TS 38.101-1 will have to be updated accordingly. This is assumed to be covered by the NR_FR1_lessthan_5MHz_BW work item [6].

- UL power control mechanism

UL power control mechanism is supported by NR spec and does not require additional RAN4 requirements.

Based on the above discussion, the following proposals were formulated:

Proposal 1: Define PC1 power class for n100 CAB-radio UE.

# CAB-radio regulations for n101

Referring to annex A2.2 of [2], the following technical conditions were defined for RMR CAB-radios operating in band n100:

Table 2: CAB-radio regulations for n101 [2]

Below we provide brief analysis:

- Maximum output power

Similar to n100 case, the n101 CAB-radio maximum output power of up to 31 dBm limit in [2] is aligned with the PC1 class output power limit, as well as PC1 intended use (i.e. not a smartphone form factor; at least some PC1 terminals intended for public safety type applications).

- ACLR

Same as for n100 case, the n100 CAB-radio ACLR requirement in [2] is aligned with the PC1 ACLR currently specified in NR specification.

- UL power control mechanism

UL power control mechanism is supported by NR spec and does not require additional RAN4 requirements.

- Unwanted emissions mask

There is additional unwanted emissions mask requirement for the CAB-radio operating in n101, to protect 1920-1980 MHz frequency range:

Based on the above discussion, the following proposals were formulated:

Proposal 2: Define PC1 power class for n101 CAB-radio UE.

Proposal 3: investigate introduction of new NS signalling to implement protection of the 1920-1980 MHz frequency range from the n101 CAB-radio UE.

# Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Define PC1 power class for n100 CAB-radio UE.

Proposal 2: Define PC1 power class for n101 CAB-radio UE.

Proposal 3: investigate introduction of new NS signalling to implement protection of the 1920-1980 MHz frequency range from the n101 CAB-radio UE.

# References

[1]	RP-221878, -radio - High Power UE support for band n100 and n101 for Rail Mobile Radio (RMR) in Europe, WID

[2]	ECC Decision (20)02, https://docdb.cept.org/document/16736

[3]	ECC Report 313, Technical study for co-existence between RMR in the 900 MHz range and other applications in adjacent bands

[4]	ECC Report 314, Co-existence between Future Railway Mobile Communication System (FRMCS) in the frequency range 1900-1920 MHz and other applications in adjacent bands

[5] 	ECC Report 318, Compatibility between RMR and MFCN in the 900 MHz range, the 1900-1920 MHz band and the 2290-2300 MHz band

[6]	RP-222645, NR support for dedicated spectrum less than 5MHz for FR1, NR_FR1_lessthan_5MHz_BW, WID