3GPP TSG-RAN4 Meeting #106	R4-2302520

Athens, Greece, 27th February – 3rd March 2023

Agenda item:	9.5.2

Source:	Keysight Technologies

Title:		Testing Considerations for Multi AoA Rx Testing

Document for:	Approval

# Introduction

This contribution outlines our views on open test system aspects of multi-AoA Rx testing following the decision in the last meeting to focus on Measurement Setup Option 2a.

# Discussion

In RAN4#105, an important decision in terms of the measurement setup was reached, i.e., Setup 2a [1] was endorsed [2].

- Agreement: - RAN4 to consider option 2a as the starting point and revisit if option 2a could not verify the UE RF requirements properly.

This contribution addresses various open topics that were discussed last meeting without any conclusion.

# Maximum Angular Separation between Probes

Some discussions were held in RAN4#104bis-e on the minimum and maximum AoA separations [13][14]

Issue 1-2-3: AoA angular separations for UE RF testing - Agreement: - Companies can share the views on the possible min angular separation for the potential solutions listed in Issue 1-2-1 from testability point of view. The target is to consider a wide range of AoA angular separations between AoA1 and AoA2.

with more discussions held in RAN4#105 [1]. Consensus was reached with respect to the minimum separation between probes, i.e., a minimum of 30° separation was agreed [2].

Issue 1-2-3: Minimum AoA angular separation for UE RF testing ... - Agreement: - 30º for option 2a based on the limitation of test system capability.

However, no consensus was reached on the maximum angular separation between probes. Even though three TE vendors voiced feasibility concerns about a maximum of 180° separation during the meeting, the feasibility of a 180° separation was considered FFS [2].

Issue 1-2-4: Maximum AoA angular separation for UE RF testing - Agreement: - From test system feasibility aspect, 150º confirmed feasible - FFS on the feasibility of supporting values beyond 150º (up to 180º) which subject to further request from RF core requirement discussion)

The maximum angular separation between probes could be as large as 180°. However, this angular separation would result in significant blocking between one AoA probe and the DUT positioner, as illustrated in Figure 1, and the line-of-sight path between both probes also introduce a significant amount of coupling. It is therefore suggested to limit the maximum angular separation between probes/AoAs to 150° (blue reflector/probe in Figure 1).

Figure 1: Illustration of maximum proposed angular separation including blocking for angular separation of 180°.

Additionally, placing two reflectors opposite to each other will yield higher variations in field/power measurements within the QZ caused by reflections from one reflector facing another head on. As illustrated in Figure 2, the two dark grey reflectors with 180° separation illuminate each other with plane waves, i.e., without much attenuation; scattering from the metal surfaces including edges of the reflector will thus reflect energy back towards the source probe. On the other hand, the cross coupling between the reflector at 0° and the red or blue reflectors is minimized as they are not in each other’s direct path. It was argued that the cross coupling between the two opposing reflectors should be insignificant given the large separation and thus large path losses. However, the reflectors are in each other’s collimated beam’s path, i.e., signal from the right reflector arriving at the left reflector should not have experienced much of an attenuation when compared to the signal at the QZ. Additionally, if that argument about the large attenuation causing no effect, mm-wave systems would not require any absorber panels. However, in order to achieve the QoQZ MUs considered for mm-wave conformance testing [3][4], absorbers are certainly a requirement.

It should furthermore be highlighted that 2 AoA RRM systems currently limit the maximum separation between probes to 150° [5], i.e.,

Table A.3.15.3-1: Set of relative angular offsets between active probes for each power class UE Power class; Relative angular offset between active probes. 1; FFS. 2; FFS. 3; 30°, 60°, 90°, 120° and 150°. 4; FFS. 5; FFS. 7; FFS.

Requiring a 180° separation for multi-AoA Rx testing would therefore not allow a re-use of existing 2 AoA RRM systems as they would not be able to support the larger separation.

Figure 2: Illustration of maximum proposed angular separation including blocking for angular separation of 180°.

Proposal 1: Limit the maximum angular separation between probes/AoAs to 150° to avoid blocking effects, to minimize power variations inside the QZ, and allow the re-use/augmentation of existing 2 AoA RRM systems for multi-AoA UE RF testing.

# Impact of Probe Locations and 3D Scans on DL Directions

In RAN4#105, an important decision was made with respect to absolute probe locations [2], i.e.,

Issue 1-2-2: Probe locations for UE RF testing - Proposals - Option 1 (Keysight): For measurement setup option 2a, absolute probe locations must be defined to guarantee different system vendors yield the same UE RF test results - … - Agreement: Option 1 is agreed

Justification for this proposal/agreement can be found in [1] and will not be repeated here other than the relevant observations/proposals

Observation 1: Fixed AoA2 probes yield different DL directions perceived by the DUT. Proposal 2: Multi-AoA measurement setups for UE RF utilizing fixed probe locations during the testing must have the absolute probe directions/locations defined to guarantee that the same test parametric results are obtained between different system vendors

Some important probe location aspects outlined in [1] will be repeated here together with additional analyses as no conclusion was reached in RAN4#105. A full 3D scan, typically required for beam peak searches, spherical coverage test cases and TRP measurements, can be performed in various ways as illustrated in Table 1. Here, a single-probe, combined-axes OTA system with the probe aligned in the z direction is shown with two different methodologies to achieve a full 3D UE rotation/scan:

- full rotation in f (around z axis) and half rotation in q (around y axis) as shown in left column

- full rotation in q (around y axis) and half rotation in f (around z axis) as shown in right column

As shown in the table for the constant-step size grid with 312 grid points (Dq=Df=15°, 266 of which are unique due to the redundant points at the poles) on top or the constant-density grid with 200 grid points on the bottom, either approach will yield the same test point coverage.

Table 1: 3D scan implementations and resulting coverage using single-probe system

|  | Single-probe system configuration with 3D scan using full rotation in f and half rotation in q | Single-probe system configuration with 3D scan using full rotation in q and half rotation in f |
| --- | --- | --- |
| System Configuration |  |  |
| Constant-Step Size Grid with 312 grid points (Dq=Df=15°); 266 unique grid points |  |  |
| Constant Density Grid with 200 grid points |  |  |

Observation 1: For single-probe OTA systems, different 3D UE rotation/scan options, e.g., full rotation in f (around z axis) and half rotation in q (around y axis) or full rotation in q (around y axis) and half rotation in f (around z axis), will yield the same test point coverage.

While it was agreed to define absolute probe locations, different 2D planes containing the various AoA probes were shown to have vastly different effects on the perceived DL directions by the UE. For simplicity, sample test systems with just 4 total probes (1 AoA1 probe and 3 AoA2 probes) are considered next. Here, the AoA2 probes are displaced by 60° (P60), 120° (P120), and 150° (P150) from the AoA1 probe placed along the z direction (P0). As illustrated in Table 2, one system configuration has the AoA2 probes aligned in the xz plane (left), and another system configuration has the AoA2 probes aligned in the yz plane (right). Aside from studying the effect of different probe locations on the perceived DL directions, the two different 3D approaches are investigated as well, i.e., by rotating the device a full turn in f (rotation axis z) and a half turn in q (rotation axis y) as shown the top of Table 2 and alternatively, a full turn in q (rotation axis y) and a half turn in f (rotation axis z) as shown the bottom of Table 2.

The following analyses for constant step-size grids take into account that multiple measurements at the pole for P0 are performed; while these measurements yield unique DL directions for P0, they translate to non-unique DL directions for P60, P120, and P150.

Table 2: System configurations with probes in the xz plane (left) and yz plane (right). 3D scan performed with full rotation in f and half rotation in q (top) and with full rotation in q and half rotation in f (bottom)

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| 3D Scan: full rotation in f and half rotation in q |  |  |
| 3D Scan: full rotation in q and half rotation in f |  |  |

The system configurations with full rotation in  and half rotation in  and the perceived DL directions are further illustrated in Table 3 for a constant-step size grid with 312 grid points and in Table 4 for a constant density grid with 200 grid points. Similarly, the system configurations with full rotation in q and half rotation in f and the perceived DL directions are further illustrated in Table 5 for a constant-step size grid with 312 grid points and in Table 6 for a constant density grid with 200 grid points. The following observations can be made from these results.

Observation 2: Probes aligned in the yz plane introduce large keep-out areas for the angular coverage/DL directions towards each pole.

Observation 3: With full rotation in f and half rotation in q, probes aligned in the xz plane introduce a conical sector towards the pole at q=0° with no angular coverage/DL directions for offset probes.

Observation 4: Constant density grids do not maintain the constant density coverage/DL direction distribution for probes offset from P0, a higher density of DL directions is observed especially near the poles.

Observation 5: When probes are aligned in the xz plane, constant-step size grids maintain the constant-step size distribution of DL directions for probes offset from P0.

Observation 6: Probes aligned in the xz plane generally provide a wider angular coverage for AoA2 DL directions when compared to probes aligned in the yz plane.

Observation 7: With full rotation in q and half rotation in f and probes aligned in the xz plane, the AoA2 DL directions are distributed in 3D without a conical region with lack of coverage.

Observation 8: Only with full rotation in q and half rotation in f, probes aligned in the xz plane, and constant-step size grids, uniform angular coverage/DL directions is achieved, i.e., the distribution of DL directions for all probes on the xz plane match the distribution of grid points.

The DL polarizations received from the DUT are investigated in Annex A. The same observations as above can be made using the illustrations using a coarser grid. Additionally, the following observation can be made

Observation 9: When the AoA2 probes are placed in the xz plane, probe antenna DL q/f polarizations map to the same DUT q/f polarizations, while when AoA2 probes are placed in the yz plane, probe antenna DL q/f polarizations generally map to a combination of DUT q/f polarizations.

The AoA1-AoA2 DL orientation vectors, i.e., the vectors between the DL orientation perceived by the UE between AoA1 and AoA2, are discussed and illustrated in Annex B. The following observations can be made

Observation 10: With probes aligned in the xz axis, the AoA1-AoA2 DL orientation vectors point towards the pole at q=180° for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards the pole at q=0° for DL directions perceived from y≤0.

Observation 11: With probes aligned in the yz axis, the AoA1-AoA2 DL orientation vectors point towards positive f for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards negative f for DL directions perceived from y≤0.

Proposal 2: OEMs and chipset vendors to comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality.

Based on the observations made in this section, the following proposals are made:

Proposal 3: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in .

Proposal 4: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

Proposal 5: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, place the AoA2 probes in the xz plane.

Table 3: DL directions perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size grid using 312 grid points (Dq=Df=15°). Full rotation in f and half rotation in q.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in f and half rotation in q |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 4: DL directions perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant density grid using 200 grid points. Full rotation in f and half rotation in q. Full rotation in f and half rotation in q.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in f and half rotation in q |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 5: DL directions perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size grid using 312 grid points (Dq=Df=15°). Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f | s |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 6: DL directions perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant density grid using 200 grid points. Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

# Blocking Considerations with Full rotations in  and Half Rotation in  Positioning Approach.

The positioner blocking effects and mitigation techniques for the system proposed in the previous section, see Figure 3, are discussed and illustrated more closely in this section.

Figure 3: Proposed system configuration with probes in the xz plane. 3D scan performed with full rotation in  and half rotation in 

With the turntable covering a full 360° rotation and the probes aligned in the xz plane, positioner blocking is expected for each AoA/probe within the first half turn of the turntable (AZ motion) as illustrated in Figure 4 (first half turn highlighted in yellow, second half turn highlighted in grey).

Figure 4: Positioner blocking of AoAs during first half turn (highlighted in yellow). Top left: blocking of P150, top right: blocking of P120, bottom left: blocking of P60, bottom right: blocking of P0

Obviously, such blocking is highly undesirable as it could skew the test results and potentially lead to dropped calls and longer test times.

It should be noted that during the scan in the second half turn of the turntable, no positioner blocking is experienced since the positioner does not pass in front of any of the probes again as illustrated in Figure 5 with first half turn highlighted in grey, second half turn highlighted in yellow.

Figure 5: Lack of positioner blocking of AoAs during second half turn (highlighted in yellow).

The re-positioning concept, discussed in [3][7], was adopted for FR2 UE RF testing already to eliminate blocking and reduce the effect of signal ripple on EIS measurements by allowing to perform two separate tests in a single hemisphere with a device rotation by 180° in between.

Figure N.3-2: DUT re-positioning for an example of combined-axes system

This concept could be applied to Multi-AoA RX testing as well. As illustrated earlier, the hemisphere without blocking is for q ranging between 180° and 360°, or alternatively -180° ≤ q ≤ 0°. The test approach with minimized blocking is outlined and visualized in Table 7.

Table 7: Test steps for Multi-AoA RX testing based on repositioning concept

| Step 1: Position device in default alignment option |  |
| --- | --- |
| Step 2: Test single hemisphere -180° ≤ q ≤ 0° for 1st time |  |
| Step 3: Return to (0°,0°) position |  |
| Step 4: Apply repositioning to DUT, e.g., rotation by 180° around x axis |  |
| Step 5: Test single hemisphere -180° ≤ q ≤ 0° for 2nd time |  |

It is therefore proposed to consider the repositioning concept for multi-AoA RX testing to reduce the effect of blocking.

Proposal 6: Consider the repositioning concept with the proposed test system setup for multi-AoA RX testing to reduce the effect of blocking

# Polarization Combinations/Permutations

No decision has been made yet on the polarization combinations/permutations necessary for AoA1 and AoA2 [6]. Four different combinations for the multi-AoA Rx test are possible as in Table 8 for AoA1 and AoA2 placed within the xz plane, i.e., (AoA1q, AoA2q), (AoA1q, AoA2f), (AoA1f, AoA2q), (AoA1f, AoA2f).

Table 8: Possible Polarization Combinations/Permutations between AoA1 and AoA2 with AoA2 probe within xz plane

|  | q-Pol. for AoA1 (AoA1q) | f-Pol. for AoA1 (AoA1f) |
| --- | --- | --- |
| q-Pol. for AoA2 (AoA2q) |  |  |
| f-Pol. for AoA2 (AoA2f) |  |  |
| Note: The polarizations at the poles are ambiguous; the AoA1 q/f polarization was selected arbitrarily |  |  |

The proposal in [6] to limit the polarizations to the cross-polarized combinations (AoA1q&AoA2f and AoA1f&AoA2q) effectively reduces the overall test time by ½.

Proposal 7: Limit the polarization combinations for the multi-AoA spherical coverage test case pending feedback from OEMs and chipset vendors.

# Approximate Test Time Estimates

The test time for the multi-AoA DL spherical test depends on various aspects, e.g.,

- number of AoA2 probes

- number of polarization combinations (AoA1q, AoA2q), (AoA1q, AoA2f), (AoA1f, AoA2q), (AoA1f, AoA2f)

- single-DCI vs multi-DCI schemes

- parametric vs non-parametric test approach

The impact of these aspects on the test sequences and test times are explained in more detail in [11]. The summary of test time estimates is tabulated in Table 9. Big differences in terms of test time/test efforts can be observed; as expected, the non-parametric test approach yields the lowest test time.

Table 9: Overview of Approximate Test Times

| Power Class | Grid Type | Test Approach | Minimum Number of Spherical Coverage Test Points N | Number of Polarization Combinations P | Number of AoA2 Probes M | Effort/Test Time for multi-AoA DL spherical coverage test [min] |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Min | Max |  |
| PC1/PC3 | constant-step size | Parametric single-DCI | 312 | 4 | 4 | 1331 | 1799 | Min/Max test time depends on Joint 2 AoA Sensitivity approach |
|  |  |  |  |  | 3 | 1248 | 1716 |  |
|  |  |  |  |  | 2 | 1165 | 1633 |  |
|  |  |  |  | 2 | 4 | 666 | 900 |  |
|  |  |  |  |  | 3 | 624 | 858 |  |
|  |  |  |  |  | 2 | 582 | 816 |  |
| PC1/PC3 | constant-step size | Parametric multi-DCI | 312 | any | 4 | 666 |  | N/A |
|  |  |  |  |  | 3 | 624 |  |  |
|  |  |  |  |  | 2 | 582 |  |  |
| PC1/PC3 | constant-step size | Non-Parametric single-DCI or multi-DCI | 312 | 4 | 4 | 21 | 333 | Min/Max depends on early Pass |
|  |  |  |  |  | 3 | 21 | 250 |  |
|  |  |  |  |  | 2 | 21 | 166 |  |
|  |  |  |  | 2 | 4 | 21 | 166 |  |
|  |  |  |  |  | 3 | 21 | 125 |  |
|  |  |  |  |  | 2 | 21 | 83 |  |

In [1], feedback from industry was requested whether to continue with a parametric test approach for multi-AoA spherical coverage testing or whether to adopt a non-parametric test approach based on pass/fail metrics.

# Multi-AoA Demodulation Testing

The measurement setup for demodulation testing was discussed in RAN4#104-bis-e but took second priority to UE RF [13][14]. The same probe offset options were proposed to for consideration for demodulation testing.

Issue 1-4-1: Measurement setup for UE demodulation testing - Proposals - Option 1: Consider a system utilizing 1 AoA with full degree of freedom and 1 AoA with limited, fixed degrees of freedom as starting point for FR2 4-DL demodulation testing. - Option 1a: Fixed Angular Offset(s) between AoA1 and AoA2 in the chamber. The angular separation between AoA1 and AoA2 is NOT changing during the testing mapping to option 2a in issue 1-2-1 - Option 1b: Variable Angular Offset(s) between AoA1 and AoA2 in the chamber where AoA2 is fixed with respect to the UE during the test. The angular separation between AoA1 and AoA2 is changing during the testing mapping to option 2b in issue 1-2-1 - Option 1c: Full degrees of freedom for AoA1 with partial freedom of variable angular offset(s) between AoA1 and AoA2 mapping to option 2c in issue 1-2-1 - Option 2: Seek an incremental enhancement of the Rel-15 demodulation test setup which can enable the necessary AoA control for both AoA1 and AoA2 directions

Further discussions were held in RAN4#105 with no outcome on the baseline measurement setup [2]

Issue 1-4-1: Measurement setup for UE demodulation testing - Proposals - Option 1 (Qualcomm, Keysight): RAN4 to consider to use fixed Angular Offset(s) between AoA1 and AoA2 (Same as option 2a in Issue 1-2-1) as the starting point of measurement setup for UE demodulation testing - Option 2 (R&S): RAN4 to focus on the incremental enhancement of the Rel-15 demodulation test setup with an additional AoA without defining a concrete test system architecture - Agreement: - FFS on measurement setup for UE demodulation testing with the options above. Other options are not precluded.

We still believe that it is highly desirable to leverage the UE RF test system/setup for demodulation testing as it was done for 1 AoA testing. As stated in [12], we feel confident that the proposed UE RF setup in Setup 2b is applicable for demodulation testing based on our experiences with wireless cable modes in LTE/FR1 and 4-DL MIMO with test systems that utilize limited sets of AoAs.

The 1-DL EIS spherical coverage map can be leveraged to determine suitable UE orientations and AoA1/AoA2.m probe pairs that maximize the available SNR as a starting point for demodulation testing.

There should be no harm in defining a measurement setup for multi-AoA demodulation testing and further relaxing some setup aspects later if needed.

Proposal 8: Consider Measurement Setup 2a the starting point for multi-AoA demodulation testing.

# RRM Testing

The measurement setup for RRM testing was discussed in RAN4#104-bis-e but took second priority to UE RF [13][14]. Regarding the measurement setup, it was agreed to consider the legacy RRM system the baseline and to further study whether the UE RF test system could be considered an alternative.

Issue 1-3-1: Measurement setup for UE RRM testing - Agreement: - Legacy RRM test system can be baseline, more probes might be added based on the process of RRM session - FFS whether the same test system for UE RF testing could be considered as an alternative for UE RRM testing.

Further discussions were held in RAN4#105 with no outcome on the baseline measurement setup [2]

Issue 1-3-1: Measurement setup for UE RRM testing - Proposals - Option 1 (Qualcomm): RAN4 to consider the enhancement from legacy RRM test system such as adding more probes to support more angular separations for multi-Rx RRM testing - Option 2 (Keysight): Consider measurement setup of Option 2a in Issue 1-2-1 with a minimum of 3 AoA2 probes an alternative for RRM testing. - Agreement: - FFS on measurement setup for UE RRM testing with options above. Other options are not precluded.

We feel confident that the proposed UE RF setup in Setup 2b is applicable for RRM testing while leaving the number of probes as FFS.

Proposal 9: Consider Measurement Setup 2a a suitable baseline setup for RRM testing with the number of probes FFS.

The number of multi-AoA RRM probes might be exceed that of multi-AoA RF probes required if RRM requirements dictate the use of 4 different simultaneous AoAs and whether the relative angles between the 4 AoAs must change between two subsequent iterations.

Proposal 10: The minimum number of RRM probes is pending clarifications from the requirements discussion in WI.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: For single-probe OTA systems, different 3D UE rotation/scan options, e.g., full rotation in f (around z axis) and half rotation in q (around y axis) or full rotation in q (around y axis) and half rotation in f (around z axis), will yield the same test point coverage.

Observation 2: Probes aligned in the yz plane introduce large keep-out areas for the angular coverage/DL directions towards each pole.

Observation 3: With full rotation in f and half rotation in q, probes aligned in the xz plane introduce a conical sector towards the pole at q=0° with no angular coverage/DL directions for offset probes.

Observation 4: Constant density grids do not maintain the constant density coverage/DL direction distribution for probes offset from P0, a higher density of DL directions is observed especially near the poles.

Observation 5: When probes are aligned in the xz plane, constant-step size grids maintain the constant-step size distribution of DL directions for probes offset from P0.

Observation 6: Probes aligned in the xz plane generally provide a wider angular coverage for AoA2 DL directions when compared to probes aligned in the yz plane.

Observation 7: With full rotation in q and half rotation in f and probes aligned in the xz plane, the AoA2 DL directions are distributed in 3D without a conical region with lack of coverage.

Observation 8: Only with full rotation in q and half rotation in f, probes aligned in the xz plane, and constant-step size grids, uniform angular coverage/DL directions is achieved, i.e., the distribution of DL directions for all probes on the xz plane match the distribution of grid points.

Observation 9: When the AoA2 probes are placed in the xz plane, probe antenna DL q/f polarizations map to the same DUT q/f polarizations, while when AoA2 probes are placed in the yz plane, probe antenna DL q/f polarizations generally map to a combination of DUT q/f polarizations.

Observation 10: With probes aligned in the xz axis, the AoA1-AoA2 DL orientation vectors point towards the pole at q=180° for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards the pole at q=0° for DL directions perceived from y≤0.

Observation 11: With probes aligned in the yz axis, the AoA1-AoA2 DL orientation vectors point towards positive f for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards negative f for DL directions perceived from y≤0.

Proposal 1: Limit the maximum angular separation between probes/AoAs to 150° to avoid blocking effects, to minimize power variations inside the QZ, and allow the re-use/augmentation of existing 2 AoA RRM systems for multi-AoA UE RF testing.

Proposal 2: OEMs and chipset vendors to comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality.

Proposal 3: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, apply a full rotation in  and a half rotation in .

Proposal 4: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, utilize constant-step size grids only.

Proposal 5: For optimized AoA1 and AoA2 test point/perceived DL direction coverage, place the AoA2 probes in the xz plane.

Proposal 6: Consider the repositioning concept with the proposed test system setup for multi-AoA RX testing to reduce the effect of blocking

Proposal 7: Limit the polarization combinations for the multi-AoA spherical coverage test case pending feedback from OEMs and chipset vendors.

Proposal 8: Consider Measurement Setup 2a the starting point for multi-AoA demodulation testing.

Proposal 9: Consider Measurement Setup 2a a suitable baseline setup for RRM testing with the number of probes FFS.

Proposal 10: The minimum number of RRM probes is pending clarifications from the requirements discussion in WI.

# References

- R4-2219851, Testing Considerations for Multi AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2220264, WF for FR2 OTA test SI, Qualcomm, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

- TS 38.133, Requirements for support of radio resource management

- R4-2216786, On UE RF requirements for 2AoA FR2 DL MIMO, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 104-Bis-e, October 2022

- TR38.810, Study on test methods

- R4-2219852, System Parameter Assumptions for Multi AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #105, November 2022

- R4-2217731, Email discussion summary for [104-bis-e][132] FR2_multiRx_UERF_part1, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting # 104-Bis-e, October 2022

- R4-2218755, Further views on multi-Rx chain DL reception in FR2, Sony, Ericsson, 3GPP TSG-RAN WG4 Meeting # 105, November 2022

- R4-2302522, System Parameter Assumptions for Multi-AoA Rx Testing, Keysight Technologies, 3GPP TSG-RAN4 Meeting #106, February 2023

- R4-2215540, Testing Considerations for Multi AoA UE RF Spherical Coverage Test Case and FR2 Demodulation, Keysight Technologies, 3GPP TSG-RAN4 Meeting #104-bis-e, October 2022

- R4-2217490, Email discussion summary for [104-bis-e][325] FS_NR_FR2_OTA_enh, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 104-bis-e, October 2022

- R4-2217453, WF on NR FR2 OTA testing enhancements, Qualcomm, 3GPP TSG-RAN WG4 Meeting # 104-bis-e, October 2022

# Annex A: DL Polarizations Perceived by the UE

The DL polarizations perceived by the UE from various probes placed in different locations and 2D planes is investigated in this Annex. For better visualization purposes, a coarse measurement grid with 84 grid points for the constant-step size grid (Dq=Df=30°) and 50 grid points for the constant density grid is considered.

The DL directions and polarizations perceived by the DUT for a sample system with the AoA1 probe in the z direction and AoA2 probes in the x and y directions are presented in Table 10 for just the constant-step size grid. The magenta arrows demonstrate the q polarization while the orange arrows demonstrate the f polarization with respect to the probe antenna/system coordinate system. Clearly, for the probes placed in the z and x directions, the DL polarizations w.r.t. the DUT coordinate system are preserved, i.e., the probe antenna DL q/f polarization matches the DUT q/f polarization. However, for the probe along the y axis, the DL polarization perceived by the DUT changes depending on grid point. Similar observations can be made for a system configuration with multiple AoA2 probes placed on the xz and the yz planes. The system configurations with full rotation in  and half rotation in  and the perceived DL directions are further illustrated in Table 10 for the constant-step size grid and in Table 11 for the constant density grid. Similarly, the system configurations with full rotation in q and half rotation in f and the perceived DL directions are further illustrated in Table 12 for the constant-step size grid and in Table 13 for the constant density grid.

These results again show that the probe antenna DL q/f-polarizations from AoA2 probes (w.r.t. the system coordinate system) are preserved only when the AoA2 probes are placed in the xz plane, i.e., antenna DL q/f polarizations map to DUT q/f polarizations, while this is no longer the case for AoA2 probes are placed in the yz plane, i.e., probe antenna DL q/f polarizations generally map to a combination of DUT q/f polarizations.

Observation 9: When the AoA2 probes are placed in the xz plane, probe antenna DL q/f polarizations map to the same DUT q/f polarizations, while when AoA2 probes are placed in the yz plane, probe antenna DL q/f polarizations generally map to a combination of DUT q/f polarizations.

Table 10: DL direction and polarizations perceived by the DUT from probes in the x, y, and z axes.

|  | DL Directions perceived by DUT with constant-step size grid with 84 grid points (Dq=Df=30°). Full rotation in f and half rotation in q. | DL Directions perceived by DUT with constant-step size grid with 84 grid points (Dq=Df=30°). Full rotation in q and half rotation in f. |
| --- | --- | --- |
| System Configuration |  |  |
| DL Directions and Polarizations perceived by DUT from Probe along z |  |  |
| DL Directions and Polarizations perceived by DUT from Probe along y |  |  |
| DL Directions and Polarizations perceived by DUT from Probe along x |  |  |

Table 11: DL direction perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size spherical coverage grid using 84 grid points (Dq=Df=30°). Full rotation in f and half rotation in q.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in f and half rotation in q |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 12: DL directions and polarizations perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant density spherical coverage grid using 50 grid points. Full rotation in f and half rotation in q. Full rotation in f and half rotation in q.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in f and half rotation in q |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 13: DL directions and polarizations perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size spherical coverage grid using 84 grid points (Dq=Df=30°). Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

Table 14: DL directions and polarizations perceived by the DUT from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant density spherical coverage grid using 50 grid points. Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f |  |  |
| DL Directions perceived by DUT from Probe P60 |  |  |
| DL Directions perceived by DUT from Probe P120 |  |  |
| DL Directions perceived by DUT from Probe P150 |  |  |

# Annex B: AoA1-AoA2 DL Orientation Vectors

The relative orientation between the P0 and the three offset probes, i.e., P60, P120, and P150, are investigated in this Annex. For better visualization purposes, a coarse measurement grid with 84 grid points for the constant-step size grid (Dq=Df=30°) and 50 grid points for the constant density grid is considered. Given the clear advantages of the 3D measurement scan using a full rotation in q and half rotation in f for multi-AoA testing, only this approach is further considered here.

The AoA1-AoA2 DL orientation vectors, i.e., the vectors between the DL orientation perceived by the UE from AoA1 to AoA2, are visualized in Table 15 for the constant step-size grid and in Table 16 for the constant-density grid. The vectors are illustrated in two different ways. In the figures on the left, e.g.,

the AoA1 DL directions are plotted on top of the sphere with the black marker while the AoA2 DL directions are plotted slightly elevated from the sphere for clarity. The corresponding AoA1 & AoA2 pairs received by the UE are connected with arrows (pointing from AoA1 to AoA2) with the arrows plotted in random colours to support better differentiation. In the figures on the right, e.g.,

only the AoA1 DL directions are plotted on top of the sphere with the black marker while the direction towards the corresponding offset AoA2 DL direction is visualized with the single arrow (the length of the arrow corresponds to the value of the offset).

The following observations can be drawn from these figures:

Observation 10: With probes aligned in the xz axis, the AoA1-AoA2 DL orientation vectors point towards the pole at q=180° for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards the pole at q=0° for DL directions perceived from y≤0.

Observation 11: With probes aligned in the yz axis, the AoA1-AoA2 DL orientation vectors point towards positive f for DL directions perceived from y≥0 while the AoA1-AoA2 DL orientation vectors point towards negative f for DL directions perceived from y≤0.

Feedback from OEMs and chipset vendors is requested whether the directionality of the AoA1-AoA2 DL orientation vectors matters or whether it needs to be compensated somehow.

Proposal 2: OEMs and chipset vendors to comment on the directionality of the AoA1-AoA2 DL orientation vectors, e.g., whether reciprocity of the TRPs will tolerate the observed directionality.

Table 15: Vectors of DL orientations between AoA1 (P0) and AoA2 from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant-step size grid using 84 grid points (Dq=Df=30°). Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f |  |  |
| Orientation between Probes P0 and P60 |  |  |
| Orientation between Probes P0 and P120 |  |  |
| Orientation between Probes P0 and P150 |  |  |

Table 16: Vectors of DL orientations between AoA1 (P0) and AoA2 from two different system configurations with AoA2 to AoA1 probe offsets of 60°, 120°, and 150° with a constant density grid using 50 grid points. Full rotation in q and half rotation in f.

|  | Probes in the xz plane | Probes in the yz plane |
| --- | --- | --- |
| System Configuration with full rotation in q and half rotation in f |  |  |
| Orientation between Probes P0 and P60 |  |  |
| Orientation between Probes P0 and P120 |  |  |
| Orientation between Probes P0 and P150 |  |  |