3GPP TSG-RAN WG4 Meeting # 106	R4-2302529

Athens, Greece, 27 February – 3 March 2023

Agenda Item:	9.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on UE RF requirements for FR2-1 UL 256QAM

Document for:	Approval

# 1.	Introduction

The revised work item on NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved at TSG RAN#96 [1]. One of the objectives of this work item is to investigate and enable UL 256QAM for FR2-1.

One of the main tasks to investigate and enable UL 256QAM for FR2-1 is to specify the UE RF requirements. This topic was discussed at TSG RAN4#104-bis-e and the WF was agreed [2], and the TP on link level simulation assumptions and results was approved [3] at TSG RAN4#105. This contribution provides proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF and the simulation results in [3].

# 2.	Discussion

2.1.	EVM requirement for 39GHz

It was agreed in the WF [2].

- 3.5% EVM for 39GHz and using average value FFS for operating SNR with limited MCS.

  - The limited MCS is the subset of MCS with 256QAM

  - FFS on the list of MCS

  - Decide the operating SNR based on list of MCS

  - The target power class is PC1, PC2, and PC5.

The defined CP-OFDM/DFT-s-OFDM MCS with 256QAM are provided in Table 5.1.3.1-2 in TS 38.214 [4]. The table are extracted below where MCS 20 to 27 are with 256QAM.

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

The simulation results with 256QAM using MCS 21 and MCS 23 presented at RAN4#104-bis are summarized in [3]. The simulation results showed that 256QAM using MCS 23 cannot provide throughput gain over 64QAM using MCS 24 (defined in Table 5.1.3.1-1 in [4]) in the simulated fading channels over the simulated operating SNR. Therefore, it is proposed to limit MCS with 256QAM to 21 and 22 in the table above.

For the operating SNR, it was proposed to adopt the average SNR of 29dB at 29GHz (with 3.5% EVM) above which 256QAM provides throughput gain over 64QAM from the simulation results presented at TSG RAN4#104-bis [5]. Using the same approach, it is proposed to adopt the average SNR of 27dB at both 28GHz and 39GHz (with 3.5% EVM) above which 256QAM provides throughput gain over 64QAM from the simulation results summarized in [3]. Our system level simulation results in [6] show that, a significant percentage of PC1/PC2/PC5 UE can achieve the target SNR at BS side above which 256QAM using MSC 21 provides throughput gain over 64QAM using MCS 23 at both 29 GHz (n257) and 39 GHz (n260) in all simulated scenarios using these operating SNR as target. Therefore, it is proposed to adopt 27 dB at both 29GHz and 39GHz as the target operating SNR.

2.2	PTRS configuration

The following options were discussed at RAN4#104-bis but no agreement was made [3].

  - Option 1: PTRS configuration shall be aligned with the UE’s recommended PTRS configuration. (IE PTRS-DensityRecommendationSetUL)

  - Option 2: Using a fixed PTRS configuration for all devices for the EVM test:

- For CP-OFDM: LPT-RS = 1 and KPT-RS = 2

- For DFT-s-OFDM: (,)=(8, 4)

  - Option 3: Others.

For option 1, we have question whether UE performance would be worse in the field if gNB cannot configure such PTRS for the UE, as this would impact gNB UL scheduling which need to consider all UEs under coverage, including the majority that would not have 256QAM UL scheduled. We consider the EVM test should ensure UE performance even though gNB cannot configure the recommended PTRS for the UE in the field. Therefore, it is proposed to adopt option 2 to use a fixed PTRS configuration for all devices for the EVM test. Note that LPT-RS = 1 should also be used for DFT-s-OFDM in option 2.

2.3	Phase noise assumption on EVM budget

The following options were discussed at RAN4#104-bis but no agreement was made [3].

  - Option 1: Using the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis.

  - Option 2: Others

Option 1 was not agreed by some companies, but the findings recorded in TR 38.803 [7] were thoroughly discussed in RAN4 before they were approved into the TR, and they were not targeted at any modulation scheme so should equally be applicable with 256QAM. It is proposed that opponents of option 1 should provide concrete alternative proposal to be discussed in RAN4 to progress the discussion in this topic.

# 4.	Conclusion

This contribution has provided proposals on UE RF requirements for FR2-1 UL 256QAM according to the agreed WF and the related discussion, they are summarized as follows:

Proposal 1: To limit MCS with 256QAM for 39GHz to 21 and 22 in Table 5.1.3.1-2 in TS 38.214.

Proposal 2: To adopt 27 dB at both 29GHz and 39GHz as the target operating SNR.

Proposal 3: To adopt option 2 (with LPT-RS = 1 also for DFT-s-OFDM) to use a fixed PTRS configuration for all devices for the EVM test.

Proposal 4: Opponents of using the findings recorded in TR 38.803 on phase noise for mm-wave frequencies as a basis should provide concrete alternative proposal to be discussed in RAN4 to progress the discussion in this topic.

# References

[1]	RP-221862, “Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3”, Nokia, Xiaomi.

[2]	R4-2217729, “WF on UL 256QAM”, Xiaomi.

[3]	R4-2219122, “TP for TR 38.891 on link level simulation results and system level simulation assumption for FR2 UL 256QAM”, Xiaomi.

[4]	3GPP TR 38.214 v17.3.0: “NR: Physical layer procedures for data”.

[5]	R4-2217783, “Email discussion summary for [104-bis-e][131] FR2_enh_req_Ph3_part2”, Xiaomi.

[6]	R4-2300193, “System level simulation results for FR2-1 UL 256QAM”, Nokia, Nokia Shanghai Bell.

[7]	3GPP TR 38.803 v14.3.0: “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.