3GPP TSG-RAN WG4 Meeting #106	R4-2302568

# Athens, Greece, Feb 27 – Mar 3, 2023

Title:	[Draft] LS on intra-band EN-DC band combination support

Response to:

Release:	Rel-16

Work Item:

Source:	RAN4

To:	RAN2

Cc:	RAN

Contact Person:

Name: 	Zhifeng Ma

E-mail Address: 	ma.zhifeng@zte.com.cn

Attachments:	N/A

1. Overall Description:

The B48+n48 related intra-band EN-DC configurations are concluded to be supported in RAN4 spec which can be categorized into 6 classes, i.e. Case 3/3a and Case 4/4a-1/4a-2/4a-3 as follows. The ‘both’ value of IE intraBandENDC-Support in UE capability should be with no ambiguity for mixed intra-band contiguous and non-contiguous EN-DC configurations.

Table 1: Intra-band EN-DC configurations related to B48+n48

| Case | Downlink EN-DC configuration | Uplink EN-DC configurations | Continuity in DL & UL |  |
| --- | --- | --- | --- | --- |
|  |  |  | DL | UL |
| 3 | DC_(n)48CA | DC_48A_n48A | Contiguous | Non-contiguous |
|  |  | DC_(n)48AA | Contiguous | Contiguous |
| 3a | DC_(n)48AA | DC_(n)48AA | Contiguous | Contiguous |
| 3 | DC_(n)48DA | DC_48A_n48A | Contiguous | Non-contiguous |
|  |  | DC_(n)48AA | Contiguous | Contiguous |
| 4 | DC_48A-(n)48AA | DC_(n)48AA | Contiguous for LTE and NR adjacent carriers but non-contiguous for LTE carriers | Contiguous |
|  |  | DC_48A_n48A | Contiguous for LTE and NR adjacent carriers but non-contiguous for LTE carriers | Non-contiguous |
| 4a-1 | DC_48A-48A_n48A | DC_48A_n48A | Non-contiguous (with non-contiguous LTE carriers) | Non-contiguous |
| 4a-2 | DC_48A_n48A | DC_48A_n48A | Non-contiguous (with single LTE carrier) | Non-contiguous |
| 4a-3 | DC_48C_n48A | DC_48A_n48A | Non-contiguous (with contiguous LTE carriers) | Non-contiguous |
| 4a-3 | DC_48D_n48A | DC_48A_n48A | Non-contiguous (with contiguous LTE carriers) | Non-contiguous |

From the perspective of future extension, the following intra-band EN-DC configuration having three band entries with two entries from NR band and one entry from E-UTRA band, such as DC_(n)XAA-nXA should also be taken into consideration.

| Case | Downlink EN-DC configuration | Uplink EN-DC configurations | Continuity in DL & UL |  |
| --- | --- | --- | --- | --- |
|  |  |  | DL | UL |
| 5 | DC_(n)XAA-nXA | DC_(n)XAA | Contiguous for LTE and NR adjacent carriers but non-contiguous for NR carriers | Contiguous |
|  |  | DC_XA_nXA | Contiguous for LTE and NR adjacent carriers but non-contiguous for NR carriers | Non-contiguous |

Whether there are future requirements for the following two cases are still under discussion in RAN4.

- ‘Case x’:    DL: DC_(n)XCA    UL: DC_XA_nXA

- ‘Case y’:    DL: DC_YA-(n)YAA    UL: DC_YA_nYA

2. Actions:

To RAN2:

RAN4 respectfully asks RAN2 to take the above into consideration when evaluate the ‘both’ value for IE intraBandENDC-Support in UE capability.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#106bis-e                      April 2023										E-meeting

TSG-RAN4 Meeting#107	                          May 2023								 Incheon, Korea