3GPP TSG-RAN WG4 Meeting # 106	R4-2302640

# Athens, Greece, February 27 – March 03, 2023

Agenda item:	9.12.5.2

Source: 	Qualcomm Incorporated

Title: 	Intra-band CA requirements for FR2 HST

Document for:	Discussion

# Introduction

In this contribution, we present our view on the following objective of the work item according to WID [1]:

- Specify the RF requirements for intra-band carrier aggregation (CA) scenario and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4].

- RAN4 shall not specify requirements for deactivated SCell in FR2 HST scenario

- Whether basic CA requirements should be further enhanced can be discussed after more clarification is achieved.

  - Option 1 (Nokia):

    - RAN4, firstly, to specify CA RRM requirements in HST FR2 scenarios only for open space deployment and for PC6 UEs capable of single-panel reception, i.e., by specifying CA capability based on Rel-17 PC6 UEs.

    - RAN4 to discuss what is the minimal set of UE functionalities need for the support of CA in HST FR2 scenario and defined requirements only for those.

  - Other options are not precluded

- For Rel-18 FR2 PC6, by introducing CA operation, RRM impacts are expected at least for:

  - Measurement period for Inter-frequency measurement

  - L1-RSRP measurement requirements

- For Rel-18 FR2 PC6, the necessity of the following requirements shall be FFS:

  - PSS/SSS detection, Time index detection for Intra-frequency and inter-frequency measurements

  - FFS IDLE mode in inter-frequency measurement requirements

  - Beam failure recovery on SCell

  - TCI state switch in SCell.

  - Active SCell measurement requirement (measurement period)

- FFS whether enhancement on CA also need to consider tunnel deployment scenario and multi-RX capabilities

  - FFS RX beam scaling factor for tunnel scenario

- HST signaling for CA enhancement:

  - Option 1: Reuse PCell signaling (i.e., Rel-17 HST FR2 flag with CA configuration from NW)

    - Option 1a (Nokia): Reuse PCell signalling partially, i.e., highSpeedMeasFlagFR2-r17 for HST FR2 scenario indication on SCell

During the last meeting, RAN4 discussed various proposals on intra-band CA requirements for FR-2 HST and agreed to the following [2]

# Intra-band CA requirements for FR2-HST UE

We think that RAN4 should focus on defining the basic intra-band CA requirements for FR2-HST and to specify requirements only for open space deployment and for PC6 UEs capable of single-panel reception, i.e., by specifying CA capability based on Rel-17 PC6 UEs. Other scenarios such as multi-panel operation can be discussed later, no need to consider tunnel scenario for intra-band CA operation.

Proposal 1: RAN4 to specify CA RRM requirements in HST FR2 scenarios only for open space deployment and for PC6 UEs capable of single-panel reception, i.e., by specifying CA capability based on Rel-17 PC6 UEs.

  - RAN4 to discuss what is the minimal set of UE functionalities need for the support of CA in HST FR2 scenario and defined requirements only for those.

  - No need to consider tunnel scenario to define intra-band CA requirements.

Now coming to which are requirements are needed to support intra-band CA operation, we think only inter-frequency measurement requirements are needed. Intra-frequency PSS/SSS detection and intra frequency measurement requirements are already specified for the PCell and the same shall apply to SCell as well. However, inter frequency cell detection and measurement requirements are missing from the spec which are needed to support intra-band CA for FR2-HST.

Proposal 2: FR2-HST specific intra-frequency PSS/SSS detection and intra frequency measurement requirements are already specified for the PCell and the same shall apply to SCell as well. No additional intra-frequency requirements are needed to support intra-band CA for FR2-HST.

Proposal 3: RAN4 to specify inter-frequency cell identification (PSS/SSS detection) and measurement requirements to support intra-band CA for FR2-HST.

For exact inter-frequency requirements, we can follow the same principles that were used to specify FR2-HST requirements in Release 17 – use beam-sweeping scaling factor of 2 and 6 for highSpeedMeasFlagFR2-r17 = set1 and highSpeedMeasFlagFR2-r17 = set2, respectively.

Proposal 4: RAN4 to use Rel-17 FR2-HST principles to define inter-frequency cell detection and measurement requirements, i.e., use beam-sweeping scaling factor of 2 and 6 for set1 and set2, respectively.

# Conclusion

Proposal 1: RAN4 to specify CA RRM requirements in HST FR2 scenarios only for open space deployment and for PC6 UEs capable of single-panel reception, i.e., by specifying CA capability based on Rel-17 PC6 UEs.

  - RAN4 to discuss what is the minimal set of UE functionalities need for the support of CA in HST FR2 scenario and defined requirements only for those

  - No need to consider tunnel scenario to define intra-band CA requirements.

Proposal 2: FR2-HST specific intra-frequency PSS/SSS detection and intra frequency measurement requirements are already specified for the PCell and the same shall apply to SCell as well. No additional intra-frequency requirements are needed to support intra-band CA for FR2-HST.

Proposal 3: RAN4 to specify inter-frequency cell identification (PSS/SSS detection) and measurement requirements to support intra-band CA for FR2-HST.

Proposal 4: RAN4 to use Rel-17 FR2-HST principles to define inter-frequency cell detection and measurement requirements, i.e., use beam-sweeping scaling factor of 2 and 6 for set1 and set2, respectively.

# References

[1] RP-220985, “New WID on enhanced NR support for high-speed train scenario in frequency range 2 (FR2)”, Samsung, RAN#95e

[2] R4-2220397, “WF on NR FR2 HST RRM Core requirements impact”, Samsung, RAN4#105