3GPP TSG-RAN WG4 #106	R4-2302707

February 27th ‒ March 3rd, 2023

Athens, GR

Agenda item:	8.30.1

Source: 	Qualcomm Incorporated

Title: 	TP for TR 38.892: UE requirements

Document for:	Approval

# Introduction

This document consists of a text proposal for Section 7.1 of TR 38.892 on UE requirements based on the endorsed CR for TS 38.101-1 in [1].

# Reference

- R4-2220487, “Introduction of APT 600 MHz band,” Qualcomm Incorporated

<< Start of TP >>>

# 7	UE and BS requirements for APT 600 MHz

## 7.1	UE requirements

### 7.1.1 	UE transmitter characteristics

#### 7.1.1.1	UE TX maximum output power

Power class 3 is specified for Band n105 with a maximum output power of 23 dBm.  The tolerance is +2/-2.5 dB similar to Band n71.

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |

#### 7.1.1.2 	NS value and Spectrum Emission Mask

No additional spurious emission requirements have at this time been identified for Band n105.  Therefore, no NS other than the default NS_01 has been specified.  In the future if regulators deem it necessary to impose additional spurious emission requirements to protect adjacent or nearby services for deployment in certain countries, the NS specifications for Band n105 can be revised accordingly.

The spectrum emission mask follows the general mask as specified in clause 6.5.2.2 of TS 38.101-1.  No additional spectrum mask is applicable to Band n105.

### 7.1.2	UE receiver characteristics

#### 7.1.2.1	UE REFSENS

Reference sensitivity for Band n105 has been agreed as follows

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n105 | 15 | -97.28 | -94.0 | -91.6 | -86.9 | -85.1 | -83.8 | -82.5 |  |  |  |
|  | 30 |  | -94.3 | -91.9 | -87.9 | -85.5 | -84.3 | -82.6 |  |  |  |
| NOTE 8: DL channels overlapping the 612-617MHz range have 0.5dB added to the REFSENS |  |  |  |  |  |  |  |  |  |  |  |

#### 7.1.2.2	In-band blocking

In-band blocking for Band n105 has been specified as Case 1, Case2, and Case 5.  Case 5 is specified with an interferer power of -22 dBm except for Band n105 channels overlapping the lowermost 5 MHz of the band where it is relaxed to -34 dBm.

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 | -224 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |  |
| n105 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |  | FDL_low – 7 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. NOTE 4: For Band n105 channels overlapping the 612 - 617 MHz frequency range, Pinterferer is modified to -34 dBm. |  |  |  |  |  |  |  |

#### 7.1.2.3 	Out-of-band blocking

Out-of-band blocking for Band n105 is specified generally as it is for other bands below 2700 MHz.

Table 7.6.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n105 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |

<< End of TP >>>