3GPP TSG-RAN WG4 Meeting #106	R4-2302926

Athens, Greece, February 27th ‒ March 3rd, 2023

Title: 	WF on FR1 4Tx demodulation requirement

Agenda Item:	9.6.6.2

Source: 	Ericsson

Document for:	Approval

# Topic 1: Test scope

## Sub-topic 1-1 Test scope

Agreement: Only consider PUSCH requirement

## Sub-topic 1-2 Scope for PUSCH

Agreement: Only consider basic feature of Rel-15

## Sub-topic 1-3 Duplex mode

Agreement: Define requirements for TDD with 15kHz SCS with TDD pattern DDDSU, 30kHz SCS with TDD pattern 7D1S2U, but the requirements are applicable for both FDD and TDD.

# Topic 2: PUSCH requirement

## Sub-topic 2-1 Bandwidth and SCS

Agreement:

  - For 15kHz:

    - 5MHz requirement will be introduced

    - 20MHz and 50MHz for initial simulation, and then choose one for performance requirements definition.

  - For 30kHz: 10MHz and 100MHz requirement will be introduced

## Sub-topic 2-2 MIMO layers

Agreement: Only consider rank 4 for performance requirements definition.

## Sub-topic 2-3 PUSCH mapping type

Agreement: Define requirements for both PUSCH Type A and Type B, but BS can conduct testing for one of the requirements based on BS manufacture declaration.

## Sub-topic 2-4 Propagation conditions

Agreement: Following propagation models are used for simulation assumption. The specific MCS for each propagation condition is FFS that is based on the further evaluation, but only one MCS for each propagation condition will be selected for performance requirements definition.

    - TDLA30-10

    - TDLB100-400

    - TDLC300-100

## Sub-topic 2-5 MCS

Agreement: Use MCS table 1 for the evaluation. The following MCS can be used as starting point for evaluation.

- TDLA30-10: MCS16, MCS17 and MCS20

- TDLB100-400: MCS2

- TDLC300-100: MCS12 and MCS16

## Sub-topic 2-6 Antenna configuration

Agreement:

- For initial simulation: include both 4x4 and 4x8

- For the testing of performance requirement, only the performance for the highest supported Rx is used for testing if agreed to define performance requirements for both 4x4 and 4x8.

## Sub-topic 2-7 DM-RS configuration

Agreement: Only additional DM-RS pos1 is defined for 4Tx demodulation requirements.

## Sub-topic 2-8 Precoding

Agreement:

Take TPMI index 0 in Table 6.3.1.5-7 of 38.211 for performance requirement definition.

FFS on TPMI index 4 in Table 6.3.1.5-7 of 38.211 for vehicular use cases.

## Sub-topic 2-9 Transform precoding

Agreement：Only transform precoding disabled (CP-OFDM) will be introduced for performance requirements definition.

## Sub-topic 2-10 Test metric

Agreement: Use 70% of maximum throughput as the test metric for performance requirement definition.

## Sub-topic 2-11 Simulation assumptions

Agreement:

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3dB |
|  | DM-RS port | {0, 1, 2, 3} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0, FFS on 4 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

## Sub-topic 2-12 Test limiting

Agreement: Initially conductivity testing is conducted of this feature, FFS on OTA test set-ups.