3GPP TSG-RAN WG4 Meeting # 106	R4-2302932

Athens, Greece, February 27 – March 3, 2023

Title: 	WF on IoT-NTN UE demodulation and CQI reporting requirements

Agenda Item:	10.5.8

Source: 	MediaTek

Document for:	Approval

Introduction

This document is to capture all agreements for [106][329] IoT_NTN_Demod in RAN4#106.

Topic #1: Workplan

Issue 1-1: Work plan for IoT-NTN UE demodulation requirements

Agreement:

- 3GPP RAN4 #106 meeting (Feb., 2023)

- Approve work plan.

- Agree on simulation assumptions.

- Collect the initial simulation results.

- Agree on the skeleton for UE demodulation performance requirements in TS36.102 clause 8.

- Decide the CR work split.

- 3GPP RAN4 #106-bis-e meeting (April, 2023)

- Update simulation assumptions if necessary.

- Collect the simulation results.

- Review the draft CR for TS36.102.

- 3GPP RAN4 #107 meeting (May, 2023)

- Collect the simulation results.

- Agree with the CR for TS36.102.

Topic #2: TS38.102 skeleton for IoT-NTN UE demodulation

Issue 2-1: TS38.102 skeleton for IoT-NTN UE demodulation

- Agreement:

- RAN4 agreed TS 36.102 skeleton on IoT-NTN UE demodulation requirements as below table:

| Clause number | Clause name |
| --- | --- |
| 8 | Demodulation Performance requirements |
| 8.1 | General |
| 8.2 | Demodulation performance requirements for UE category M1 |
| 8.2.1 | FDD and half-duplex FDD |
| 8.2.1.1 | PDSCH demodulation requirements |
| 8.3 | Demodulation performance requirements for UE category NB1 and NB2 |
| 8.3.1 | Half-duplex FDD |
| 8.3.1.1 | NPDSCH demodulation requirements |
| 8.3.1.1.1 | Minimum Requirements for Standalone |
| Annex A (normative): | Measurement channels |
| Annex B (informative) | Change history |
| Annex C | Downlink physical channels |
| Annex D | Environmental conditions |

Topic #3: Simulation assumptions for Cat-M1 and NB-IoT over NTN

Issue 3-1: Whether the UE is expected to adjust DL frequency compensation on the receiver end within a block of repetitions

- Agreement:

  - From RAN4 performance requirements assumption perspective, UE assumed to be capable of DL frequency compensation on the receiver end within a block of repetitions.

Issue 3-2: Maximum Doppler spread for Cat-M1 PDSCH Test 2

- Agreement:

  - 200Hz

Issue 3-3: FRCs for Cat-M1 PDSCH demodulation requirements for NTN

- Agreement:

  - Test 1: TM1, 3RB, 16QAM, Channel bits: 1656 bits, TBS: 744 bits (Effective CR: 0.46)

  - Test 2: TM1, 6RB, QPSK, Channel bits: 1656 bits, TBS: 504 bits (Effective CR: 0.32)

  - Test 3: TM1, 6RB, QPSK, Channel bits: 1656 bits, TBS: 152 bits (Effective CR: 0.10)

Issue 3-4: DL scheduling for Cat-M1 PDSCH demodulation requirements

- Agreement:

  - dropping/puncturing MPDCCH/PDSCH when MPDCCH/PDSCH subframe collides with PSS/SSS/PBCH/SI subframe

  - The following DL scheduling pattern are agreed to be the baseline and companies are encouraged to bring simulation results based on the following DL scheduling pattern.

  - Other scheduling patterns are not precluded.

    - For Cat-M1 PDSCH test 1:

    - For Cat-M1 PDSCH test 2:

    - For Cat-M1 PDSCH test 3:

Issue 3-5: Assumption for channel estimation length in NPDSCH simulation

- Agreement:

  - 1ms

  - Note: 1ms is used in simulation assumption to define minimum requirements, not to restrict UE implementation

Issue 3-6: Maximum Doppler spread for NB-IoT NPDSCH Test 1

- Agreement:

  - 200Hz

Issue 3-7: Repetition factor for NB-IoT NPDSCH Test 2

- Agreement:

  - 128

Issue 3-8: PDSCH demodulation requirements for Cat-M1 over NTN

- Agreement

| Test number | CBW | MCS | Transmission mode | PDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 1.4MHz | 16QAM 1/2 | TM1 | 1 | NTN-TDLC5- 30 | 1x1 | 8ms | TBD |
| 2 | FDD 1.4MHz | QPSK 1/3 | TM1 | 8 | NTN-TDLA100-200 | 1x1 | 8ms | TBD |
| 3 | FDD 1.4MHz | QPSK 1/10 | TM1 | 64 | NTN-TDLA100-10 | 1x1 | 8ms | TBD |

Issue 1-4-1: PDSCH demodulation requirements for NB-IoT over NTN

- Agreement:

| Test number | CBW | Carrier type | MCS | NPDSCH repetition | Propagation condition | Antenna configuration | K_offset | SNR requirements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FDD 200kHz | Anchor | QPSK 1/2 (Reuse R.NB6.FDD) | 32 | NTN-TDLC5-200 | 1x1 | 8ms | TBD |
| 2 | FDD 200kHz | Non-anchor | QPSK 1/3 (Reuse R.NB6-1.FDD) | 128 | NTN-TDLA100-10 | 1x1 | 8ms | TBD |

Topic #4: CR spilt

| Clause number | Clause name | Company name |
| --- | --- | --- |
| 8 | Demodulation Performance requirements | Huawei |
| 8.1 | General |  |
| 8.2 | Demodulation performance requirements for UE category M1 | Ericsson |
| 8.2.1 | FDD and half-duplex FDD |  |
| 8.2.1.1 | PDSCH demodulation requirements |  |
| 8.3 | Demodulation performance requirements for UE category NB1 and NB2 | MediaTek |
| 8.3.1 | Half-duplex FDD |  |
| 8.3.1.1 | NPDSCH demodulation requirements |  |
| 8.3.1.1.1 | Minimum Requirements for Standalone |  |
| Annex A (normative): | Measurement channels | Qualcomm |
| Annex B (informative) | Change history | Ericsson |
| Annex C | Downlink physical channels |  |
| Annex D | Environmental conditions |  |