3GPP TSG-RAN WG4 Meeting # 106														   R4-2303304

Athens, Greece, February 27 – March 3, 2023

Agenda item:			9.9.4

Source:	OPPO

Title:	Way forward on R18 RRM enhancement (part 2)

Document for:	Approval

- Topic #1: RRM Core requirements for FR1-FR1 DC

Sub-topic 1-1: General and TP/CR plan

Issue 1-1-1: TP/CR split for FR1+FR1 NR-DC

Agreement:

  - Agree on the work split of CRs for FR1+FR1 NR-DC RRM requirements based on the table as below.

| Potential RRM requirements for FR1+FR1 NR-DC | Impacted section/clause in TS 38.133 | Volunteer of draft CR |
| --- | --- | --- |
| Applicability rule | 3.6.2.4 | QC |
| PSCell addition/release delay and conditional PSCell addition delay | 8.2.4.1, 8.9.2 | vivo |
| Scheduling availability | 8.1.7, 8.5.7, 8.5.8, 9.5.6, | QC, OPPO, MTK, Nokia |
| CSSF | 9.1.5.1.4, 9.1.5.2.4 | OPPO |
| HO with PSCell | 6.1.5.1, 6.1.5.4 | Xiaomi |
| SCG activation/deactivation | 8.17.2 | Ericsson |

Sub-topic 1-2: Scheduling availability

Issue 1-2-1: Scheduling availability for intra-frequency measurement without MG (of UE performing measurements in TDD bands on FR1)

Agreement:

  - For intra-frequency measurement without MG,

    - For UE who does not support simultaneousRxTxInterBandCA in ca-ParametersNR-ForDC, the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

    - FFS how to capture the agreements in the spec

Issue 1-2-2: Scheduling availability for intra-frequency measurement without MG (of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1)

Agreement:

  - No need to explicitly update the spec for scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario

Issue 1-2-3: How to capture scheduling availability for RLM/BFD/CBD/L1-RSRP measurements for FR1-FR1 NR-DC in TS 38.133

Agreement:

  - Specify the scheduling availability for RLM/BFD/CBD/L1-RSRP measurements for FR1-FR1 NR-DC in TS 38.133 in existing section of FR1.

Sub-topic 1-3: Measurement availability

Issue 1-3-1: Whether to define measurement restriction for the scenario that network configures mixed numerology on two CCs while UE doesn’t support simultaneous reception for two CCs?

Agreement:

  - Measurement restriction requirement is not applicable when network configures mixed numerology on two CCs, while UE doesn’t support simultaneous reception for two CCs.

    - no need to update spec for this issue since TS38.133 section 3.6.10 has already covered it.

  - Note: above conclusion means no measurement restriction requirement is applied for the case when network configures mixed numerology on two CCs while UE doesn’t support simultaneous reception for two CCs.

Sub-topic 1-4: SCG activation/deactivation

Issue 1-4-1: Tsearch for RACH-less SCG activation

Agreement:

  - For RACH-less based SCG activation for FR1+FR1 NR-DC,

    - [if TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell]

    - FFS on requirements for additional scenarios including FR1 PSCell is unknown

Issue 1-4-2: Whether to discuss SCG activation with direct multiple SCell activation in this WI?

Agreement:

  - This issue will not be discussed in this R18 eFeRRM WI.

Issue 1-4-3: Whether to consider relax requirements on inter-frequency measurement during deactivated SCG state in R18

Agreement:

  - This issue will not be discussed in this R18 eFeRRM WI.