# 3GPP TSG-RAN WG4 Meeting # 106                              R4-2303545

# Athens, Greece, February 27 – March 3, 2022

# Source: 	ZTE Corporation, Skyworks, Huawei, Qualcomm, Murata, Xiaomi

# Title: 	TP for TR 38.872 to complete the open issues for CA_n5-n28

# Agenda item:	9.2.3.2

# Document for:	Approval

# 1	Introduction

This TP to TR 38.872 [1] aims to capture the complete open issue for CA_n5-n28 in terms of the discussion in the moderator summary [2].

# 2	Reference

[1] R4-2218418, TR 38.872 v0.3.0, CATT

[2] R4-2302817, Topic summary for [106][124] FS_NR_700800900, Moderator(CATT)

# 3	TP to TR38.872

  - 5.2		CA_n5-n28

    - 5.2.1	UE RF architecture assumption

The n5 filter parameter assumption in the analysis is based on without sharing path of band n26.

The UE RF architectures with 2 antenna and 3 antenna can be assumed in the feasibility study for CA_n5-n28.The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands. Figure 5.2.1.0-1 shows the aggregated spectrum allocation for CA_n5-n28 where no frequency range restriction has been indicated for either of the constituent bands.

Figure 5.2.1.0-1 CA_n5-n28 aggregated spectrum allocation

<Next changes>

    - 5.2.2.4	∆TIB,c and ∆RIB,c values

For CA_n5-n28 with 2 antenna implementation, the TIB,c and RIB,c values has been specified in TS 38.101-1.

Referring to contribution R4-2300657, it is suggest to reuse the DC_28_n5 TIB,c and RIB,c values with an additional 0.2dB to all to enable the two-antenna n-plexers in the future and including the support of full band n28, which are shown below:

Table 5.2.2.4-0: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.7 |
|  | n28 | 0.7 |

Table 5.2.2.4-0a: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0.2 |
|  | n28 | 0.2 |

For CA_n5-n28 with 3 antenna implementation, the TIB,c and RIB,c values are shown  below.

Table 5.2.2.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

Table 5.2.2.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n5A-n28A | n5 | 0 |
|  | n28 | 0 |

#### 5.2.2.5	REFSENS evaluation

For CA_n5-n28 with 2 antennas implementation, referring to contribution R4-2202036, the n28 MSD evaluation for CA_n5-n28 is shown as below.

Figure 5.2.2.5-1 the n28 MSD evaluation for CA_n5-n28

Based on the agreed CR R4-2206134, one MSD test configuration (n28 30MHz) can be  a reference considering the impacts from band n5 UL.

Table 5.2.2.5-1 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 17.5 | ACLR2 |

For CA_n5-n28 with 3 antennas implementation, referring to contribution R4-2301266, the n28 MSD evaluation on the same MSD test configuration (n28 30MHz) as 2 antennas implementation is shown as below.

Table 5.2.2.5-2 MSD due to cross band isolation for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 800.5 | 5 | 18.6 | ACLR2 |

<Next changes>

    - 5.2.3.3	REFSENS evaluation

Since there is no two UL IMD issue for CA_n5-n28, no need to specify the MSD due to two UL IMD interference.

Referring to contribution R4-2300657, the estimation of the n5+n28 UL interference in band n28 DL versus n5 and n28 1UL case can be found in table 5.2.3.3-1.

Table 5.2.3.3-1: Estimation of the n5+n28 UL interference in band n28 DL versus n5 and n28 1UL case

| n28DL CBW (MHz) at the top of band 28 | 5 |  | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| n28 UL transmitter output noise at 23dBm (dBm) | -58.5 |  | -52.2 | -50.0 | -42.3 | -28.1 |
| 38.101-1 n28 REFSENS de-sense due to Tx (dB) | 0.0 |  | 0.0 | 1.4 | 7.0 | 11.9 |
| n28 UL transmitter output noise at 20dBm (dBm) | -61.5 |  | -55.3 | -53.8 | -50.5 | -39.2 |
| n5 UL transmitter output noise at 23dBm (dBm) | -45.2 |  | -41.1 | -41.0 | -40.8 | -40.7 |
| n5 UL transmitter output noise at 20dBm (dBm) | -53.8 |  | -49.6 | -48.9 | -47.8 | -48.9 |
| n28UL -> n28DL quadplexer isolation (dB) | 50 |  | 50 | 50 | 50 | 50 |
| n5UL -> n28DL quadplexer isolation (dB) | 45 |  | 45 | 45 | 45 | 45 |
| n28 transmitter output noise at 23dBm (dBm) at main LNA | -108.5 |  | -102.2 | -100.0 | -92.3 | -78.1 |
| n5+n28 transmitter output noise at 23dBm (dBm) at main LNA | -90.4 |  | -94.8 | -94.1 | -93.3 | -88.0 |
| delta n5+n28 vs n28 at 23dBm output noise at main LNA | 18.1 |  | 7.4 | 5.8 | -1.0 | -9.9 |

The resulting MSD (on top of n28 without self de-sense) with MRC calculations is provided in Table 5.2.3.3-1 to enable a two-antenna architecture in the long-term.

Table 5.2.3.3-2: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 793 | 20 | [3.6] | ACLR2 |
| n28 |  | 738 | 20 | 15 | 25 (Rbstart=81) |  |  |  | ACLR2 |

Referring to contribution R4-2302097, the relationship for n28 DL impacted by both band n5 UL and n28 UL is shown in figure 1, when both band n5 UL and n28 UL are configured for CA_n5-n28, band n28 DL receiver will be impacted by the leakage interference from band n5 UL and n28 UL.

Figure 5.2.3.3-1 The relationship for band n28 DL impacted by both band n5 UL and n28 UL

Some measurement results were proposed in R4-2202036 for band n28 30MHz case. -32.4dBm Tx noise in n28 Rx band can be observed from band n5 PAout. Since 45dB Tx-Rx isolation between n5 UL band and n28 DL band was assumed, 2.9dB MSD can be observed based on n28 30MHz -78.5dBm REFSENS for 2 antenna implementation, which is shown in Table 5.2.3.3-3:

Table 5.2.3.3-3: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 2.9 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |

Referring to contribution R4-2302253, the most straightforward way would be to use the same test point for 2UL MSD due to Cross-Band Isolation as has been used for 1UL Cross-band Isolation for 2 antenna implementation, which is shown in Table 5.2.3.3-4:

Table 5.2.3.3-4: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (Rbstart=0) | 800.5 | 5 | 17.5 | n5 ACLR2 |
| n28 |  | 5 | 5 | 15 | 25 (Rbstart=0) |  |  |  |  |

Referring to contribution R4-2218129, for 2UL MSD due to Cross-Band Isolation as has been used for 1UL Cross-band Isolation for 3 antenna implementation is shown in Table 5.2.3.3-5. It includes a legacy filter rejection for n5 TX OOB leakage in n28DL. The antenna isolation helps attenuate the TX leakage to give a similar MSD result specified in Table 5.2.3.3-3 for a 30MHz DL.

Table 5.2.3.3-5: 2UL cross band MSDs for CA_n5-n28

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n28 | 834 | 20 | 15 | 20 (RBstart=0) | 788 | 30 | 4.5 | ACLR2 from n5 UL band ACLR1+ACLR2 from n28 UL band |
| n28 |  | 733 | 30 | 15 | 25 (RBstart=135) |  |  |  |  |