3GPP TSG-RAN WG4 Meeting #106												R4-2303711

Athens, Greece, 27 February –03 March, 2023

Title: 	WF on LP-WUS UE RF

Agenda Item:	9.20.4

Source: 	vivo

Document for:	Approval

Topic #1: General and Workplan

Issue 1-1: SI Workplan

Agreements:

  - Approve the proposed RAN4 workplan for Rel-18 low-power Wake-up Signal and Receiver SI, in R4-2301565.

Issue 1-2-1: management on Reply LS to RAN1

Agreements:

  - Aim to provide feedback to RAN1 in multiple batches aligned with RAN4 progress instead of waiting until all answers are fully complete.

  - Agreements and clarification issues can be sent in reply LS.

Topic #2: LP-WUR architectures

- Sub-topic 2-1 General for WUR architecture

Issue 2-1-1: General views on WUR

Agreements:

  - RF requirement study in RAN4 for WUR evaluation

  - FFS criteria (e.g. sensitivity, SNR and power consumption) for architecture evaluation

Issue 2-1-3: Down-selection of WUR architectures in RAN4

Agreements:

  - Pros and Cons of each architecture from RAN4 perspective can be replied to RAN1

  - Further discuss whether RF-ED could be down-scoped

Issue 2-1-4: Frequency range

Agreements:

  - Both FR1 and FR2 are considered in the scope of SI

Issue 2-1-5: Number of RX chain for LP-WUR

Agreements:

  - RAN4 assume 1RX architecture for LP-WUR as starting point

- Sub-topic 2-2 UE ACS evaluation

Issue 2-2-1: ACS evaluation for LP-WUR in RAN4

Agreements:

  - RAN4 should study a new approach to evaluate ACS for LP-WUS.

    - Option 1: Study feasible ACS and ASCS for each architecture, based on assumed typical filter characteristic (e.g. filter order and cut-off frequency) and LP-WUS guard band design

    - Option 2: Average selectivity against adjacent subcarriers and/or adjacent channels as well as resulting SINR of the wanted signal at detector input can be used to evaluate and compare different RF architectures from selectivity perspective.

    - Other options are not precluded

  - FFS whether similar coverage compared to main radio for LP-WUS can be assumed.

Issue 2-2-2: Starting point for ACS

Agreement:

  - Values from current UE specifications are used as a starting point for discussion to evaluate LP-WUR performance

- Sub-topic 2-3 UE Adjacent Sub-Carrier Selectivity (ASCS) evaluation

Issue 2-3-2: Adjacent subcarrier impacts

Agreement:

  - 	Consider 1.4MHz and 5MHz WUS bandwidth for FR1 evaluation as the starting point

    - FFS on how many LP-WUS RBs can be allocated in channel bandwidth

    - Guard band if needed, can be located within 1.4MHz and 5MHz RF bandwidth

Issue 2-3-3: WUS location within the carrier

Agreements:

  - Study whether LP-WUS could be flexible located within the carrier

  - FFS whether WUS can be located in a band separate from the UE’s NR band

Issue 2-3-4: Guard band for WUS

Agreements:

  - Guard band between LP-WUS and NR signals could be studied

    - Details and definition of guard band for LP-WUS need further study

  - Guard band design would be depending on WUR filter parameters, WUS CBW, SNR/SINR

  - FFS Whether the guard band is recommended to RAN1

- Sub-topic 2-4 UE Noise figure and impacts of RF impairments

Issue 2-4-1: General views on NF

Agreements:

  - RAN4 may recommend different NF value for different LP-WUR architecture

  - NF evaluation may consider SNR, sensitivity, power consumption

- Sub-topic 2-5 BS RF impacts

Issue 2-5-1: General BS RF

Agreements:

  - No impact of LP-WUS on the existing gNB emissions and compliance requirements is baseline

Issue 2-5-2: LP-WUS power boosting

- Proposals

  - Whether RAN4 should study feasible LP-WUS power boosting level.

Agreements:

  - FFS

- Sub-topic 2-6 Multi-band capability

Issue 2-6-1: RF envelop detection

Agreements:

  - RF envelop detection architecture is more appropriate for single-band operation

Issue 2-6-2: IF envelop detection

Agreements:

  - IF envelop detection is more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received.

Issue 2-6-3: BB envelop detection

Agreements:

  - BB envelop detection is more appropriate for multi-bands operation. Multi-band here still means that only one band at a time is being received.

- Sub-topic 2-7 SNR evaluation

Issue 2-7-1: SNR evaluation factors

- Proposals

  - Frequency error and ADC resolution as affecting factors should be included in the SNR evaluation.

Agreements:

  - FFS

Issue 2-7-2: Simulation assumption for SNR

- Proposals

  - Option 1: It is proposed to agree on the above simulation assumptions.

- Table 2: Simulation assumptions for SNR evaluation

| Parameters | Assumptions | Note |
| --- | --- | --- |
| Modulation | OOK/FSK/existing OFDMA signals(channels) | pending on RAN1 progress |
| Bandwidth | 4RB |  |
| Data rate | 14 kbps/28 kbps/56 kbps/84kbps |  |
| TB size | 48 bit |  |
| Manchester code | ½ | optional, depends on modulation |
| SCS | 15 kHz, 30 kHz | depends on operating bands |
| Channel model | AWGN |  |
| Antenna configuration | 1x1 |  |
| Test metric | 1% BLER | similar to PDCCH |
| ACI | 4 RB adjacent signal with 16QAM modulation | optional, for frequency error evaluation |
| Guard band | 12SC on both sides | optional, for frequency error evaluation |
| IF Filter | Butterworth 5th order Passband width = 1.44 MHz | optional, for frequency error evaluation |
| Frequency offset | 0/15kHz/60kHz/150kHz | optional, for frequency error evaluation |
| ADC bits | 2, 4 |  |

Agreements:

  - FFS