3GPP TSG RAN WG4 Meeting #106bis-e  	                                                                   R4-23xxxx

Online, April 17 – April 26, 2023

Title:	Reply LS to RAN1 on low-power wake-up receiver architectures

Release:	Rel-18

Work Item:	FS_NR_LPWUS

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

In this meeting, to evaluate the RF impacts of LP-WUR architecture, RAN4 reach further basic agreements for next-step evaluation, some of ACS/ASCS and guard RBs related agreements have been attached in the Annex.

In addition, RAN4 also has the following question:

- Whether the case when the WUS/WUR is same as NR channel bandwidth, e.g. 5MHz WUS within 5MHz NR CBW (Max 25 RBs/15kHz SCS), is considered for LP-WUS/WUR evaluation.

2	Actions

To RAN1:

ACTION: 	3GPP RAN4 respectfully asks RAN1 to take the above feedback into account.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #107                           	 22-26 May, 2023    		     Incheon, KR

TSG-RAN WG4 Meeting #108                           	 21-25 August, 2023    		Toulouse, FR

Annex (Agreements for ACS/ASCS and Guard RBs evaluation) needs update as same as final WF

Issue 2-2-1: Guardgap definition for LP-WUS

Agreements:

- RAN4 use guard RBs (if needed) for LP-WUS, which is Granularity of RB. The traditional guardband for NR channel bandwidth defined in TS 101-1 should not be changed.

  - For case when WUS is smaller than NR channel bandwidth

    - For case 2-1, the LP-WUS guard RB is number RBs between LP-WUS and NR signals (edge of WUR RB location to nearest edge of eMBB RB)

    - For case 2-2, the WUS is placed at the edge of the NR channel bandwidth, i.e. the lowest/highest RB of WUS with guard RBs is aligned with the lowest/highest NR transmission bandwidth configuration in spec TS 38.101-1.

  - [For case when the WUS/WUR is same as NR channel bandwidth]

    - For case 1, the LP-WUS guard RBs is number RBs between LP-WUS and traditional guardband (edge of WUR RB location to Outermost of NRB)

    - RAN4 should further check with RAN1 for this case

- FFS whether the guard RBs should be symmetric within the WUS channel bandwidth.

Issue 2-2-2: Whether guardgap is needed for LP-WUR

Agreement:

- How many RBs for guard is FFS. RAN4 should further evaluate this number based on the cases identified in issue 2-2-1.

- The size of guard RBs from implementation perspective for LP-WUS should be determined in RAN4.

Sub-topic 2-3 UE Adjacent Carrier/Sub-Carrier Selectivity (ACS/ASCS) evaluation

Issue 2-3-1: General evaluation framework for both ACS and ASCS

Agreement:

- 	The following aspects can be starting point for further discussions

- Framework in RAN4 that the ACS and ASCS value can be evaluated based on the following aspects:

  - Typical filter characteristic, e.g. filter order, pass BW, cut-off frequency

  - Guard RB size within LP-WUS channel bandwidth

  - RF impairment can also be considered

- Averaged power antennation at ACS or ASCS frequency range

- FFS whether SINR of the wanted signal at detector input is needed

- FFS whether use ICS to instead ASCS

- Coexistence-simulation-based framework can also be considered

  - FFS on details of coexistence study (if needed) of LP-WUS

  - Coverage should be considered

Issue 2-3-2: LP-WUS evaluation scenarios for study purpose

Agreements:

- Consider a limited set of WUS scenarios in table below for study purpose in RAN4

Table 1: LP-WUS evaluation scenarios

| NR RF channel BW | Decided in 2nd round |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | 1.44MHz, 5.04 MHz |
| WUS RB allocation (Note 1) | [6] RB in 1.44 MHz, total 8 RBs, or other number of RBs [24] RB in 5.04 MHz, total 28 RBs, or other number of RBs |
| WUS placement within NR channel | 3 cases: - case 1: Center; - case 2: edge; - case 3: between center and edge of NR channel |
| Guard RB size of LP-WUS | - 0 RB, 1RB at each side, 2RBs at each side, or other number of RBs. - Asymmetric guard RB can also be considered |
| ACS interferer | According to RF CBW |
| Filter characteristic | Lowpass, 2nd to 5th order Butterworth Both analog and digital filter can be considered |
| Filter passband BW | At least WUS bandwidth (number of RBs), depends on guard RB size |
| LO frequency | Case 1: In the middle of WUS (modeling fixed WUS position) Case 2: In the middle of RF channel (modeling flexible WUS location) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| RF impairment | FFS |
| Note 1: the maximum number of allocated WUS RBs, depends on how many Guard RBs are needed. 5MHz WUS within 5MHz NR CBW is not considered currently. |  |

Sub-topic 2-5 BS RF impacts

Issue 2-5-1: Whether and which power boosting level RAN4 should study LP-WUS power boosting

Proposals:

  - Option 1: RAN4 should study the power boosting assumed by RAN1, to check whether the values are feasbible from RAN4 perspective

  - Option 2: other

Proposed Agreements:

  - Option 1

Issue 2-5-3: other gNB impacts

Agreements:

- RAN4 recommends RAN1 to prioritize signal design which allow re-use of current gNB HW.