3GPP TSG-RAN WG4 Meeting #107	R4-2308266

Incheon, KR, May 22nd – May 26th, 2023

Agenda Item:	8.20.2.2.3

Source:	vivo

Title: 	TP to TR 38.858 on Feasibility of FR1 UE aspects

Document for:	Discussion

- Introduction

In RAN4#106bis-e, there was one TP [2] endorsed on feasibility of FR1 UE aspects for TR 38.858 [1]. However, after reviewing the content for FR1, there was some misalignment with the latest agreement in [3]. In this contribution, we propose some changes based on the endorsed TP [2].

The reasons for changes are listed as follows:

- In Table 10.6.1.0-1, for adjacent channel interference models, power dependent ACLR is referred to section 10.6.1.1.2 for the Tx side; for the Rx side, it is ‘Adjacent channel selectivity’ instead of ‘Subband adjacent channel selectivity’;

- The subband selectivity needs to be aligned with the latest agreement in [3]:

- Subband in-channel selectivity is the ratio of the interference power in the assigned downlink subband to the received jammer power in the adjacent uplink subband, as measured after the FFT operation

- For ‘10.6.1.1.4 Discussion process’ and ‘10.6.1.1.4.1 RAN4 Agreement’, we suggest to move these sections in the Annex if some RAN4 agreements need to be captured.

The changes for the TP contents are made with revision marks.

- Text Proposal

<<Start of Change for TR 38.858>>

## 10.6. FR1 Feasibility of UE aspects

### 10.6.1	Interference analysis

#### 10.6.1.0 General

In the objective of this study item, half duplex operation at UE side is assumed. In this part of feasibility of UE aspects, FR1 is considered.

In the UE feasibility study, two types of UEs are considered, legacy UE and SBFD-aware UE. For legacy UE, the current UE RF architecture can be assumed without any RF architecture improvement. In the feasibility study for legacy UE, some typical RF performance along with some current RF requirements from TS 38.101-1 have been used. The SBFD-aware UE supports half duplex operation just as the legacy UE, however it receives the sub-band configuration from the network. For this kind of UE, the RF architecture and applicable RF requirements need further study in future releases. In the following study, legacy UE is the main focus.

In the UE feasibility study in FR1, the co-channel inter-subband UE-UE CLI model and adjacent channel UE-UE CLI model are mainly discussed. Co-channel/adjacent channel interference models at the UE side are summarized in Table 10.6.1.0-1. For co-channel models, the UE IBE model can be used for Tx side; for receiver sub-band/in-channel selectivity, no rejection/attenuation due to RF/BB filtering is assumed. For adjacent channel models, UE ACLR and selectivity can be used as Tx and Rx side, respectively.

Table 10.6.1.0-1. Existing UE interference models based on RF requirements in RAN4

| Co-channel RF interference models |  | Adjacent channel RF interference models |  |
| --- | --- | --- | --- |
| Tx side | Rx side | Tx side | Rx side |
| UE IBE for Tx | Subband/In-channel selectivity (Note 1) | Power dependent ACLR as described in Section 10.6.1.2.2 | Adjacent channel selectivity |
| Note 1. For legacy UE, there is no UE RF requirement for Sub-band/in-channel selectivity. It is only used in SBFD feasibility study purpose. |  |  |  |

#### 10.6.1.1	UE-UE co-channel inter-subband CLI modeling

Editor's note: This section captures the CLI modeling.

- 10.6.1.1.1 Overview and analysis framework

The objectives of UE-UE co-channel inter-subband CLI modeling is to analyze the impact of interference that occurs between two UEs in close proximity, operating on adjacent sub-bands within the same channel. This interference occurs when UL transmission of an aggressor UE in the channel interferes with the DL reception of a victim UE in the same channel.

For this SI RAN4 has decided to use typical UE parameters in the analysis, as opposed to worst-case parameters most often used for minimum performance requirements.  We are considering what improvements may be needed for gNB performance, and those should be considered with a population of typical UEs, not worst case UEs.

[Subband in-channel selectivity is the ratio of the interference power in the assigned downlink subband to the received jammer power in the adjacent uplink subband, as measured after the FFT operation.] In an ideal scenario, the UL transmission of the aggressor UE should not impact the DL reception of the victim UE due to the OFDM wave orthogonality. However, non-ideal FFT suppression can cause interference to the victim UE, particularly when the UL sub-band has frequency errors and is not time-synchronized with the DL sub-band. The analysis indicates that the IBE interference is higher and dominates the sub-band co-channel selectivity, and frequency and time offset are not significant factors influencing UE-UE interference. It is worth noting that the RF degradations can cause inter-subband interference as well and the impact will depend on the targeted Rx IM and EVM performance. The measurement data submitted by [one] company shows the subband selectivity of FR1 UE can be 33 dB. Nonetheless, this interference will not be any worse than the selectivity value. For this reason, the 33 dB was agreed for modeling the inter-sub-band selectivity.

For legacy UE, no sub-band filtering is considered.

To model the AGC and NF modeling for co-channel CLI in a system level simulation, a fixed value noise figure is used.

Apart from the selectivity, it is important to mention that degradation can be caused by transmitter leakage from the UL sub-band into the DL sub-band. For co-channel case, the leakage was agreed to be modelled using IBE based model with granularity of 1 RB. Additionally, the IQ image contribution for the IBE model for co-channel CLI can be ignored for the DUD configuration since the image is fully contained in the uplink sub-band.

- 10.6.1.1.2 UE co-channel Tx model

In-band emissions

For UE co-channel Tx model, RAN4 has decided to use the IBE requirements from 38.101-1 clause 6.4.2.3 as shown in Table 10.6.1.1-1 in the feasibility study. This model consists of three parts, General, IQ image, Carrier leakage. In the system level simulation, the general and IQ image parts shall be considered, while the carrier leakage part can be ignored in the feasibility study. For DUD configuration, the IQ image from the uplink is fully contained in the UL sub-band and does not land in the DL subband, thus the IQ image can also be ignored in the simulation. The granularity of this model is 1RB and it is not pursued to simplify this model to a frequency flat model. It is understood these requirements are minimum performance requirements as opposed to typical requirements. RAN4 has agreed to use typical requirements for the UE parameters, however, did not conclude on the typical values so we are using the formulation from the MPS.

Table 10.6.1.1-1: Requirements for in-band emissions in TS 38.101-1

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |

It should also be assumed the LO location is in the center of the channel for the purposes of system studies in RAN4.The LO location is important as it allows placement of the image.

- 10.6.1.1.3 UE co-channel Rx model

For UE co-channel Rx model, currently there is no corresponding RF requirement for this model. In the feasibility of UE co-channel Rx model, the definition of Sub-band/In-channel selectivity is introduced for SBFD feasibility study purpose:

- Subband in-channel selectivity is the ratio of the interference power in the assigned downlink subband to the received jammer power in the adjacent uplink subband, as measured after the FFT operation

For legacy UE, no sub-band filtering is considered, and no rejection/attenuation due to RF/BB filtering is assumed and only the selectivity and performance of the FFT is studied. For now, 33 dB was agreed for FR1 in RAN4 for subband/in-channel selectivity considering FFT operation.

For new SBFD aware UE, it needs further study on whether sub-filtering can be considered or not. The subband/in-channel selectivity needs further study.

Thermal self-noise performance

RAN4 decided on a simple fixed-value noise figure model for the UE receiver. Generally, the receiver noise figure will vary with the input power level, however the single value noise figure model was considered to be sufficient for the purpose of system studies for SBFD. RAN4 decided on a NF of 9 dB.

Effect of jammer – non-thermal self-noise aspect

There are a few factors to consider in determining the in-subband interference in the presence of a co-channel jammer. With an in-channel adjacent-subband interferer the 3rd order distortion, reciprocal mixing, residual sideband, quantization noise, phase noise, ADC distortion, and analog filtering should be considered.

Measurements were made of a UE receiver for various signal levels, interferer levels, interferer offsets, sub-band bandwidths, and interferer bandwidths. The measurements included the entire receiver, which includes everything through the FFT. 120 various conditions were measured.

We find that the interference in the victim sub-band can be modelled as [20 to 33] dB below the total input power level. Interference is approximately frequency flat across the victim.

FFT leakage and selectivity

In the SBFD system an aggressor UE (UE2) operating in the UL sub-band may interfere with a UE (UE1) receiving in the adjacent DL sub-band. The UE2 UL may arrive at UE1 misaligned in time or frequency, and potentially causing UE1 DL SINR degradation in the FFT.

We simulated and OFDM waveform begin converted to spectrum with an FFT, injecting timing and frequency errors. We assumed a 5RB guard band. We found the major contributor to spectral leakage to be time-misalignment, and even small timing errors produce leakage.

It appears reasonable to consider the leakage as a single average value, the data shows about 33 dB down from the jammer level would be appropriate. We should also consider the leakage effect and compare it to the aggressor IBE-based interference. IBE interference is higher than the FFT leakage so RAN4 has decided to exclude any factor for FFT leakage. Further, RAN4 has concluded no factor is needed for FFT selectivity.

Figure 10.6.1.1.3-1: FFT leakage with time and frequency misaligned blocker (5 RB guard band)

- 10.6.1.2  UE-UE adjacent channel CLI modelling

Editor's note: This section captures the CLI modeling.

- 10.6.1.2.1 Overview

The UE-UE adjacent channel CLI occurs when the UL transmission of the aggressor UE in a channel interferes with the DL reception of the victim UE in an adjacent channel. Unlike the case of co-channel interference, there is no need to consider any FFT selectivity in the adjacent channel scenario. Apart from the selectivity, it is necessary to consider leakage by the transmitter from the UL sub-band into the DL sub-band. It was decided to assume the power-dependent ACLR of the aggressor UE and selectivity of the victim UE when modeling adjacent channel interference.

To model the AGC and NF modeling for adjacent channel CLI in a system-level simulation, a fixed value noise figure shall be used. Additionally, UE ACLR should be modeled as 30 dB at max power, improving 1 dB/dB with back-off up to a maximum of 10 dB of improvement. Therefore, when the back-off is 10 dB, the ACLR is 40 dB.

- 10.6.1.2.2 UE adjacent channel Tx model

UE adjacent channel leakage ratio is used in the feasibility study for adjacent channel UE-UE CLI Tx model. Only one-step of ACLR shall be considered in the study item and two-step ACLR was precluded. In the UE Tx model, only power class 3 was assumed with 30dB ACLR considering a fully allocated uplink subband. Partially allocated UL subband was not considered in the system simulation. This ACLR model can be seen as frequency flat model, and the distortion is modelled as a flat power spectral density across the frequency range of the distortion. In the simulation, improved ACLR with power backoff is considered.

- 10.6.1.2.3 UE adjacent channel Rx model

UE adjacent channel selectivity (33dB for FR1) is used as adjacent channel UE-UE CLI model under the assumption that the blocker from adjacent channel does not exceed the maximum input level (-25 dBm) for UE. If the blocker is higher than -25dBm, it is assumed it will result large receiver degradation and hence the RX will not correctly decode the data (100% packet loss).

### 10.6.2	Summary

Editor's note: This section captures the conclusion of feasibility.

[

For co-channel interference case, the RF effect is dominant, and the frequency and time offset are not significant factors influencing UE-UE interference. The leakage is modelled using IBE based model.

As for the adjacent channel case, it was decided to assume power-dependent ACLR of the aggressor UE and selecitivty  of the victim UE when modeling adjacent channel interference.

For legacy UE, no sub-band filtering is considered,

A fixed value noise figure shall be used to model the AGC and NF modeling for co-channel and adjacent channel CLI in a system level simulation.

]

<<End of Change>>

References

- R4-2302887 3GPP TR 38.858 V0.4.0

- R4-2305970, TP to TR 38.858 on Feasibility of FR1 UE aspects, MediaTek, RAN4#106bis-e.

- R4-2305969, WF for SBFD feasibility study and requirements impact from UE aspect, Qualcomm, RAN4#106bis-e.