3GPP TSG-RAN WG4 Meeting #107	R4-2309807

Incheon, Korea (Republic of), May 22nd ‒ May 26th, 2023

Title: 	WF for NonCol_intraB_ENDC_NR_CA_Demod

Agenda Item:	8.12.5

Source: 	Ericsson

Document for:	Approval

# Topic #1: Test scope

## Sub-topic 1-1 Test scope

UE type assumption

Agreement:

- RAN4 should discuss the need for demodulation requirements only for Type-2 UEs.

  - Preclude Type-3a/3b and Type-4a/4b UE from the UE demodulation performance requirements discussion.

# Topic #2: Type 2 UE demodulation requirements

## Sub-topic 2-1 Test case for Type 2 UE demodulation requirements

Whether to define new Type 2 UE demodulation requirements in TS38.101-4

Agreement:

- Define new Type 2 UE demodulation requirements with NR-CA scenario only.

Test setup for Type 2 UE demodulation requirements

Agreement:

- Assume the power difference of 25dB and received time difference of 33us under the assumption that requirements introduced under static channel.

Test method for Type 2 UE demodulation requirements

Way forward:

- For NR-CA measure both PCell and SCell at same time.

- Channel model: AWGN

  - FFS for Test criteria: Reuse X% of the maximum throughput

  - FFS for How to select MCS for each carrier

- Interested companies are encouraged to propose test methods of UE demodulation requirements for NR-CA to verify Type 2 UE features.