3GPP TSG-RAN WG4 Meeting # 107	R4-2309895

# Incheon, KR, May 22 – 26, 2023

Title:	LS on required DCI signalling for advanced receiver on MU-MIMO scenario

Response to:

Release:	Rel-18

Work Item:	NR_demod_enh3-Perf

Source:	RAN WG4

To:	RAN WG1

Cc:	RAN

Contact person:	Chu-Hsiang Huang, chuhsian@QTI.QUALCOMM.COM

Jingzhou Wu, wujingzhou@CHINATELECOM.CN

Ke Li, like54@hisilicon.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

Within the Release 18 work item on NR demodulation performance evolution (NR_demod_enh3), RAN4 has studied the required signalling overhead for the advanced receiver to cancel inter-user interference for MU-MIMO. 2 candidate advanced receivers, E-MMSE-IRC and R-ML, are included in the study.

Based on RAN4’s evaluation, RAN4 observes that R-ML receiver can achieve better performance in most scenarios. To enable the implementation of R-ML receiver within feasible complexity, RAN4 has agreed that it is beneficial to have DCI based network assistant signalling to know the essential information related to the interfering layers associated with the co-scheduled UE(s).

| Bit field mapped to index | Content |
| --- | --- |
| 0 | No co-scheduled UE(s) which has same DMRS sequence as target UE exists |
| 1 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have QPSK scheduled |
| 2 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 16QAM scheduled |
| 3 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 64QAM scheduled |
| 4 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 256QAM scheduled |
| 5 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 1024QAM scheduled |
| 6 | Not covered by cases corresponding to index 0~5. In each individual PRB allocated to the target UE, the following condition is satisfied: Only single modulation order is allocated for the co-scheduled UE(s) which has the same DMRS sequence as the target UE, if the co-scheduled UE(s) exist |
| 7 | Others |

- The existence of MU-MIMO DCI signalling is configured by RRC signalling.

- The field is intended to be included in a DCI which can be based on the format 1_1.

2	Actions

To RAN WG1

ACTION:

RAN4 kindly request RAN1 to take the above RAN4 agreements into consideration, and introduce DCI based signalling discussed in section 1 if feasible, to reduce UE complexity and achieve better UE performance.

3	Dates of next RAN WG 4 meetings

TSG RAN WG4 Meeting #108			21st – 25th Aug. 2023   	    	Toulouse, France

TSG RAN WG4 Meeting #108-bis			9th – 13rd Oct. 2023   	    	Xiamen, China