3GPP TSG-RAN WG4 Meeting #107	R4-2310040

Incheon, Korea, May 22 ‒ May 26, 2023

Title: 	Summary of RRM simulation results for less than 5MHz BW

Agenda Item:	8.15.6

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Information

# Introduction

This paper collects the simulation results provided by companies in RAN4#107 meeting related to WI NR_FR1_lessthan_5MHz_BW in [REF].

The results are for information.

# Simulation results

## PBCH decoding, TDL C 300ns

Table 1 99% PBCH decoding, SSB samples or SINR needed TDL C 300ns

|  | SINR of one shotneeded for 12 PRBs | Number of attempts needed for 12 PRBs, SINR-6dB | Number of attempts needed for 12 PRBs, SINR-4dB | SINR of one shotneeded for 15 PRBs | Number of attempts needed for 15 PRBs, SINR-6dB | Number of attempts needed for 15 PRBs, SINR-4dB | SINR of one shotneeded for 20 PRBs | Number of attempts needed for 20 PRBs, SINR-6dB | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | >2dB | 5 | 3 | 0dB | 3 | 2 | - | - | - |
| Huawei | 11dB | - | - | - | - | - | 3dB | - | - |
| Ericsson | - | - | - | -1dB | 5 | 4 | - | - | - |
| MTK | >0dB | NA, >4 | 4 | - | - | - | -3dB | 3(Soft combing) | 2 |
| Nokia | - | 2(Soft combing) | - | - | 2(Soft combing) | - | - | 1 | 1 |
| Apple | - | - | - | - | - | - | - | - | - |

## PBCH decoding, HST

Table 2 99% PBCH decoding, SSB samples or SINR needed HST

|  | SINR of one shotneeded for 12 PRBs | Number of attempts needed for 12 PRBs, SINR-6dB | Number of attempts needed for 12 PRBs, SINR-4dB | SINR of one shotneeded for 15 PRBs | Number of attempts needed for 15 PRBs, SINR-6dB | Number of attempts needed for 15 PRBs, SINR-4dB | SINR of one shotneeded for 20 PRBs | Number of attempts needed for 20 PRBs, SINR-6dB | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | -2dB | 3 | 2 | -6dB | 1 | 1 | - | - | - |
| Huawei | - | - | - | - | - | - | - | - | - |
| Ericsson | -4.5dB | 2 | 1 | -7dB | 1 | 1 | - | - | - |
| MTK | -3dB | 2(Soft combing) | 2 | - | - |  | - | - |  |
| Nokia | -2.5dB | 6 (3 Soft combing) | 2 | -6dB | 1 | 1 | -8.5dB | 1 | 1 |
| Apple | - | - | - | - | - | - | - | - | - |

## PBCH decoding, AWGN

Table 3 99% PBCH decoding, SSB samples or SINR needed AWGN

|  | SINR of one shotneeded for 12 PRBs | Number of attempts needed for 12 PRBs, SINR-6dB | Number of attempts needed for 12 PRBs, SINR-4dB | SINR of one shotneeded for 15 PRBs | Number of attempts needed for 15 PRBs, SINR-6dB | Number of attempts needed for 15 PRBs, SINR-4dB | SINR of one shotneeded for 20 PRBs | Number of attempts needed for 20 PRBs, SINR-6dB | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | -4dB | 2 | 1 | -6dB | 1 | 1 | - | - | - |
| Huawei | - | - | - | - | - | - | - | - | - |
| Ericsson | -5dB | 2 |  | - | - |  | - | - | - |
| MTK | - | - | - | - | - | - | - | - | - |
| Nokia | -4dB | 4 (2 Soft combing) | 1 | -7dB | 1 | 1 | -9dB | 1 | 1 |
| Apple | - | - | - | - | - | - | - | - | - |

# References

- R4-2309702, RRM requirements for NR less than 5MHz, Qualcomm Incorporated.

- R4-2308678, Discussion on RRM impacts for less than 5MHz BW, Huawei, HiSilicon

- R4-2309592, On RRM requirements for less than 5MHz, Ericsson

- R4-2309573, Discussion on issues for less than 5MHz, MediaTek inc.

- R4-2309659, RRM simulation results for less than 5MHz BW, Nokia, Nokia Shanghai Bell

- R4-2307324, RRM for spectrum less than 5MHz, Apple