3GPP TSG-RAN WG4 Meeting #107 	R4-2307025

Incheon, Korea, May 22 – 26, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Supporting Simulation results for PDSCH demodulation for 8Rx

Agenda item: 	8.5.3.1.2

Document for: 	Discussion

Introduction

The achieved agreements from previous RAN4 meetings (RAN4#105, RAN4#106, RAN4#106-bis-e) are detailed in [1],[2], and [3].

In this contribution, we provide the simulation results for 8 Rx PDSCH UE demodulation requirements. Discussion, observations, and proposals will be made in our companion TDoc [4].

PDSCH simulation results

Test parameters

We provide in this section the initial simulation results for PDSCH based on simulation assumption showed in Table 1.

Table 1: Simulation assumption for PDSCH

| Simulation parameters |  |  |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD pattern |  | 7D1S2U |
|  |  | S=6D:4G:4U (Not schedule PDSCH in special slot for 8 layers cases) |
| SCS(kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A and B |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% |
|  |  | 256QAM:3% |
| Test metric |  | • Only 70% of maximum TP |
|  |  | Rank 2, 4 and 8 |
| Antenna configuration |  | • Rank 2: 2T8R |
|  |  | • Rank 4: 4T8R |
|  |  | • Rank 8: 8T8R |
| Antenna correlation |  | • Rank 2: ULA Medium B |
|  |  | • Rank 4 and 8: ULA Low |
| Propagation conditions |  | • Rank 2: TDLC300-100, TDLA30-10 |
|  |  | • Rank 4 and 8: TDLA30-10 |
| MCS |  | Rank 2: MCS19 Rank 4: MCS26 Rank 8 MCS13 |
| Overhead |  | Overhead = 0 |
| TRS configuration |  | • Configure TRS symbols in symbol#{5, 9} |
|  |  | • NZP CSI-RS configuration of row 6 for 8 layers. |
| N1 and N2 configuration for 8Tx case |  | (N1,N2)=(4,1) |

Simulation results: SNR 70% throughput

Table 2 provides the SNR at which 70% throughput is achieved using the simulation parameters presented in the previous section.

Table 2: SNR at 70% for different number of layers and MCS (MCS table 2).

| Rank | MCS | Channel Correlation | Propagation Condition | SNR @ 70% of max TP |
| --- | --- | --- | --- | --- |
| 2 | 2 (Table 2) | ULA Medium B | TDLA 30-10 | -5.34 |
|  |  |  | TDLC 300-100 | -5.67 |
|  | 7 (Table 2) |  | TDLA 30-10 | 0.92 |
|  |  |  | TDLC 300-100 | 0.75 |
|  | 19 (Table 1) |  | TDLA 30-10 | 4.84 |
|  |  |  | TDLC 300-100 | 5.31 |
| 4 | 13 (Table 1) | ULA Low | TDLA 30-10 | 3.29 |
|  | 17 (Table 1 |  |  | 6.12 |
|  | 26 (Table 1) |  |  | 13.41 |
| 8 | 13 (Both CW) (Table 1) |  |  | 8.39 |
|  | 17 (Both CW) (Table 1) |  |  | 12.69 |

Conclusion

In this contribution we provided the results of our simulation campaign on 8 Rx for different number of layers and MCS to support the definition of UE demodulation performance requirements for 8Rx.

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2307024 “Discussion on PDSCH Demodulation Requirements for 8Rx”, Nokia, Nokia Shanghai Bell, RAN4 #107, Incheon, Korea, May 22 – 26, 2023