3GPP TSG-RAN WG4 Meeting #107 	R4-2307028

Incheon, Korea, May 22 – 26, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Simulation results of 8Rx CQI requirements

Agenda item: 	8.5.3.1.4

Document for: 	Discussion

Introduction

The achieved agreements from previous RAN4 meetings (RAN4#105, RAN4#106 and RAN4#106-bis-e) are detailed in [1],[2] and [3].

In this paper we provide CQI simulation results to support the 8Rx CQI discussions.

Discussion

In this section we present the simulation results for CQI based on simulation assumption shown in Table 1.

Table 1: Simulation assumption for CQI

| CQI |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| Bandwidth |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  | 7D1S2U S=6D+4G+4U |  |  |  |
| SNR |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  | static channel model with 4T8R |  |  |  |
| Antenna configuration |  |  | XP 4×8 |  |  |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 in 38.101-4 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 |  |  |  |
|  | CSI-RS | slot | 10-janv |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |  |  |
|  | CDM Type |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4,(0) |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig | slot | 10-janv |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping |  | (4, 9) |  |  |  |
|  | (kCSI-IM,lCSI-IM) |  |  |  |  |  |
|  | CSI-IM timeConfig | slot | 10/1 |  |  |  |
|  | periodicity and offset |  |  |  |  |  |
| ReportConfigType |  |  | Periodic |  |  |  |
| CQI-table |  |  | Table 2 |  |  |  |
| reportQuantity |  |  | cri-RI-i1/CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  | Wideband |  |  |  |
| Sub-band Size |  | RB | 16 |  |  |  |
| Csi-ReportingBand |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  | slot | 10-sept |  |  |  |
| aperiodicTriggeringOffset |  |  | Not configured |  |  |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  | Codebook Mode |  | 1 |  |  |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | -2,1 |  |  |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | -4,1 |  |  |  |
|  | CodebookSubsetRestriction |  | The codebook is restricted to i1,1=0, i1,2=0,i1.3=0 and i2=0 |  |  |  |
|  | RI Restriction |  | 1000 |  |  |  |
| Physical channel for CSI report |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  | 1 |  |  |  |
| Measurement channel |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |
| - (N1,N2)=(2,1); (O1,O2)=(4,1) |  |  |  |  |  |  |
| - ReportQuantity: CRI/Ri/i1/CQI |  |  |  |  |  |  |
| - Two-one-TypeI-SinglePanel-Restriction: 0b00000001 |  |  |  |  |  |  |
| - TypeI-SinglePanel-codebookSubsetRestriction-i2: 0x0000000000000001 |  |  |  |  |  |  |

Table 2 presents the results of our simulations for Median CQI with 4T8R antenna configuratio, with 40MHz BW and 30kHz SCS.

Table 2 : Simulation Results for 8Rx CQI

| CQI TDD FR1 (static condition) | SNR | CQI | Count | Percent | Median | Prob (Median CQI-1<CQI< Median CQI+1) |
| --- | --- | --- | --- | --- | --- | --- |
| CQI TDD 40M 30kHz FR1 4T8R UE | 18 | 12 | 3998 | 99.95 | 12 | 100 |
|  |  | 11 | 2 | 0.05 |  |  |
|  | 16 | 12 | 205 | 5.125 | 11 | 100 |
|  |  | 11 | 3795 | 94.875 |  |  |
|  | 14 | 11 | 662 | 16.55 | 10 | 100 |
|  |  | 10 | 3338 | 83.45 |  |  |
|  | 12 | 10 | 3925 | 98.125 | 10 | 100 |
|  |  | 9 | 75 | 1.875 |  |  |
|  | 10 | 9 | 1610 | 40.25 | 8 | 100 |
|  |  | 8 | 2390 | 59.75 |  |  |
|  | 8 | 8 | 22 | 0.55 | 7 | 100 |
|  |  | 7 | 3978 | 99.45 |  |  |
|  | 6 | 7 | 474 | 11.85 | 6 | 100 |
|  |  | 6 | 3526 | 88.15 |  |  |
|  | 4 | 6 | 19 | 0.475 | 5 | 100 |
|  |  | 5 | 3981 | 99.525 |  |  |
|  | 2 | 5 | 62 | 1.55 | 4 | 100 |
|  |  | 4 | 3621 | 90.525 |  |  |
|  |  | 3 | 317 | 7.925 |  |  |
|  | 0 | 4 | 1 | 0.025 | 3 | 100 |
|  |  | 3 | 3941 | 98.525 |  |  |
|  |  | 2 | 58 | 1.45 |  |  |
|  | -2 | 3 | 17 | 0.425 | 2 | 100 |
|  |  | 2 | 3740 | 93.5 |  |  |
|  |  | 1 | 243 | 6.075 |  |  |
|  | -4 | 2 | 33 | 0.825 | 1 | 100 |
|  |  | 1 | 3244 | 81.1 |  |  |
|  |  | 0 | 723 | 18.075 |  |  |
|  | -6 | 1 | 101 | 2.525 | 0 | 100 |
|  |  | 0 | 3899 | 97.475 |  |  |

Conclusion

In this contribution we provided the results of our simulation campaign on 8 Rx CQI parameters in order to define UE demodulation performance requirements.

These results will be discussed further in our companion discussion TDoc [4].

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2307028 “R4-2307028 “Simulation results of 8Rx CQI requirements”, Nokia, Nokia Shanghai Bell, RAN4 #107, Incheon, Korea, May 22 – 26, 2023”, RAN4 #107, Incheon, Korea, May 22 – 26, 2023