3GPP TSG-RAN WG4 Meeting # 107		R4-2307090

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.5.1.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Consideration of scaling factor into configured transmitted power

Document for:	Approval

# Introduction

We raised an issue in RAN4#106bis-e in [1] and several comments on the issue were captured in [2]. This contribution addresses the comments and share our alternative based on the comments.

# Discussion

## Observations on the received comments

This sub-section 2.1 discusses some of the received comments, which are summarised in Annex in the bottom of this paper. Full list of Issues and associated comments can be seen in the summary of [2]. It’s noted that the comments in the Annex don’t include Nokia’s comments and comments not including specific technical aspects like a comment showing a preferred option.

Comments

OPPO Pcmax is the upper limit of power control defined in RAN1, and in RAN4 spec it was defined based on the TPMIs which can derives the max power and be used in the tests to verify UE performance The Pcmax defined in RAN4 is based on certain TPMIs and be verified with that. For the TPMIs which is not covered by RAN4 spec, UE still follows the RAN1 power control and derive the target Tx power. Qualcomm We agree the reported PHR should reflect the applicable Pcmax for that uplink instant This is the key goal referenced in the proponent's contribution: 'power to be interpreted uniquely’ by gNB. Considering the comments from other companies we would like to suggest a compromise to help the gNB; RAN4 adopt a less intrusive approach to the UE by agreeing that the reported PHR should reflect the applicable Pcmax for that uplink instant. We are open to discussing whether an explicit test is required to verify this behavior. Even without a requirement this type of agreement would further a common understanding of desired UE behavior from a network scheduler perspective. Ericsson The TPMI does not affect the Pcmax,f,c but the power per antenna port with non-zero PUSCH power (together with the scaling factor).

Our understanding

For OPPO’s comments, it seems the 1st and 2nd comments look slightly contradicting. We understand that PCMAX is RAN4 terms that consists of lower limit and upper limit of PCMAX, f, c, and a UE configures PCMAX, f, c within the range. It means that the UE is not always required to have ability to achieve the power up to the advertised power class. RAN1 uses PCMAX, f, c, that is the configured maximum output power by UE. The reported PCMAX, f, c is the reference of PHR. Hence, if the reported PCMAX, f, c is lower or higher than reality, PHR is underestimated or overestimated, respectively. In any case, it seems that at least they understand that UE needs to follow TS38.213 that means that the UE shall reduce its PUSCH power once scaling factor is invoked under certain configurations.

Regarding Qualcomm’s comment, yes, PHR itself must be a snap shot, where the reported PCMAX, f,c is still the reference and required PUSCH power under a certain configuration/channel condition is subtracted.

With respect to Ericsson’s comment, we agree with the comment that TPMI alone must not affect the PCMAX,f,c.

Observation 1: At least some companies have a common understanding that the upper limit of the output power of PUSCH is affected by combinations of some parameters like TPMI configuration, scaling factor and/or ul-FullPowerTransmission.

## Precedents

What we would like to better understand is that if the upper limit is affected by TMPI and associated scaling factor, why are companies against reflecting that aspect into the configured transmitted power formula in RAN4. As can be seen from the below formulas, ΔPPowerClass has been used to reflect UE’s true ability under a certain condition, while ΔPPowerClass itself normally doesn’t affect the original power class itself, e.g., even if powerBoostPi2BPSK is set to 1 and ΔPPowerClass becomes -3 dB for a power class 3 capable UE, still the power class is 3. The exceptional case is power class fallback due to exceedance of UL duty cycle, in this case the UE shall meet requirements for default or those for a power class between default and the advertised power class if any so that it is identical to change power class since the UE shall meet ACLR, MPR, A-MPR etc for the default power class.

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass }

Observation 2: Even now, a range of PCMAX,f,c is adjusted to reflect UE’s expected achievable output power by ΔPPowerClass, and/or some other impairments, depending on configured features, e.g.,  powerBoostPi2BPSK, or SRS antenna switching etc.

When it comes to SRS antenna switching, there has been a discussion on if we should cap the upper limit of the output power for UE capable of e.g., PC2 with TxD during SRS antenna switching. Currently, the upper limit is reduced by 3 dB compared to the advertised power class to prevent the UE from utilizing antenna virtualization. With this, at least we can understand or at least expect that the UE’s performance corresponds to PC3 single Tx requirements. For instance, it means that a referred MPR table by the UE during the switching is neither Table 6.2.2-2 nor Table 6.2D.2-1, but rather Table 6.2.2-1. It may not be realistic to capture every single corresponding requirement for each state like PC2 TxD SRS antenna switching, but this kind of adjustment by ΔPPowerClass with some complementary texts gives network an insight on the expected UE behaviour/performance.

Observation 3: ΔPPowerClass and associated text to explain have provided readers with expected UE behaviour/performance, e.g., applicable MPR, A-MPR table during SRS antenna switching, etc.

## Identified issue

In order to simplify the discussion, here we discuss a case that ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1 with layer 1 two Tx ports. Further, we assume that other than MPR can be ignored (all the deltas as well as P-MPRc are 0 dB and PEMAX,c is equal to the advertised PC) in configured transmitted power boundaries calculation below.

Case 1: PC1.5 with fullpowerMode1 with TPMI2

PCMAX_L,f,c = MIN {29 dBm,  29 dBm –MPRc} = 29 dBm –MPRc

PCMAX_H,f,c = MIN {29 dBm,  29 dBm } = 29 dBm

Hence,

29 dBm –MPRc ≤  PCMAX,f,c  ≤  29 dBm

In this case, there is no issue that the UE can signal PCMAX,f,c within the range. It’s noted that MPRc is MPRPC1.5, 2Tx.

Case 2: PC1.5 with fullpowerMode1 with TPMI0 or 1

Given that Power Class stays regardless of port configurations, the range of TPMI0 or 1 is the same as that of TPMI2.

29 dBm –MPRc ≤  PCMAX,f,c  ≤  29 dBm

According to TS 38.214, however, maximum output power shall be scaled down to half due to s = ½.

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

Hence, the achievable output power shall be 26 dBm according to TS38.213. Provided that the UE uses one of the half-rated PAs (26 dBm each) which is capable of delivering 26 dBm at physical antenna connector, MPR to be referred to must be MPRPC2, 1Tx for PC2 single Tx, i.e., Table 6.2.2-2 instead of MPRPC1.5, 2Tx. It’s noted that this applicability also is not clear in the current specifications, but this is not scope of this discussion.

Under the above consideration, if RBs with QPSK is scheduled to inner RB allocation, allowed MPR is 1.5 dB. This leads to a following PCMAX,f,c range.

24.5 dBm ≤  PCMAX,f,c  ≤  26 dBm	(1)

If, however, we didn’t make any alternations on configured transmitted power formula regardless of both port configurations and scaling factor,  and if MPR for PC1.5 dual Tx is referred to (MPR is 2 dB), it leads to a following PCMAX,f,c range. It’s noted that even if MPR for PC2 single Tx is referrred to the situation just becomes even worse since the MPR is 1.5 dB in this case.

27 dBm ≤  PCMAX,f,c  ≤  29 dBm	(2)

Observation 4: TS38.101-1 doesn’t reflect what TS38.213 expects in terms of configured transmitted power range.

Observation 5: TS38.101-1 allows higher power more than TS38.213 expects. This inconsistency makes it impossible for readers to uniquely interpret what the expected UE behaviour/performance.

Comparison of PCMAX,f,c range following current 38.101-1 to that of following 38.213/expected UE implementaiton is summarized in a following Table 1.

Table 1: Comparison between of PCMAX,f,c range following 38.101-1 and that following 38.213 for PC1.5 fullpowerMode1

Hence, if TS38.101-1 didn’t consider achievable power according to TS38.213, UE might take following measures.

- Option 1: A UE may report a value of PCMAX,f,c within 27 dBm and 29 dBm, but cannot achieve the reported value due to usage of one 26 dBm capable PA only. Then, PHR is overestimated.

- Option 2: A UE may use allowed lower tolerance of 2 dB meaning that the specification allows the UE to pass the test requirement even if PCMAX_L,f,c – 2 dB = 25(27 dBm – 2 dB) dBm, and the UE reports 27 dBm as PCMAX,f,c and transmit power larger or equal to 25 dBm. It’s noted that if the UE uses one 26 dBm capable PA, still transmitting more than 25 dBm must be quite challenging, since the UE can use only 1 dB MPR and no lower tolerance anymore.

- Option 3: A UE may use antenna virtualization meaning that pretends as if two Tx chains were one Tx chain (txDiversity-r16). In this case, the UE can achieve the same output power as that for TPMI2, while this is not the expected UE behaviour (in fact, the conformance test must not be conducted by summing the output power from both antenna connectors). And NW doesn’t expect this UE behaviour, where MPR/A-MPR conditions are different from what the NW expects.

Observation 6: Without considering the impact of port configuration on configured transmitted power, UE behaviours in terms of power (and also applicable MPR/A-MPR conditions) is not clear.

Observation 7: Uncertainty of expected achievable UE power (and MPR/A-MPR) may lead to inefficient allocation of resources.

## Resolution

Actually, there were proposals by MediaTek in [3] on how to address the issue.

The proposal 2 was a good starting point for the discussion. We, however, understand that ΔPPowerClass itself cannot be used to adjust achievable output power depending on TPMI configurations and scaling factor. For instance, for a simple case, if a network configures a PC.5 capable UE with ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, the UE may be using ΔPPowerClass of e.g., 3 dB or 6 dB due to some other matters, e.g., UL duty cycle exceedance.

For ΔPPowerClass of 3 dB due to UL duty cycle exceedance, the upper limit of output power is 26 dBm at a time. This holds no matter what the TPMI configuration is. If TPMI 2 is configured with the UE, the UE has an ability to transmit 29 dBm as hardware, but the UE needs to lower powers from two antenna ports not to exceed the total power of 26 dBm. If TPMI 0 or 1 is configured with, still the UE can keep 26 dBm with from one antenna port.

For ΔPPowerClass of 6 dB due to UL duty cycle exceedance, the upper limit of output power is 23 dBm at a time due to the said ΔPPowerClass of 6 dB. If TPMI 2 is configured with the UE, the UE has an ability to transmit 29 dBm as hardware, but the UE needs to lower powers from two antenna ports not to exceed the total power of 23 dBm. If TPMI 0 or 1 is configured with, still the UE has an ability with this TPMI to transmit up to 26 dBm with from one antenna port, but the UE needs to reduce its power down to 23 dBm.

Hence, taking max (ΔPPowerClass, a new parameter pertinent to scaling factor) must be the right way as far as ΔPPowerClass is a negative value. On the other hand, it seems that in case ΔPPowerClass is a positive value like pi/2 BPSK boosting, that boosting must be possible irrespective of TPMI configuration. In summary, it can be seen that followings may be a way to accommodate achievable output power of PUSCH transmission with scaling factor s.

Observation 8: ΔPPowerClass alone may not work to adjust achievable output power of PUSCH associated with TPMI configuration and scaling factor.

Proposal: Followings are incorporated into configured transmitted power formulas for PUSCH transmission.

- if 0 dB ≤ ΔPPowerClass, PPowerClass - Max (ΔPs , ΔPPowerClass)

- if 0 dB > ΔPPowerClass, PPowerClass - ΔPs - ΔPPowerClass

- Note that ΔPs is 10*log(1/s), where s is a scaling factor.

- Note that 0 dB > ΔPPowerClass happens when power boosting feature is enabled.

# Conclusion

This document has made the following observations and proposal.

Observation 1: At least some companies have a common understanding that the upper limit of the output power of PUSCH is affected by combinations of some parameters like TPMI configuration, scaling factor and/or ul-FullPowerTransmission.

Observation 2: Even now, a range of PCMAX,f,c is adjusted to reflect UE’s expected achievable output power by ΔPPowerClass, and/or some other impairments, depending on configured features, e.g.,  powerBoostPi2BPSK, or SRS antenna switching etc.

Observation 3: ΔPPowerClass and associated text to explain have provided readers with expected UE behaviour/performance, e.g., applicable MPR, A-MPR table during SRS antenna switching, etc.

Observation 4: TS38.101-1 doesn’t reflect what TS38.213 expects in terms of configured transmitted power range.

Observation 5: TS38.101-1 allows higher power more than TS38.213 expects. This inconsistency makes it impossible for readers to uniquely interpret what the expected UE behaviour/performance.

Observation 6: Without considering the impact of port configuration on configured transmitted power, UE behaviours in terms of power (and also applicable MPR/A-MPR conditions) is not clear.

Observation 7: Uncertainty of expected achievable UE power (and MPR/A-MPR) may lead to inefficient allocation of resources.

Observation 8: ΔPPowerClass alone may not work to adjust achievable output power of PUSCH associated with TPMI configuration and scaling factor.

Proposal: Followings are incorporated into configured transmitted power formulas for PUSCH transmission.

- if 0 dB ≤ ΔPPowerClass, PPowerClass - Max (ΔPs , ΔPPowerClass)

- if 0 dB > ΔPPowerClass, PPowerClass - ΔPs - ΔPPowerClass

- Note that ΔPs is 10*log(1/s), where s is a scaling factor.

- Note that 0 dB > ΔPPowerClass happens when power boosting feature is enabled.

# Reference

- R4-2304161, Relation between configured transmitted power and TPMI configuration, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #106bis-e

- R4-2306286, Topic summary for [106][124] FR1_enh2_part2, Moderator (vivo), TSG-RAN WG4 #106bis-e

- R4-2305707, Discussion on FR1 4Tx UE RF requirements, MediaTek Inc., TSG-RAN WG4 #106bis-e

# Annex (technical comments in [3])

Issue 1-3-1: Whether power scaling related to ULFPTx TPMI(s) should be considered in Configured transmitted power definition.

- Proposals

  - Option 1: No.

    - This is the current 38.101-1 status.

  - Option 2: Yes

    - Not align with current 38.101-1

    - It should be noted Mode 2 would be much more complicated compared to Mode 1.

| Company | Comments |
| --- | --- |
| OPPO | Option 1. Pcmax is the upper limit of power control defined in RAN1, and in RAN4 spec it was defined based on the TPMIs which can derives the max power and be used in the tests to verify UE performance. |
| Ericsson | Option 1. This is also consistent with the 38.213. |

Issue 1-3-2: Whether acceptable for UE to have power scaling even using an unscaled configurated transmitted power, in other word not able to achieve Pcmax it reports, e.g. for certain TPMI configurations;

- Proposals

  - Option 1: Yes.

    - Corresponds to option 1 in Issue 1-1

    - Reported PHR would be higher than the real value in certain configurations, though may be known to the network

  - Option 2: No

    - Corresponds to option 2 in Issue 1-1

    - Reported PHR would be the real value, but the configurated transmitted power may be much more complicated.

| Company | Comments |
| --- | --- |
| OPPO | Option 1. The Pcmax defined in RAN4 is based on certain TPMIs and be verified with that. For the TPMIs which is not covered by RAN4 spec, UE still follows the RAN1 power control and derive the target Tx power. |
| Qualcomm | Option 3: Based on the post GTW comment where it was agreed: - For 4x23dBm use case, no fall back mode test with achievable power lower than power class or power scaling will be considered. - Power class is static for the band and UE can achieve the different output power with different antenna configurations We agree the reported PHR should reflect the applicable Pcmax for that uplink instant |
| Ericsson | Option 3: other. We expect that the UE follows the 38.213 and the power scaling used with an UL output power according to the power-control equations. The power scaling for a given UL output power depends on the mode configured and the DCI/configured grants. In our understanding, the PHR is always based on the actual Pcmax,f,c configured by the UE within the range specified by RAN4 and does not change with power scaling for different antenna ports configured. |

Issue 1-3-2’(2nd round): Whether acceptable for UE not able to achieve Pcmax it reports, e.g. for certain TPMI configurations;

- Proposals

  - Option 1: No.

    - This is the current 38.101-1 status.

  - Option 2: Yes

    - Not align with current 38.101-1

    - It should be noted Mode 2 would be much more complicated compared to Mode 1.

  - Option 3: Others

| Company | Comments |
| --- | --- |
| Qualcomm | There is a problem in the wording of 1.3.2; there are two conditions that are being considered equal: - UE to have power scaling using an ‘unscaled config Tx power’ this aspect is about compliance with RAN1 Rel-15 requirements. - Not able to achieve PCmax it reports. This aspect is about compliance with RAN4 requirements, since PCMAX is defined in RAN4. In our view, to prevent combing the two different discussions, 1.3.2 is better shortened to: Whether acceptable for UE not able to achieve Pcmax it reports, e.g. for certain TPMI configurations. Agree with moderator WF for this shortened version. |
| Ericsson | Option 1: the TPMI does not affect the Pcmax,f,c but the power per antenna port with non-zero PUSCH power (together with the scaling factor). |

Issue 1-3-3: What kind of solution might be way forward?

- Proposals

  - Option 1: Keep the current RAN4 spec, that Pcmax would not be impacted by TPMI

    - Pcmax may not be always achievable under certain configuration

    - Corresponds to option 1 in Issue 1-1 / 1-2

  - Option 2: Consider revising RAN4 spec, that Pcmax can be adjusted by TPMI

    - Pcmax can always be achievable, or introduce a maximum achievable output power for a configuration via ΔPpower_class in PCMAX calculation

    - Corresponds to option 2 in Issue 1-1 / 1-2

| Company | Comments |
| --- | --- |
| OPPO | Option 1. The Pcmax defined in RAN4 is based on certain TPMIs and be verified with that. |
| Qualcomm | Option 3, other: This is the key goal referenced in the proponent's contribution: 'power to be interpreted uniquely’ by gNB. Considering the comments from other companies we would like to suggest a compromise to help the gNB; RAN4 adopt a less intrusive approach to the UE by agreeing that the reported PHR should reflect the applicable Pcmax for that uplink instant. We are open to discussing whether an explicit test is required to verify this behavior. Even without a requirement this type of agreement would further a common understanding of desired UE behavior from a network scheduler perspective. |
| Ericsson | Option 3: the Pcmax,f,c is not affected by the TPMI in our understanding (Option 1 but not the sub-bullets). |