3GPP TSG-RAN WG4 # 107	R4-2307114

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.5.3.1.3

Source:	MediaTek Inc.

Title:	Discussion on SDR requirements for 8Rx UE

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, a WF for 8Rx UE SDR requirements was agreed [1]. In this contribution, we provide simulation results based on [1] and propose suitable MCS to define SDR requirements.

- Discussion

For 8Rx UE SDR requirements, there are still some options after RAN4#106bis-e.

Issue 3-4: Maximum MCS and MIMO layers for 256QAM - 2 and 4 layers: reuse the requirements defined in Table 5.5A-5 of TS 38.101-4 - Candidate options for maximum achievable MCS for 8 layers - Option 1: MCS22 - Option 2: MCS24 Issue 3-5: Maximum MCS and MIMO layers for 1024QAM - Introduce SDR requirements for 1024QAM at least for 2 MIMO layers - Candidate options - Option 1: Only 2 layers: - Option 1a: MCS23 - Option 1b: MCS24 - Option 2: Both 2 and 4 layers: - Option 2a: MCS23 - Option 2b: MCS24

Considering Tx EVM, 3% for 256QAM and 2.5% for 1024QAM, the maximum achievable SNR for 256QAM and 1024QAM are 30.4dB and 32dB. Based on simulation results for 1024QAM 4layers, it can be shown that the required SNR for considered MCS is very high. Also, we need to add some margin for RF and other transmitter/receiver impairments. Therefore, we propose only define SDR requirements for 1024QAM 2layers.

Proposal 1: For 1024QAM, define SDR requirements only for 2 layers.

Also, according to our simulation results, we have the following proposal for the maximum achievable MCS.

Proposal 2: For 8Rx UE SDR requirements, we propose the following maximum achievable MCS:

- MCS24 (MCS table 2) for 8 layers 256QAM.

- MCS24 (MCS table 4) for 2 layers 1024QAM

Table 1. SNR point for 85% maximum throughput

| Maximum number of PDSCH MIMO layers | Maximum modulation format | MCS | SNR (dB) @ 85% max TP |
| --- | --- | --- | --- |
| 8 | 8 | 27 | 28.7 |
|  |  | 26 | 27.7 |
|  |  | 25 | 26.7 |
|  |  | 24 | 25.8 |
|  |  | 23 | 24.8 |
| 2 | 10 | 24 | 25.9 |
|  |  | 23 | 23.7 |
| 4 | 10 | 24 | 30.1 |
|  |  | 23 | 27.8 |
| Note 1: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Based on the proposal 1, proposal 2 and agreement in last meeting (maximum MCS for 64QAM is MCS26), we propose the MCS indexes for 8Rx UE SDR requirements in Table 2 and Table 3.

Proposal 3: The MCS indexes for 8Rx UE SDR requirements are proposed in Table 2 and Table 3.

Table 2. MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Table 3. MCS indexes for indicated UE capabilities (1024QAM)

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8 | 2 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Conclusion

In this contribution, we provide simulation results for SDR based on agreed simulation assumption in the last meeting and have the following proposals.

Proposal 1: For 1024QAM, define SDR requirements only for 2 layers.

Proposal 2: For 8Rx UE SDR requirements, we propose the following maximum achievable MCS:

- MCS24 (MCS table 2) for 8 layers 256QAM.

- MCS24 (MCS table 4) for 2 layers 1024QAM

Proposal 3: The MCS indexes for 8Rx UE SDR requirements are proposed in Table 2 and Table 3.

Table 2. MCS indexes for indicated UE capabilities

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 8 | 8 | 1 | 24 |
| 8 | 8 | 0.8 | 23 |
| 8 | 8 | 0.75 | 22 |
| 8 | 8 | 0.4 | 12 |
| 8 | 6 | 1 | 26 |
| 8 | 6 | 0.8 | 24 |
| 8 | 6 | 0.75 | 23 |
| 8 | 6 | 0.4 | 14 |
| 8 | 4 | 1 | 16 |
| 8 | 4 | 0.8 | 16 |
| 8 | 4 | 0.75 | 16 |
| 8 | 4 | 0.4 | 11 |
| 8 | 2 | 1 | 9 |
| 8 | 2 | 0.8 | 9 |
| 8 | 2 | 0.75 | 9 |
| 8 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Table 3. MCS indexes for indicated UE capabilities (1024QAM)

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 8 | 2 | 10 | 1 | 24 |
|  |  | 10 | 0.8 | 21 |
|  |  | 10 | 0.75 | 19 |
|  |  | 10 | 0.4 | 9 |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |  |

- Reference

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon

- Appendix

Table 1: Simulation assumptions SDR requirements

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | 1 for Rank <= 4 2 for Rank > 4 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000 – 1005} for 6 Layers CCs {1000 – 1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 2 layers CCs 2 for others |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of HARQ transmission |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with PRB bundling granularity chosen avoid zeros on Rx |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Tx EVM |  |  | Tx EVM = 6% for up to 64QAM Tx EVM = 3% for 256QAM Tx EVM = 2.5% for 1024QAM |
| MIMO layers |  |  | 64QAM and 256QAM: 8 1024QAM: 2, 4 |
| Antenna configuration | 2 layers |  | 2x8 |
|  | 4 layers |  | 4x8 |
|  | 8 layers |  | 8x8 |