3GPP TSG-RAN WG4 Meeting # 107		R4-2307124

# Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.28.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	UL duty cycle scheme improvement

Document for:	Approval

# Introduction

This contribution further addresses the comments on enhancements for issues like SAR accompanied with higher power that obtained in online and offline (in summary) in RAN4#106bis-e.

# Discussion

## Report of power class

### Relation with ΔPPowerClass

This sub-section elaborates why we don’t propose to enable UE to report ΔPPowerClass, but rather we propose that to report for which power class requirements are being referred to or going to be referred to only when that change happens. There are several cases that ΔPPowerClass is set to a value of either of -3, 3, or 6 dB by a UE.

| Case | ΔPPowerClass | Requirements | Configurable by NW |
| --- | --- | --- | --- |
| 1. When the IE powerBoostPi2BPSK is set to 1 | -3 dB | Stay | YES |
| 2. When P-max of P dBm or lower is indicated (P is lower than advertised power class) | 3, or 6 dB depending on advertised power class | Stay | YES |
| 3. When the UE is configured with SUL configurations | 3 dB | Stay | YES |
| 4. When the percentage of uplink symbols transmitted in a certain evaluation period is larger than default, maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC2-FR1/2, or maxUplinkDutyCycle-PC1dot5-MPE-FR1 is exceeded (the details are omitted). | 3, or 6 dB depending on advertised power class | Change | NO |
| 5. For TxD UE, during SRS AS with t1ry(y>1)(the details are omitted). | 3 dB | Stay | YES |

As summarized in the above table, apart from a case of UL duty cycle exceedance, the features accompanied with ΔPPowerClass with other than 0 dB requires a configuration of the corresponding feature with the UE by network. Assuming that a UE follows 3GPP specifications, the status of ΔPPowerClass is implicitly realized by the network. Even if the change was reported, network wouldn’t obtain something new information since we know that the requirements stay. If PC2 UE reports PCMAX, f, c = 23 dBm, the network can interpret that still power class is 2, but due to some UE impairments, the achievable output power is 23 dBm, where the ACLR is 31 dB and MPR/A-MPR tables for PC2 must be referred to by the UE. On the other hand, in case of ΔPPowerClass change due to UL duty cycle exceedance, the applicable requirements will change. This information is valuable.

Observation 1: Features accompanied with ΔPPowerClass with other than 0 dB requires a configuration of the corresponding feature with the UE by network except for a change due to UL duty cycle exceedance. For these cases, the report doesn’t bring any new information to a network.

Observation 2: For a case of ΔPPowerClass change due to UL duty cycle exceedance, ΔPPowerClass report can be meaningful from network perspective since the appliable requirements change as well.

Observation 3: From the above observation 1 and 2, reporting ΔPPowerClass for every occasion of the changes is redundant. If reported, the occasion of the report should be limited to the case 4, where applicable requirements changes.

### Why for which power class requirements is referred to is valuable

There was a comment that why it is useful for gNB to know MPR and/or A-MPR values in the last meeting. At least we haven’t ever said that MPR and/or A-MPR values should be reported. When UL duty cycle exceeds an advertised duty cycle or default duty cycle if not advertised e.g., by a PC2 UE, the UE shall meet all the requirements for PC3. It means that tables for MPR/A-MPR to be referred to by the UE has changed. For instance, if a UE is meeting requirements for PC2, a network may avoid scheduling resources to edge allocation to avoid 3.5 dB MPR for e.g., 64QAM DFT-s-OFDM, while if the same UE is meeting requirements for PC3, network may not consider MPR itself for the same waveform with MCS since MPR is 2.5 dB regardless of position of RBs and its length. Yet another example is that if a UE reports ul-FullPwrMode1-r16 for a band with PC2, and if the UE has started meeting the requirements for not PC2 but rather PC3, then, ul-FullPwrMode1-r16 must not be appropriate anymore. It is not our interest that if power class fallback never happens but stays or requirements change from those for a PC to those for a different PC should be called fallback or return.

Observation 4: For which power class requirements that a UE is meeting at a moment is useful for gNB to know some aspects in terms of resource scheduling and/or feature configuration, e.g.,

- If PC2 MPR for 16QAM DFT-s-OFDM is being referred to, the edge allocation may be avoided, while if PC3 MPR for the same waveform/MCS is being referred to, no difference due to resource position/length in terms of MPR

- If PC2 ul-FullPwrMode1-r16 for a band is reported, but if the UE has started meeting PC3 requirements, ul-FullPwrMode-r16 may be more appropriate.

### Others

There was also a comment by UE vendors that UE doesn’t change for which power class requirements that it meets due to UL duty cycle, but the UE just uses P-MPR only. It may be true given that an NR TDD configuration may follow LTE TDD configuration to co-exist each other in the same band or different bands without simultaneous Rx/Tx. With the most popular LTE TDD configuration, e.g., configuration 2, the UL duty cycle for PC1.5 wouldn’t exceed 25 %. Given that gradually, higher power classes have been introduced, some enhancements not relying on current P-MPR scheme is valuable. There may be a proposal about higher power classes than PC1.5 or higherpowerlimit with e.g., PC1.5 + PC3, PC1.5 + PC2. In addition, since with the current P-MPR scheme for FR1, expected UE behaviour is unknown to gNB. Supposed inter band uplink CA and a UE uses P-MPR, power of one of the bands is significantly reduced or powers of both bands are reduced is not clear. This is very different from power scaling within CA, though the UE behaviours due to power scaling itself is also not clear, though. Finally, currently, handling of SAR has been discussed under 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC, where still using UL duty cycle is one of the candidate options.

Observation 5: UL duty cycle scheme enhancement is more reasonable given that in the end, higher power classes than PC1.5 may be introduced, where the threshold becomes less than 25 % and as far as a UE follows LTE TDD configuration 2 whose UL duty cycle % would exceed the threshold.

Observation 6: Despite the comments that some companies don’t use UL duty cycle at all, still UL duty cycle scheme is one of the candidate options under 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC WI.

Provided that only two meetings are left in RAN1 and three meetings are left in RAN2, it may not be realistic to complete the requirements for some assistant information such as duration of duty cycle etc., on top of reporting for which power class requirements are being referred.

Proposal 1: Introduce at least a scheme for a UE to report the information that for which power class requirements that the UE is referring to only when the UE has changed reference power classes in Rel-18 without assistance information like evaluation period etc.

With respect to P-MPR reporting, though we don’t see reporting P-MPR alone is not helpful, but some assistance information is needed if P-MPR is reported, though we understand that at least sudden power drop due to P-MPR is not network friendly UE behaviour. If, however, companies that have proposed to introduce P-MPR report into FR1 gives us ways to utilize that information (P-MPR alone without no assistance information), we are open to introduce it.

Proposal 2: Consider the introduction of P-MPR repot in FR1 if a specific way on how to utilize that information is clarified.

- .

# Conclusion

This document has made the following observations and proposals.

Observation 1: Features accompanied with ΔPPowerClass with other than 0 dB requires a configuration of the corresponding feature with the UE by network except for a change due to UL duty cycle exceedance. For these cases, the report doesn’t bring any new information to a network.

Observation 2: For a case of ΔPPowerClass change due to UL duty cycle exceedance, ΔPPowerClass report can be meaningful from network perspective since the appliable requirements change as well.

Observation 3: From the above observation 1 and 2, reporting ΔPPowerClass for every occasion of the changes is redundant. If reported, the occasion of the report should be limited to the case 4, where applicable requirements changes.

Observation 4: For which power class requirements that a UE is meeting at a moment is useful for gNB to know some aspects in terms of resource scheduling and/or feature configuration, e.g.,

- If PC2 MPR for 16QAM DFT-s-OFDM is being referred to, the edge allocation may be avoided, while if PC3 MPR for the same waveform/MCS is being referred to, no difference due to resource position/length in terms of MPR

- If PC2 ul-FullPwrMode1-r16 for a band is reported, but if the UE has started meeting PC3 requirements, ul-FullPwrMode-r16 may be more appropriate.

Observation 5: UL duty cycle scheme enhancement is more reasonable given that in the end, higher power classes than PC1.5 may be introduced, where the threshold becomes less than 25 % and as far as a UE follows LTE TDD configuration 2 whose UL duty cycle % would exceed the threshold.

Observation 6: Despite the comments that some companies don’t use UL duty cycle at all, still UL duty cycle scheme is one of the candidate options under 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC WI.

Proposal 1: Introduce at least a scheme for a UE to report the information that for which power class requirements that the UE is referring to only when the UE has changed reference power classes in Rel-18 without assistance information like evaluation period etc.

Proposal 2: Consider the introduction of P-MPR repot in FR1 if a specific way on how to utilize that information is clarified.

# Reference

- R4-2304161, Relation between configured transmitted power and TPMI configuration, Nokia, Nokia Shanghai Bell, TSG-RAN WG4 #106bis-e

- R4-2306286, Topic summary for [106][124] FR1_enh2_part2, Moderator (vivo), TSG-RAN WG4 #106bis-e

- R4-2305707, Discussion on FR1 4Tx UE RF requirements, MediaTek Inc., TSG-RAN WG4 #106bis-e