3GPP TSG-RAN WG4 Meeting # 107	R4-2307128

# Inchon, Korea, 22th - 26th, May, 2023

Source: 	KDDI

Title: 	Views on the Type 2 EN-DC/NR-CA UE for Non-collocated deployment

Agenda Item:	8.12.2

Document for:	Discussion

- Introduction

In the RAN4#106-bis meeting, RAN4 agreed the agreements and WF on UE Configuration Assumption as following [1]. In this paper, we focus on the remaining issue on UE Configuration assumption for non-collocated non-contiguous Type 2 EN-DC/NR-CA UE.

RAN4#106-bis WF: R4-2306605
< Issue 2-1-1: Whether UE supports to switch between Type 1 and Type 2 on a request from BS >

Agreement:

- Type 1 and Type 2 means different UE capability.

- Type 1 is mandatory and Type 2 is optional.

< Issue 2-1-2: Whether Type 1 shall be default when no request comes from BS >

Way Forward:

- Type 1 is default as baseline to continue the following discussions.

- How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2.

- Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution.

- FFS about the definition of Type1 in the next meeting.

< Issue 2-1-2-1: How UE behaves in case of UE is configured change from default (Type 1 UE) to Type 2 >

Way forward:

- Encourage companies to provide their views on how UE behave in case of UE is configured change from default (Type 1 UE) to Type 2.

- Continue further discussions in the next meeting.

< Issue 2-1-2-2: Whether to need signalling from BS to UE operating as Type 1 for configuring Type 2 as the standardized solution >

Way forward:

- Continue further discussions in the next meeting.

- Discussion

Firstly, the definition of Type 1 UE has already been introduced in 38.306 [2] as follows.

#### 4.2.7.9	MRDC-Parameters

| interBandMRDC-WithOverlapDL-Bands-r16 Indicates the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC/NE-DC MRTD according to clause 7.6.2/7.6.5 in 38.133 [5] and inter-band RF requirements (i.e Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially DL bands with (NG)EN-DC/NE-DC MRTD<3us according to clause 7.6.3 in 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |

Observation 1: The definition of Type 1 has already been defined in 38.306.

Next, with regard to the default UE Type (i.e. Type 1 vs Type 2), legacy UE is clearly only Type 1. In order to decrease complexity on operation and configuration, the straightforward way is that the default UE Type is Type 1.

Observation 2: Type 1 UE is the default.

And then, in the last RAN4 meeting, companies discussed how UE behaves from Type 1 to Type 2. Adding to UE capability, they also mentioned that there are possibilities of solutions on the both UE assistance procedure (the new BS signaling) or the autonomous UE configuring by itself. In this section, we try to summarize the last meeting’s results and specifically show directions on UE configuration assumption here.

First, based on the above Observation 2 that Type 1 UE is the default, there seems to be two directions, NW signaling based and autonomous UE configuring based. Basically, regardless of both directions, since collocated or non-collocated is fully up to network deployment, the MIMO layer/Rx Chain should not be restricted even if UE indicates Type 2 capability.

Proposal 1: The MIMO layer/Rx Chain should not be restricted even if UE indicates Type 2 capability, since collocated or non-collocated is fully up to network deployment.

Option 1. UE Capability + NW Signaling

With regard to NW signaling based solution, based on companies’ views in the last RAN4 meeting, there are different two directions. This means that there is the difference between existing RRC Reconf. and UE assistance procedure (new BS signaling).

Option 1-1. UE Capability + Existing RRC Reconf.

Basically, both Type 1 and Type 2 are based on EN-DC/NR-CA operation. Actually, no matter the UE supports Type-1 or Type-2, NW is collocated or non-collocated, the practice is always BS configure NR-CA/EN-DC operation to UE using existing RRC reconfiguration procedure. However, the current spec has a restriction in 38.101-1 7.10A from operation point of view.

TS38.101-1 V18.1.0 (2023-03)

For a UE indicating [intraBandNonColocatedCA-r18] for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier and the requirements for four Rx ports per component carrier do not apply. Table 7.10A.2-2: NR CA combinations CA combination. CA_n77(2A). CA_n78(2A).

This means that UE supporting Type 2 is now under non-collocated BS and configured NR-CA with 2-layer/Rx per CC, and then this UE moves to collocated BS. At this time, UE might needs a new signalling to tell UE that now is collocated BS, so that UE can do hardware/software preparation (switching) to support 4 layer/Rx per CC (i.e. Type 1). If there is no such new signalling from BS, consequently it might occur that UE indicating Type 2 is always working under 2 layer per CC mode and has no way to switch to 4 layer per CC. The restriction is common for both EN-DC and NR-CA, because it was not discussed and companies seemed to be aware of it in Rel-16 EN-DC discussion.

Observation 3: Current specs might have restrictions on 4 layer/Rx per CC operation.

Observation 4: The restriction is common for both EN-DC and NR-CA.

Observation 5: If current specs have the restriction actually, RAN4 should solve it.

Option 1-2. UE Capability + UE assistance procedure (new BS signaling)

As we mentioned in option 1-1, UE might need BS new signaling HW preparation/switching such as MIMO layer change. Additionally, if UE actually need to hardware/software preparation, the time to do that might be needed. However, currently whether the time/gap is necessary or how long are not clear, so RAN4 need to discuss this including LS to RAN2 furthermore. On the other hand, if the reconfigurations is appeared only at the first time to activate the SCell, the gap for reconfiguration might not be needed.

Observation 6: Option 1-2 needs LS to RAN2 on adding new BS signaling.

Observation 7: UE might need a gap for HW preparation & configuration time such as MIMO layer change.

Observation 8: If the reconfigurations is appeared only at the first time to activate the SCell, the gap for reconfiguration might not be needed.

Option 2. UE Capability + autonomous UE configuring by itself

In last RAN4 meeting, some companies also mentioned the autonomous UE configuring solution between Type 1 and Type 2 by UE itself detecting the current BS circumstance. However, this would lead to increase UE complexity and should not be pursued anymore. Additionally, in case of the autonomous UE configuring, RAN4 seem to discuss whether to need to specify which KPI metrics UE use to configure Type 2. Additionally, it is also not clear whether UE can know that the quality degradation of those KPI are derived from non-collocated deployment or other factors of circumstances. Currently, RAN4 has not discussed it yet at all.

Observation 9：The autonomous UE configuring by UE itself detecting the current BS circumstance will increase UE complexity.

Observation 10：Not clear on whether to need to specify which KPI metrics UE use to configure Type 2

Observation 11：Not clear on whether UE can know that the quality degradation of those KPI are derived from non-collocated deployment or other factors of circumstances

On the other hand, RAN4 has only one TU for UE RF and two TU for RRM.

Observation 12: RAN4 has only one TU for UE RF and two TU for RRM.

Consequently, we show the default and the transition of UE Type between non-collocated BS and collocated BS in the following table. We propose to discuss the three options in this meeting.

Assumed transitions of UE Type

| UE Type supporting Type 2 | Option 1-1 | Option 1-2 | Option 2 |
| --- | --- | --- | --- |
| Default UE Type | Type 1 as default | Type 1 as default | Type 1 as default |
| Collocated BS → Non-collocated BS | Type 2 configured by existing RRC Reconf. | Type 2 configured by new BS signaling | Type 2 configured by UE itself |
| Non-collocated BS → Collocated BS | [Type 2 (current spec’s restriction)] | Type 1 configured by new BS signaling | Type 1 configured by UE itself |
|  | [Type 1 (solve the above restriction)] |  |  |
| (As reference) Legacy UE | Type 1 | Type 1 | Type 1 |

Proposal 2：Discuss the following directions and complete UE RF part of this WI in this meeting.

- Option 1-1. UE Capability + Existing RRC Reconf.

- Option 1-2. UE Capability + UE assistance procedure (new BS signaling)

- Option 2. UE Capability + autonomous UE configuring by itself

- 3. Conclusion

Observation 1: The definition of Type 1 has already been defined in 38.306.

Observation 2: Type 1 UE is the default.

Observation 3: Current specs might have restrictions on 4 layer/Rx per CC operation.

Observation 4: The restriction is common for both EN-DC and NR-CA.

Observation 5: If current specs have the restriction actually, RAN4 should solve it.

Observation 6: Option 1-2 needs LS to RAN2 on adding new BS signaling.

Observation 7: UE might need a gap for HW preparation & configuration time such as MIMO layer change.

Observation 8: If the reconfigurations is appeared only at the first time to activate the SCell, the gap for reconfiguration might not be needed.

Observation 9：The autonomous UE configuring by UE itself detecting the current BS circumstance will increase UE complexity.

Observation 10：Not clear on whether to need to specify which KPI metrics UE use to configure Type 2

Observation 11：Not clear on whether UE can know that the quality degradation of those KPI are derived from non-collocated deployment or other factors of circumstances

Observation 12: RAN4 has only one TU for UE RF and two TU for RRM.

Proposal 1: The MIMO layer/Rx Chain should not be restricted even if UE indicates Type 2 capability, since collocated or non-collocated is fully up to network deployment.

Proposal 2：Discuss the following directions and complete UE RF part of this WI in this meeting.

- Option 1-1. UE Capability + Existing RRC Reconf.

- Option 1-2. UE Capability + UE assistance procedure (new BS signaling)

- Option 2. UE Capability + autonomous UE configuring by itself

- 4. Reference

[1] R4-2306605, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI, RAN4#106-bis-e

[2] 3GPP TS 38.306 V17.4.0 (2023-03)