3GPP TSG-RAN WG4 Meeting #107	R4-2307132

Incheon, Korea 22th – 26th May 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 					Simulation results for LTE US 900

Agenda Item: 			9.4.3

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for LTE US 900 band. We study the required A-MPR to protect band 5 with different spectrum emission limits.

# 2	Simulation assumptions and scenario

- 3 MHz LTE channel at 897.5-900.5 MHz

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

Table 1: Spectrum emission limits to protect band 5.

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 869 ≤ f ≤ 894 | -35, -40, -45, -50 | 1 MHz |

# 3	Simulation results

A complete set of A-MPR results on 3 MHz channel are provided in appendix. In Figure 1 we present results on QPSK modulation.

Observation 1: A-MPR is required for 3 MHz channel in order to protect band 5 with -35 dBm@1MHz.

-50 dBm@1MHz

-45 dBm@1MHz

-40 dBm@1MHz

-35 dBm@1MHz

Figure 1: Examples of A-MPR needed to protect band 5. All the subfigures show results with the same parameters except the spectrum emission limit.

# 4	Conclusions

We have provided simulation results for LTE US 900 on band 5 protection. Based on the results we have made the following observation.

Observation 1: A-MPR is required for 3 MHz channel in order to protect band 5 with -35 dBm@1MHz.

# 5	Appendix: A-MPR triangle plots