3GPP TSG-RAN WG4 Meeting #107	R4-2307153

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.171 | CR | 0020 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.171: Corrections to NR A-GNSS requirements (Rel 15) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In reality, Galileo E1 signal is +1.25 dB higher than the GPS L1 C/A signal. This can be seen by looking at the values of Minimum received power [dBW] in the GPS and Galileo specifications https://www.gps.gov/technical/icwg/IS-GPS-200N.pdf and https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo_OS_SIS_ICD_v2.0.pdf While GPS L1 C/A is at -158.5 dBW, Galileo E1 is at -157.25 dBW. This should be represented also in the tests (Table 4.1), by adding a +1.25 dB power offset to the Galileo E1 signal. Similarly, in the above specs, the GPS L5 signal is at -153.99 dBW (when adding I and Q components) which leads to the +3.6dB offset in the table, while the Galileo E5a, E5b and E6 are each at -155.25 dBW. This leads to an offset factor vs. GPS L1 C/A of +3.25dB, not +2 as stated in the Table 4.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 4.1, the relative signal levels of Galileo E1, E5 and E6 signals have been corrected to align with the Galileo specification values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UEs supporting Galileo may fail unjustified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.571-1 CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

## 4.8	UEs supporting multiple signals

For UEs supporting multiple signals, different minimum performance requirements may be associated with different signals. The satellite simulator shall generate all signals supported by the UE. Signals not supported by the UE do not need to be simulated. The relative power levels of each signal type for each GNSS are defined in Table 4.1. The individual test scenarios in clause 6 define the reference signal power level for each satellite. The power level of each simulated satellite signal type shall be set to the reference signal power level defined in each test scenario in clause 6 plus the relative power level defined in Table 4.1.

Table 4.1: Relative signal power levels for each signal type for each GNSS

|  | BDS |  |  | Galileo |  | GLONASS |  | GPS/Modernized GPS |  | QZSS |  | SBAS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal power levels relative to reference power levels | B1I | D1 | 0 dB | E1 | +1.25 dB | G1 | 0 dB | L1 C/A | 0 dB | L1 C/A | 0 dB | L1 | 0 dB |
|  |  | D2 | +5 dB |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | E6 | +3.25 dB | G2 | -6 dB | L1C | +1.5 dB | L1C | +1.5 dB |  |  |
|  |  |  |  | E5 | +3.25 dB |  |  | L2C | -1.5 dB | L2C | -1.5 dB |  |  |
|  |  |  |  |  |  |  |  | L5 | +3.6 dB | L5 | +3.6 dB |  |  |

NOTE 1:	For test cases which involve “Modernized GPS”, the satellite simulator shall also generate the GPS L1 C/A signal if the UE supports “GPS” in addition to “Modernized GPS”.

NOTE 2:	The signal power levels in the Test Parameter Tables represent the total signal power of the satellite per channel not e.g. pilot and data channels separately.

NOTE 3:	For test cases which involve "BDS", D1 represents MEO/IGSO satellites B1I signal type and D2 represents GEO satellites B1I signal type.

< End of changes >