3GPP TSG-RAN WG4 Meeting #107	R4-2307156

Incheon, KR, May 22 – May 26, 2023

Source:	Huawei, Hisilicon

Title:	Discussion on Rel-18 Tx switching with single TAG

Agenda item:			8.24.2.1

Document for:	Approval

# 1. Introduction

In the contribution, we further discuss the left open issues on Rel-18 multi-carrier with single TAG.

# 2. Discussion

## 2.1. Two Tx chains switched sequentially between two band pairs

In RAN4#106bis-e, it was proposed to re-visit the length of switching time given that the two Tx chains may execute switching sequentially for some UEs’ implementation with the following WF.

#### Issue 1-4-1: Switching case across four bands, i.e., {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} Way forward after round 2 discussion: - Further discuss in the next meeting: - Option A: As optional UE behaviour, total switching period can be extended if UE is not capable for concurrent TX switching on the two TX chains. - Option A1: add new values {70, 175} usec in addition to the agreed set of {35 us, 140 us, 210 us} - Option A2: Sum of two switching periods - Option B: Keep the previous agreements

At the very first meeting discussing Rel-18 multicarrier in RAN4, it was agreed and included in the LS to other working groups in [1] to reuse the same set of switching period value as Rel-16/17:

RAN4 Answer: Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

We didn’t see there is any error in the previous agreement, and the Rel-18 Tx switching can work well based on it. Introducing new values into the switching period value set is not expected. In the last meeting, operator had concern on additional values in that UE reporting more sw period values would increase the complexity of network side. In that case, some company pointed that the {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} is a rare case and operators could do scheduling restriction to avoid the increased complexity. We do not think this is a good argument. In one hand, scheduling restriction is kind of complexity as well, no matter out of standard efforts or network implementation. In the other hand, if the network does not perform such scheduling, it is pointless to introduce the new values.

Proposal 1: Adding new values to the agreed switching period set is not expected.

- Rel-18 Tx switching can work well based on the previous agreement without error.

- Adding new values would increase the complexity from the perspective of network.

The WF has been approved in [2] to preclude the illustrated time mask on summing of two switching periods #3 and #4 since it is not aligned with the RAN1 agreement that the impact from switching would be on either switch-from bands or switch-to bands.

- Issue: Is the switching period location in the following figure possible? WF: - It is observed that the location of switching period in the figure above is not valid since it is not aligned with the RAN1 agreement.

If the sum of switching periods for the two band pairs locates on either switch-from bands or switch-to bands, it results in longer outage time on the very few UL resources. RAN1 evaluated the length of switching period before, and if the switching period is too long, the performance with Tx switching would lag behind that without switching. Summing of two switching period is up to 420μs, in that case, it may be better for the UE to be configured semi-statically with UL CA or single uplink rather than Tx switching.

In addition, for the logic of UE capability from RAN2, the optional capability is equivalent to advanced capability. And it is blatantly obvious that the switching period of advanced capability should not be longer than the baseline. The worse capability results from the sum of switching periods should not be introduced.

Proposal 2: Sum of two switching periods is not expected.

- The illustrated time mask on summing of two switching periods has been precluded in the last meeting since it violate the RAN1 agreement.

- According to RAN1’s evaluation, too long switching period would results in worse performance for Tx switching than semi-static UL CA or single uplink.

- The worse optional capability results from the sum of switching periods should not be introduced considering optional capability is equivalent to advanced capability from RAN2’s logic.

More bands are supported in Rel-18 Tx switching to implement more flexible utilization of spectrum resources. Correspondingly, the UE implementation should be upgraded to coordinate the promising much better performance. The two Tx chains switching concurrently would be the typical UE implementation. For some UEs that support barely switching sequentially for two band pairs, those should either complete switching for one Tx switching instance during a single determined switching gap as agreed in RAN1 in R1-2304219, or relinquish support Rel-18 Tx switching and fallback to Rel-16/17.

Agreement: In the reply LS to RAN4, the answer to RAN4 question is as follows: - RAN1 confirms that it is possible that the two Tx chains are switched concurrently between two different band pairs for one Tx switching instance during a single switching gap for the following three examples. - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D. - Example #3: In the case of 3-band Tx switching, the switching is performed from 2T on band A to 1T+1T on band B and C. - It is RAN1 understanding that the switching of two Tx chains between two different band pairs can be performed for one Tx switching instance during a single determined switching gap derived by switching periods for different band pairs reported by UE according to RAN4 LS [R1-2300029/R4-2220548]. - Whether two Tx chains are switched simultaneously or sequentially for one Tx switching instance during the single determined switching gap is up to RAN4.

Proposal 3: The UEs that support barely switching sequentially for two band pairs, should either complete switching during a single determined switching gap as agreed in RAN1, or relinquish supporting Rel-18 Tx switching and fallback to Rel-16/17.

## 2.2. Exact location of the switching period and the actual start point of transmission on the switch-to bands.

The discussion on defining exact location of the switching period was also triggered by RAN1 LS R1-2302198. RAN4 has endorsed the draftCR in [3] to clarify that the time mask in 38.101-1/3 is applicable to the case when switching period is longer than the duration of no transmission between switch-from and switch-to bands, and the RRC signalling uplinkTxSwitchingPeriodLocation does not take effect in this case. The exact time domain location of switching period should not be additionally define in Rel-16/17 since there was no such limitation. Time domain location of switching period is up to UE implementation with flexibility. Thus fixed time domain location for Rel-16/17 is not backward-compatible.

Observation 1: Fixed time domain switching period location for Rel-16/17 is not backward-compatible.

Proposal 4: RAN4 don't support defining time domain location of switching period in Rel-16/17.

However, in Rel-18, the definition of the actual start point of the transmission on the switch-to band is helpful for the network to locate the DL interruption period as it is up to scheduling restriction, and without NBC issue. In our view, from RAN1’s spec, the concept T0 is the scheduled start point of the transmission, and when the switching period impacts the transmission on switch-to band, the actual start point would be later than T0 in time domain. Therefore, the actual start of the transmission should be separated from T0, if it is agreed to be defined.

In Rel-18, one Tx switching instance includes one band pair switching and two band pairs switching concurrently. The actual start point of the transmission(s) on switch-to band(s) Tstart should be defined per Tx switching instance. For the case of two band pairs switching, when the band with the highest priority is one of the switch-to bands, or when the band with the highest priority is one of the switch-from bands and the duration of no transmission before the transmissions on the switch-to bands is sufficient for switching, define one actual start point Tstart for both the band pairs in the Tx switching instance as illustrated in Figure 1.

- (b)

Figure 1 Per Tx switching instance defined Tstart -1

When the band with the highest priority is one of the switch-from bands and the duration of no transmission before the transmissions on the switch-to bands is not sufficient for switching, define one actual start point Tstart for both the band pairs in the Tx switching instance as illustrated in Figure 2.

- (b)

Figure 2 Per Tx switching instance defined Tstart -2

Proposal 5: The actual start point of the transmission on the switch-to bands can be defined per Tx switching instance in Rel-18.

- The actual start point of transmission is different from T0 .

- Define one actual start point Tstart for both the band pairs in the Tx switching instance

The time mask for two Tx chains switched between two band pairs should be considered as Figure 3, and Figure 4 if the optional capability is introduces. There can be clarification for the time mask in Rel-18 that introducing one actual start point Tstart for both the band pairs in the Tx switching instance.

Figure 3 Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is not capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., UE does not indicate [TBD-1] in the capability [TBD tx-on-non-affected-band]

Figure 4 Time mask for one transmitter switching between band X and band Z, and between band Y and band Z, where UE is capable of uplink transmission on band Y during time period of {switching period #1 - switching period #2}, i.e., the UE indicates [TBD-1] in the capability [TBD tx-on-non-affected-band]

## 2.3. Band configurations for Rel-18 Tx switching

In the WF [2], it was approved to further discuss the necessity of introducing the band configurations for UL Tx switching across 3 and 4 bands in the specification in the next meeting.

Which band pairs to support Tx switching within 3-band or 4-band CA combinations depends on the reported UE capability. And the UE would report through FeatureSet for the band whether to support uplink or downlink transmission. It is possible for a band to get involved in Tx switching if it supports uplink transmission.

The CA of two bands in UL configuration support dualUL pattern, in other words, concurrent transmission for the two bands with 1T+1T. For a band pair supporting switchedUL, it is not mandatory for the corresponding two bands supporting UL CA.

Considering Rel-18 Tx switching includes 1Tx-1Tx. For a 4-band CA combination, even though no UL configuration, it can support switching among{1T, 0T, 0T, 0T},{0T, 1T, 0T, 0T}, {0T, 0T, 1T, 0T} and {0T, 0T, 0T, 1T} on band{A, B, C, D} if the UE report the band pair A+B, A+C, A+D, B+C, B+D and C+D.

Band combinations along with UL configuration are up to the operators’ demand. For some 3-band or 4-band CA combinations, the UE may support band pair A+B, but the UL configuration of CA_nA-nB is not supported. In this case, the band pair A+B would support switchedUL rather than dualUL pattern. When the network receives the reported UE capability and get enough information, it is smart enough to avoid scheduling dualUL Tx switching {1T, 1T, 0T, 0T} to {0T, 0T, 1T, 1T} on band {A, B, C, D} if the UE doesn’t support UL CA of A+B or C+D.

Figure 5 The UE capability signalling for a band combination that support uplink switching

Thus, the band combinations support Rel-18 Tx switching refer to existing CA/SUL configuration and it is not required to define new band combination considering there is no new requirement. And it should be specified that Rel-18 Tx switching is UE capability per band combination.

In addition, we agree with China Telecom that

2-band UL CA requirement shall be applied only for band pairs supporting dualUL, but not for band pairs supporting switchedUL. In the Rel-18 Tx switching CR endorsed in the last meeting, it already says: For each uplink band pair, according to the capability [uplinkTxSwitching-OptionSupport], – if switchedUL is supported, uplink transmission on any one band of the band pair in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with one UL band on band X or band Y shall be satisfied; – if dualUL is supported, simultaneous uplink transmission on the two NR UL bands from the band pair for which dualUL is declared in the band combination shall be supported according to the scheduling commands, and the corresponding inter-band CA requirements with two uplink bands shall be satisfied.

Proposal 6: No need to introduce the band configurations for Rel-18 Tx.

- Band combinations along with UL configuration are up to the operators’ demand. And the existing CA/SUL band combinations can be reused for Rel-18 Tx switching.

- The network would schedule the switching between band pairs for a band combination according to the reported band pairs and UL configurations. And the network can avoid scheduling Tx switching that result in concurrent transmission for two bands without UL CA configuration.

- Rel-18 should introduce Tx switching capability per band combination.

# 3. Conclusions

This contribution presents the following observations and proposals:

Proposal 1: Adding new values to the agreed switching period set is not expected.

- Rel-18 Tx switching can work well based on the previous agreement without error.

- Adding new values would increase the complexity from the perspective of network.

Proposal 2: Sum of two switching periods is not expected.

- The illustrated time mask on summing of two switching periods has been precluded in the last meeting since it violate the RAN1 agreement.

- According to RAN1’s evaluation, too long switching period would results in worse performance for Tx switching than semi-static UL CA or single uplink.

- The worse optional capability results from the sum of switching periods should not be introduced considering optional capability is equivalent to advanced capability from RAN2’s logic.

Proposal 3: The UEs that support barely switching sequentially for two band pairs, should either complete switching during a single determined switching gap as agreed in RAN1, or relinquish supporting Rel-18 Tx switching and fallback to Rel-16/17.

Observation 1: Fixed time domain switching period location for Rel-16/17 is not backward-compatible.

Proposal 4: RAN4 don't support defining time domain location of switching period in Rel-16/17.

Proposal 5: The actual start point of the transmission on the switch-to bands can be defined per Tx switching instance in Rel-18.

- The actual start point of transmission is different from T0

- Define one actual start point Tstart for both the band pairs in the Tx switching instance

Proposal 6: No need to introduce the band configurations for Rel-18 Tx.

- Band combinations along with UL configuration are up to the operators’ demand. And the existing CA/SUL band combinations can be reused for Rel-18 Tx switching.

- The network would schedule the switching between band pairs for a band combination according to the reported band pairs and UL configurations. And the network can avoid scheduling Tx switching that result in concurrent transmission for two bands without UL CA configuration.

- Rel-18 should introduce Tx switching capability per band combination.

# 4. Reference

[1]	R4-2214464, “Reply LS on UL Tx switching across 3 or 4 bands”, TSG RAN4 Meeting #106bis-e.

[2]	R4-2306621, “WF on Rel-18 Tx switching across 3/4 bands”, TSG RAN4 Meeting #106bis-e.

[3]   R4-2306647, “Rel-16 draftCR to 38 101-1 for Clarification of UL Tx Switching”, TSG RAN4 Meeting #106bis-e.