3GPP TSG-RAN WG4 Meeting #107 	R4-2307185

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on timing requirements for UL multi-DCI multi-TRP with two TAs

Agenda item:	8.30.3.2

Document for:	Discussion

Introduction

In RAN #94e, the MIMO evolution downlink and uplink was approved in [1]. Among its objectives, there is the study and specification of two timings advance (TAs) for UL multi-DCI for multi-TRP operations:

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In RAN1#109, an LS [2] was sent to RAN4 inquiring what is the maximum uplink timing difference to be assumed between the two TAs for multi-DCI multi-TRP operation.

RAN4 answered this LS in two parts [3][4]. In the latest response, RAN4 divided the maximum uplink in two cases: cases in which the RTD is within the cyclic prefix (CP), and cases in which the RTD is above CP, as captured in previous meeting WF [5] and copied below:

Issue 1-2: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR1 Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP+M1 µs for FR1. Where M1 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 33/34.6 µs. Issue 1-3: MRTD/MTTD requirement for multi-DCI multi-TRP operation in FR2 Agreements: - For both intra-cell and inter-cell multi-TRP, the MRTD between multiple TRPs can be assumed within a CP length as baseline. MTTD can be CP + M2 µs for FR2. Where M2 is FFS. - FFS whether transient period between 2 UL signals associated with 2 different TAs needs to be considered - For a UE capable of supporting RTD>CP (as an optional UE capability), MRTD/MTTD value is 8/8.5 µs.

In RAN4 meeting RAN4#106 [6], the following were agreed:

Issue 1-1-1: In general, whether to define new MTTD/MRTD requirements? Agreement: - Specify new MTTD/MRTD requirements for multi-DCI multi-TRP operation with 2 TAs, capture all the agreements related. Issue 1-1-2: How to specify new MRTD requirements for UE not supporting RTD>CP? Agreement: - For UE not supporting RTD>CP MRTD = CP

Then, in the last RAN4 meeting RAN4#106bis-e [7], the following were agreed:

Issue 2-1-2: MTTD requirements applicability Agreement: - RAN4 to discuss the MTTD requirement with two TAGs only for mDCI. Issue 2-1-6: TA adjustment accuracy Agreement: - RAN4 not to consider TA adjustment relaxation when 2 TA commands are used.

In this contribution we discuss:

- MTTD when MRTD is within CP and propose values for the margins M1 and M2.

- How to select the DL reference timing.

- Overlapping issues related to two UL transmissions for multi-TRP.

- TAG management for multi-TRP.

Discussion

Regarding MTTD requirements

The following issue was discussed in the last RAN4 meeting [7] regarding MTTD requirements:

Issue 2-1-1: What is the assumption on M1/M2 for MTTD for UE not capable of supporting RTD>CP? - Proposals: - MTTD for UE not capable of supporting RTD > CP - Option 1: (MediaTek) - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=0 and M2=0 - Option 2: (Nokia, Samsung, Xiaomi, ZTE, Huawei, QC, Ericsson, vivo, Apple) - If UE supports STxMP - The MTTD between multiple TRPs can be defined as (CP + M1) for FR1 and (CP + M2) for FR2, M1=1.6us and M2=0.5 us - If UE doesn’t support STxMP - Wait for RAN1 further progress for gap/scheduling restriction - No MTTD requirements for this case.

The maximum transmit time difference (MTTD) is given by the MRTD and a margin that accounts for the TA adjustment accuracy and transmit timing errors at the UE. The basic principles for the definition of the MTTD requirements in different component carriers are [8]:

MTTD = MRTD + (TA step size / 2+ TA adjustment accuracy + Te) in cc1 + (TA step size / 2 + TA adjustment accuracy +Te) in cc2

The MTTD is given by the MRTD and a margin to account for the TA adjustment accuracy and transmit timing error.

Taking the MRTD/ MTTD requirements for inter-band CA in TS 38.133 as an example, we have that:

|  | Maximum receive timing difference (μs) | Maximum transmit timing difference (μs) | Margin (μs) |
| --- | --- | --- | --- |
| FR1 | 33 | 34.6 | 1.6 |
| FR2-1 | 8 | 8.5 | 0.5 |

In inter-band CA requirements, the margin between MRTD and MTTD requirements is defined as 1.6 μs in FR1 and 0.5 μs in FR2.

If we consider the MRTD calculation typically used, MRTD would be comprising TAE and propagation delay, which would then result in

MTTDM1,M2≠0 = MRTD + M = TAE + Tprop + M

Where M is M1 for FR1 and M2 for FR2, and Tprop is the propagation delay. However, if we consider that M1=M2=0, the uncertainty of UL timing will still need to be considered in the practical deployment, and MTTD would have to be calculated as

MTTDM1,M2=0 = TAE + Tprop + TUL margin, Tprop = MTTDM1,M2=0 - TAE - TUL margin

where TUL margin= 2(TA step size / 2+ TA adjustment accuracy + Te), which was adopted as TUL margin=0.5 us for FR2 and TUL margin=1.6 us for FR1. That results in the maximum propagation delay to be reduced by TUL margin in comparison to the MRTD requirements. If we consider one FR2 example in which TAE is used as 65 ns and TUL margin=0.5 us, and MTTDM1,M2=0 =CP = 0.585 us, the maximum propagation delay supported by the deployment would be just 20 ns, which is equivalent to about 6 m maximum difference among all UE-TRP distances for simultaneous UL transmission.

Assuming M1=M2=0 for the MTTD requirements definition may reduce the maximum difference among all UE-TRP distances for simultaneous UL transmission to only 6 meters.

RAN4 to define MTTD requirements for multi-DCI multi-TRP operation with 2 TAs, when the MRTD is within CP, considering the same margin used for existing MTTD requirements on top of the values defined for MRTD

  - Margin of M1=1.6 μs in FR1

  - Margin of M2=0.5 μs in FR2

Regarding DL reference timing

The following issue was discussed in the last RAN4 meeting [7] regarding DL reference timing:

Issue 2-1-3: Reference timing - Proposals: - Option 1: (Nokia, Ericsson) - The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission. - Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking. - Option 2: (Apple, Nokia, MediaTek, ZTE, vivo, Huawei, Samsung) - In UL timing requirements, some clarification needs to be added to accommodate - Two DL reference timings are supported where each DL reference timing is associated with one TAG. - Two TAGs associated with different UL/joint TCI state. - Option 3: (MediaTek) - Each TAG is allowed to have its own DL reference timing. Typically, two TAGs with different DL reference timing - FFS whether RAN4 needs distinguish same or different DL timing reference in the discussion for 2 TAGs. - Option 4: (Huawei) - For multi-DCI based multi-TRP operation, the UL transmit timing for one TAG can be derived from the DL reception timing of the PDCCH/PDSCH which is associated to the same CORESET Pool Index as UL transmission. Note: FFS on whether single reference shall be allowed or not. More RAN1 input is expected for further RAN4 discussion.

Figure 1 shows examples on how the propagation delay from one TRP can arrive with different path delays to the UE depending on the CSI beam used. The different path delays for each UL/joint TCI state need to be considered when defining the UE DL reference timing.

The different path delays for each UL/joint TCI state need to be considered when defining the UE DL reference timing.

The figure illustrates an example where the most powerful CSI at the UE is CSI#2, whereas the CSI received with the shortest path delay is CSI#3.

Figure 1. Scenario where 2 CSI beams have different path delay to the UE.

The specification of which DL reference signal the UE is required to follow is not only important for the DL reception at the UE, but the DL reference timing will as well be used as reference for the timing advance.

For the scenario where the indication of UL/joint TCI state is changed without the gNB requesting UL timing synchronization, the initial transmission with a new indicated TCI state will potentially give a UL timing offset at the gNB. The timing offset will depend on the propagation delay difference for the new TCI state, but also on which DL reference signal the UE is using as reference for the timing advance.
The timing offset at the gNB receiver can cause inter symbol interference as well as inter sub-carrier interference.

Figure 2. UL timing error when switching UL TCI (CSI#1 to CSI#2).

Figure 2 illustrates an example of the timing during a switch of indicated TCI state from CSI#1 to CSI#2. In the example it is assumed that the TA from CSI#1 is used as initial TA value for UL transmission on CSI#2. Furthermore, it is assumed that the DL reference timing for each TCI state is following individual DL RS. 
As shown, the TRP will see the initial transmission from the UE received with a timing error equal to 2(D2-D1) meaning twice the propagation delay difference between the two TCI states.

The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission.

Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking.

Regarding overlapping UL transmissions

The following issue was discussed in the last RAN4 meeting [7] regarding TDM and overlapping UL transmissions for multi-TRP with 2 TAs:

Issue 2-1-4: TDM and overlapping UL transmissions for multi-TRP with 2 TAs - Proposals: - Option 1: (Apple, MediaTek) - For FR2, RAN4 shall start from assumption that UE is only able to perform TX from one panel at a time. - It is proposed to wait for more RAN1 input before RAN4 further discussion. - Option 2: - When the UE does not support UL STxMP transmission: (Nokia, Huawei, ZTE, Samsung, vivo) - Postpone discussion and wait for further progress from RAN1. - When the UE support UL STxMP transmission (FR2 only) (Nokia, Huawei, ZTE, Samsung) - no restrictions from RRM perspective - Option 3: If UL transmissions associated with different TAs overlap, the earlier slot is reduced in duration relative to the later slot. (Ericsson) - Option 4: (Xiaomi) - Scheduling restriction is needed in specific scenarios for TDM UL two TA cases. - Wait to see the switching time discussion in the RF section of the switching time.

RAN1 has agreed on the network configuration of 2 TA loops in at least two m-DCI uses cases: TDM operation and STxMP operation.

With m-DCI schemes, the TRPs may schedule the UE independently. Therefore, enabling two TA loops with a TDM pattern may cause an overlap in the time domain of the UL signals due to different propagation delays as illustrated in the Figure 3 below, where a UE transmits in UL towards two TRPs with inter-slot TDM.

Figure 3. Overlap for UL transmissions with multi-TRP inter-slot TDM and two different TAs.

The impact of the overlap in time domain may depend on the assumptions for the UE RF architectures. In fact, when a UE is equipped with multiple active Tx RF chains, it can configure its panels/sub-panels such that at least one chain transmits with a certain specific TA toward each TRP. Therefore, in such a situation, even a large difference in the two propagation delays can be handled.

The impact of overlapping symbols in UL for TDM operation is tied to the assumptions on UE RF architectures.

When a UE is equipped with multiple active Tx RF chains, there is no issue from overlapping symbols in time-domain for UL multi-TRP operations with two TAs.

Nevertheless, when a UE is equipped with just a single active Tx RF chain (for example either because equipped with a single panel or because multi-panel but with only one single UL digital baseband shared among those panels), such overlap may represent an issue which in some cases may lead to losing up to few OFDM symbol, i.e., with some non-negligible UL throughput loss.

When a UE is equipped with just a single active Tx RF chain, the overlapping symbols in time-domain may lead to UL throughput performance loss.

Both scheduling restrictions in the overlapping part and dropping rules for UL multi-TRP operations with two TAs are very simple baselines that on the other hand may be the source of large throughput performance loss:

- Scheduling restrictions, for example by imposing that such single RF UEs are not allowed to be scheduled in the overlapping region, impact the performance as resources in that overlapping region becomes just unavailable for those UEs. Other types of restrictions, for example by imposing that such single RF UEs cannot be scheduled with multi-TRP mode at all when the transmit time difference is larger than a certain threshold, for example than the CP length, may affect too much the performance of these UEs that cannot benefit from multi-TRP operations.

- Dropping rules, for example by dropping one (or potentially more) OFDM symbol(s) have the same negative impact, as those resources are lost. Dropping “samples” from one (or more) OFDM symbol(s) (instead of full OFDM symbols) is possible, but in our view is feasible only for very short transmit time difference: note that this type of dropping, in case of channels with large maximum excess delay (or large delay spread) may be the cause of inter-symbol interference (ISI) or potentially even inter-carrier interference (ICI).

When a UE is equipped with just a single active Tx RF chain, both scheduling restrictions in the overlapping part and dropping rules for UL multi-TRP operations with two TAs are simple baselines that on the other hand may be the source of large throughput performance loss.

Furthermore, the actual time between two UL transmissions that need to be introduced in order to allow a UE not capable of STxMP to switch among two UL transmissions toward two different TRPs depends at least on four parameters:

- RTD: Receive timing difference;

- TA1: Timing advance value from TRP1;

- TA2: Timing advance value from TRP2;

- Ts: Switching time, i.e., the time it takes for a UE to switch from the beamformer/TCI used toward a first TRP to the beamformer/TCI used toward a second TRP.

Eventually, we have two different situations in which we may need to use either scheduling restrictions or sample / OFDM symbol dropping, as depicted in Figure 4:

- There is an actual overlap between the two slots, as discussed above and as shown in red in Figure 4;

- There is an actual gap between the two slots, as shown in blue in Figure 4, but, because of the switching time Ts, the UE does not manage to switch quickly enough among the two beamformers/TCIs.

Figure 4. Overlaps and gaps between slots.

Either scheduling restrictions or sample / OFDM symbol dropping may need to be introduced when the UE does not support UL STxMP transmission also when there is a gap between the two slots because of a large UE switching time.

A gNB would need to know all the four parameters listed above to properly do time domain resource allocation, for example deciding the number of OFDM symbols at the beginning of a slot on which the UE cannot be scheduled. But the problem is that the gNB does not know RTD nor the exact switching time Ts required by the UE. Therefore, a very simple baseline scheme for this problem could assume scheduling restrictions by computing the worst case possible. With maximum switching time as allowed by requirements (for example, assuming the transient period requirement we would have Ts = 10 us [9]) and the maximum RTD (for example, MRTD = 8 us), we would have 3 OFDM symbols with SCS = 120 kHz at the beginning of a slot in which a UE cannot be scheduled, i.e., with a throughput performance loss of about 21%.

Finally, we observe that for a UE equipped with just a single active Tx RF chain it is possible under certain conditions, for example with very low switching time Ts, to combine the UL transmissions toward TRP1 and TRP2, for the example in Figure 3 in the overlapping region, by combining the last OFDM symbol(s) of the slot transmitted toward TRP1 and the first OFDM symbol(s) of the slot transmitted toward TRP2. The UE can create a broad beam in the overlapping region for such combined transmissions. The benefits of such scheme leveraging a combining of the OFDM symbols of the two UL transmissions in the overlapping region is that no dropping is performed, i.e., we have no throughput performance loss.

A UE assistance to the gNB about either RTD and switching time or directly indicating the number of OFDM symbols that cannot be used for UL transmission in a slot can be beneficial to let the gNB apply proper scheduling restrictions: with such information, the gNB can do time domain resource allocation by maximizing resource usage, potentially also avoiding scheduling restrictions in the conditions that they are not needed.

A UE assistance to the gNB about either RTD and switching time or directly indicating the number of OFDM symbols that cannot be used for UL transmission in a slot can be beneficial to let the gNB apply proper scheduling restrictions.

Scheduling restrictions can be optimized considering reporting by the UE about RTD, switching time or the number of OFDM symbols that cannot be used for UL transmission.

On the other hand, scheduling restrictions for UL transmissions is a topic for RAN1, and do not need to be captured in RAN4 specification.

Scheduling restrictions for UL transmissions do not need to be captured in RAN4 specification. Send LS to RAN1 to cover that instead.

Regarding TAG management

The following issue was discussed in the last RAN4 meeting [7] regarding TAG management for multi-TRP with 2 TAs:

Issue 2-1-5: TAG management for multi-TRP with 2 TAs - Proposals: - Option 1: (Apple) - RAN4 can do some study on TAG management for multi-TRP with 2 TAs. - For example: Once it is about to exceed the limit UE can support, UE can send some indication to network so that network can e.g. indicate UE to fall back to single TA to avoid waste of UL resource since UE anyway cannot maintain two UL soon. - Option 2: use LTE CA requirements as baseline (Ericsson, Huawei, ZTE) - Option 3: FFS (vivo, MediaTek, Samsung)

When the 2 UL transmissions exceed the MTTD, the UE cannot support in UL that pair of TRP, and some studies may be needed to understand which options are available to cope with such situation.

RAN4 can do some study on TAG management when the 2 UL transmissions exceed the MTTD.

Conclusion

In this contribution we discussed the MTTD requirements when MRTD is within CP, how to select the DL reference timing, the overlapping issues related to two UL transmissions in TDM operations, and the TAG management issue for multi-TRP. The following Observations and Proposals were made:

Observation 1: The MTTD is given by the MRTD and a margin to account for the TA adjustment accuracy and transmit timing error.

Observation 2: In inter-band CA requirements, the margin between MRTD and MTTD requirements is defined as 1.6 μs in FR1 and 0.5 μs in FR2.

Observation 3: Assuming M1=M2=0 for the MTTD requirements definition may reduce the maximum difference among all UE-TRP distances for simultaneous UL transmission to only 6 meters.

Proposal 1: RAN4 to define MTTD requirements for multi-DCI multi-TRP operation with 2 TAs, when the MRTD is within CP, considering the same margin used for existing MTTD requirements on top of the values defined for MRTD

a.	Margin of M1=1.6 μs in FR1

b.	Margin of M2=0.5 μs in FR2

Observation 4: The different path delays for each UL/joint TCI state need to be considered when defining the UE DL reference timing.

Proposal 2: The UE is required to track DL RS associated to each activated UL TCI state (or joint TCI state) and use it as time reference for UL transmission.

Proposal 3: Specify for each UL/joint TCI state the DL RS the UE must use for DL time tracking.

Observation 5: The impact of overlapping symbols in UL for TDM operation is tied to the assumptions on UE RF architectures.

Observation 6: When a UE is equipped with multiple active Tx RF chains, there is no issue from overlapping symbols in time-domain for UL multi-TRP operations with two TAs.

Observation 7: When a UE is equipped with just a single active Tx RF chain, the overlapping symbols in time-domain may lead to UL throughput performance loss.

Observation 8: When a UE is equipped with just a single active Tx RF chain, both scheduling restrictions in the overlapping part and dropping rules for UL multi-TRP operations with two TAs are simple baselines that on the other hand may be the source of large throughput performance loss.

Observation 9: Either scheduling restrictions or sample / OFDM symbol dropping may need to be introduced when the UE does not support UL STxMP transmission also when there is a gap between the two slots because of a large UE switching time.

Observation 10: A UE assistance to the gNB about either RTD and switching time or directly indicating the number of OFDM symbols that cannot be used for UL transmission in a slot can be beneficial to let the gNB apply proper scheduling restrictions.

Proposal 4: Scheduling restrictions can be optimized considering reporting by the UE about RTD, switching time or the number of OFDM symbols that cannot be used for UL transmission.

Proposal 5: Scheduling restrictions for UL transmissions do not need to be captured in RAN4 specification. Send LS to RAN1 to cover that instead.

Proposal 6: RAN4 can do some study on TAG management when the 2 UL transmissions exceed the MTTD.

- References

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung.

- R1-2205593, “LS on maximum uplink timing difference for multi-DCI multi-TRP with two TAs”, RAN1.

- R4-2214354, “Reply LS on MTTD for multi-DCI multi-TRP with two TAs”, RAN4 #104-e.

- R4-2217279, “Reply LS on MTTD for multi-DCI multi-TRP with two TAs”, RAN4 #104bis.

- R4-2217278, “WF on MRTD/MTTD requirement for multi-TRPs with 2 TAs”, RAN4 #104bis.

- R4-2303258, “WF on R18 NR MIMO RRM requirements”, Samsung, RAN4#106, Athens, Greece, 27 February – 03 March, 2023.

- R4-2306362, “WF on R18 NR MIMO RRM requirements”, Samsung, RAN4#106bis-e, 17 April –26 April, 2023.

- R4-2206919, “WF on NR extension to 71 GHz RRM requirements part 1”, RAN4 #102-e.

- 5G; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (3GPP TS 38.101-1 version 15.2.0 Release 15).