3GPP TSG-RAN WG4 Meeting # 107		R4-2307280

Incheon, KR, May 22 – May 26, 2023

Source: 		Qualcomm Incorporated

Title: 	L1 measurement requirements for support of simultaneous receptions from two TRPs

Agenda item:	8.8.3.2

Document for:		Approval

# Introduction

In this paper, we present our view on the following aspects:

- UE simultaneous Rx beams for mTRP support and UE Rx beam for L1 measurement/evaluation

- L1 Measurement Applicability

- L1 Measurement Period Requirements

# Discussion

UE simultaneous Rx beams for mTRP support and UE Rx beam for L1 measurement/evaluation

Although it was already agreed to postpone the discussion on L1-RSRP measurement accuracy requirements until the performance requirement phase, we would like to present our technical views again in this contribution because the rationale of the issue is also quite related to other discussions on core requirements such as beam sweeping factor reduction for RLM, BFD, etc.

Issue 1-4-3: L1-RSRP measurement accuracy requirements It was agreed to discuss in performance part

In RRM requirements and test cases, up to Rel-17, there is no 2 AoA based measurement accuracy requirement. As can be seen from Figure 1, when there are two incident angles, one from desired signal and the other from undesired signal, different UEs may consider different criteria for Rx beam training and beam codebook selection, hence, different RSRP measurement results. For instance, the criteria can be RSRP or SINR. The same issue will arise in group-based L1-RSRP measurement for FR2 mTRP, as shown in Figure 2. Note that while training one Rx beam toward TRP1 based on a CMR resource set associated with the TRP, the other CMR resource set associated with TRP2 can be treated as IMR from UE implementation perspective, and vice versa. Besides, when UE measures group-based L1-RSRP, there can be four different methods depending on the UE implementation.

- Figure 3-A1: CMR processing per best-mating UE antenna module when UE Rx beam pair is selected to maximize composite SINR of both CMRs

- Figure 3-A2: CMR processing with both of UE beams when UE Rx beam pair is selected to maximize composite SINR of both CMRs

- Figure 3-B1: CMR processing per best-mating UE antenna module when UE Rx beam pair is selected to maximize composite RSRP of both CMRs

- Figure 3-B2: CMR processing with both of UE beams when UE Rx beam pair is selected to maximize composite RSRP of both CMRs

As all these are left to UE implementation and nothing in spec forces UE to use a specific criterion and method, it would not be feasible to define L1 measurement accuracy requirements.

Fig. 1 L1 measurement and report when AoA of CMR is different from IMR

Fig. 2 Group based measurement and report when multiple L1 measurement resources are configured

Fig. 3-A1 CMR processing per best-mating UE antenna module when UE Rx beam pair is selected to maximize composite SINR of both CMRs

Fig. 3-A2 CMR processing with both of UE beams when UE Rx beam pair is selected to maximize composite SINR of both CMRs

Fig. 3-B1 CMR processing per best-mating UE antenna module when UE Rx beam pair is selected to maximize composite RSRP of both CMRs

Fig. 3-B2 CMR processing with both of UE beams when UE Rx beam pair is selected to maximize composite RSRP of both CMRs

Fig. 3 Four different UE implementations for Group-based L1-RSRP measurements

Observation 1: UE Rx beam management and TRP beam selection for L1 measurements and reports are left to UE implementation.

- gNB Tx beam selection for L1 measurement report

  - According to RAN1 specification, a selection of L1 measurement results among configured multiple measurement results is up to UE implementation.

- UE Rx beam selection for L1 measurements

  - UE may consider multiple criteria when selecting measurement results for the report, and it is also up to UE Rx beam codebook design.

- UE L1 measurement for a measurement resource with simultaneously formed two beams

  - When UE processes one measurement resource from a TRP, the UE may measure L1-RSRP received from one Rx beam or two Rx beams, which is up to UE implementation

Proposal 1: RAN4 to not assume the simultaneously formed beams are also optimal for other general purposes, e.g. the chosen beams may not maximize serving cell signal strength/quality, therefore RLM and BFD/CBD may not always be performed by using the chosen simultaneous beams particularly when the UE is in a situation where the radio link issues have to be more aggressively evaluated.

L1 Measurement Applicability

It is natural to assume that measurement resources for group-based L1 measurement should be selected and configured based on L3 measurement and report on the resources. Otherwise, UE may end up not being able to even detect the configured group-based L1 measurement resources after trying the entire codebooks for dual beams.

Proposal 2: Group-based L1 measurement period requirements are applicable only when a valid L3 measurement report associated with the L1 measurement resources was sent during the last [5] seconds

L1 Measurement Period Requirements

When UE forms two concurrent Rx beams, UE may be able to finish beam sweepings for SSB-based L1-RSRP measurement within the half of the total number of the required single Rx beam based sweepings. Figure 4 shows simultaneous dual Rx beam based L1-RSRP measurements on 4 SSB burst outside SMTC window. However, this may not be always doable, or UE may decide not to if certain conditions are not met. For instance, UE may not enable dual Rx beams if UE is not in mTRP based simultaneous reception mode or not in a good SNR regime. From NW signaling and configurations, there are ways that UE can start preparing dual Rx based measurement and reception from two TRPs, which may be different depending on sDCI or mDCI mode. Note that, as explained earlier, UE capable of dual Rx based L1 measurements may not perform RLM and CBD/CBD based on the dual beams because the optimal Rx beam for RLM and BFD/CBD may or may not be the same or sub-set of the dual Rx beams.

Fig. 4 L3- and L1-RSRP Measurements when SMTC is configured

Proposal 3: A UE Rx beam sweeping factor of N during SSB-based L1 measurements can be lowered to N/2 only if any one of the following conditions is met:

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB for each TRP, where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

- The above reduced beam sweeping factor can be subject to a new UE capability

- The above reduced beam sweeping factor does not apply to RLM and LRL (BFD and CBD)

# Conclusions

In this paper, we proposed the following.

UE simultaneous Rx beams for mTRP support and UE Rx beam for L1 measurement/evaluation

Observation 1: UE Rx beam management and TRP beam selection for L1 measurements and reports are left to UE implementation.

- gNB Tx beam selection for L1 measurement report

  - According to RAN1 specification, a selection of L1 measurement results among configured multiple measurement results is up to UE implementation.

- UE Rx beam selection for L1 measurements

  - UE may consider multiple criteria when selecting measurement results for the report, and it is also up to UE Rx beam codebook design.

- UE L1 measurement for a measurement resource with simultaneously formed two beams

  - When UE processes one measurement resource from a TRP, the UE may measure L1-RSRP received from one Rx beam or two Rx beams, which is up to UE implementation

Proposal 1: RAN4 to not assume the simultaneously formed beams are also optimal for other general purposes, e.g. the chosen beams may not maximize serving cell signal strength/quality, therefore RLM and BFD/CBD may not always be performed by using the chosen simultaneous beams particularly when the UE is in a situation where the radio link issues have to be more aggressively evaluated.

L1 Measurement Applicability

Proposal 2: Group-based L1 measurement period requirements are applicable only when a valid L3 measurement report associated with the L1 measurement resources was sent during the last [5] seconds

L1 Measurement Period Requirements

Proposal 3: A UE Rx beam sweeping factor of N during SSB-based L1 measurements can be lowered to N/2 only if any one of the following conditions is met:

- UE is configured with active TCI states from two TRPs, and the association between the TCI states and the TRPs is explicitly known to the UE, i.e.

  - (single DCI based mTRP) at least one of the codepoints in the active TCI list for PDSCH includes two reference resources for qcl-TypeD from respective TRPs

  - (multi DCI based mTRP) two CORESETs QCL’ed with two reference resources for qcl-TypeD are configured

  - [SNR > XdB for each TRP, where rank > 2 is expected]

- Group-based L1-RSRP measurement is configured based on L3 measurements for the same measurement resources

- The above reduced beam sweeping factor can be subject to a new UE capability

- The above reduced beam sweeping factor does not apply to RLM and LRL (BFD and CBD)