3GPP TSG-RAN WG4 Meeting #107 	R4-2307319

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on above 10GHz NTN UEs

Agenda item:	8.27.4

Document for:	Discussion and Approval

Introduction

The NTN WI, as presented in [1], includes the following objectives related to NTN deployment in above 10GHz bands:

4.1.2 NR-NTN deployment in above 10 GHz bands The following assumptions are taken a baseline for this work: - GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered - ESIM scenarios for NGSO in Ka band are not considered in this WI. - Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study. - FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2 - The ITU-R harmonized Ka band will serve as reference - Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification. The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022. - Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4] - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4] - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4] - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4] - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4]. - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4] - Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4] - Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed.as follows [RAN4]: - time relationship related enhancement (e.g., K_offset) - subcarrier spacing for different UL/DL signals/channels - PRACH configuration index for FDD above 10 GHz.

In this contribution we present further considerations for the introduction of NTN deployment in above 10 GHz bands.

Discussion

During RAN4 #106, a Way-Forward agreed for system parameters were agreed [2]. This discussion takes as starting point the listed issues:

- Power classes (Deployment Scenarios)

- Protection and Additional protection requirements

- NTN UE satellite beam tracking

Power classes (Deployment Scenarios)

At RAN4 #106 and previous meetings UE type and deployment scenarios have been discussed related to NTN operation above 10 GHz. In specific Earth Stations In Motion (ESIM) and Very-Small-Aperture Terminal (VSAT) are the two main types discussed.

Currently in NR FR2 UE specification as given by TS 38.101-2 there are the assumptions on UE types as listed in TS 38.101-2 Table 6.2.1.0-1 below:

TS 38.101-2 Table 6.2.1.0-1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |
| 6 | High Speed Train Roof-Mounted UE |
| 7 | RedCap UE |
| Note: RedCap variants of non-RedCap UEs are not precluded |  |

For VSAT type devices the understanding is that the typical devices antenna ranges from 75 cm to 1.2 m in aperture and to are of the dish antenna type. This type of device is understood to be stationary and can to some extend be compared to a NR FR2 fixed-wireless-access (FWA) device.

VSAT type devices could be compared to a NR FR2 fixed-wireless-access (FWA) device.

For ESIM type of devices a sub-division is already considered by the co-existence study, as captured in the WF [6], to Land, Aeronautical or Maritime ESIMs. The three types have different properties but can to some extend be compared to a NR FR2 Vehicular UE.

ESIM  type devices could be compared to a NR FR2 Vehicular UE.

From the WID it is noted that the ESIMs also are referred to as a “mobile VSAT”, indicating that the assumptions for a VSAT of typical antenna assumptions also can be applied for the EISM. In other words, the antenna assumptions make it difficult to envision a handheld device similar to that of a e.g. a smartphone.

ESIM is also in the WID referred to as a “mobile VSAT”.

None of the in the WID targeted UE types for an NTN band above 10 GHz is assumed to be a handheld device.

At RAN4#106bis it was agreed that at least in Rel-18 handheld FR2-NTN UEs are out of scope.

At least in Rel-18 handheld FR2-NTN UEs are out of scope

Following the main differentiator between the two listed UE types in the WID and the description in the previous the key aspect is the mobility capability. From that notion at least two different NTN UE typed for operation above 10GHz needs to be defined as:

- Fixed FR2-NTN UE

- Mobile FR2-NTN UE

These two types of FR2-NTN UEs can be differentiated via power-class similar to the types listed in TS 38.101-2 Table 6.2.1.0-1: Assumption of UE Types and have individual attributes.

FR2-NTN UEs shall be separated into Fixed and Mobile types

It can then be further discussed if the Mobile FR2-NTN UEs need to be sub-divided corresponding to the three types being Land, Aeronautical or Maritime ESIMs. This may be needed due to potential allowed maximum out-put-power based on the outcome of the ongoing co-existence study.

For the Fixed FR2-NTN UEs the specification progress may be more straight forwards as the mobility aspect can be neglected while it is needed address for the Mobile FR2-NTN UEs.

Protection and Additional protection requirements

A key concept for a NR UE ensuring conformance to Protection and Additional protection requirements is founded in the Network Signalling (NS). The terrestrial network informs the UE via the NS which set of requirements given in the TS to follow to conform in the specific network. The concept of NS is also already included for NTN operation as a method for indicating applicable A-MPR as found in e.g. clause 6.2.3 of 38.101-5. However, since the UE location is not currently coupled by the network to the UE and potentially relevant NS, conformance cannot be guaranteed, why NS concept from terrestrial network can’t always be directly reused.

Relaying on the current Network Signalling (NS) framework for a NTN UE is not enough to ensure conformance to Protection and Additional protection requirements.

Currently some UEs may use location information to ensure conformance to requirements for a specific area/region. Since a NTN UE per RAN1 design shall be location aware the 3D location information can be a potential mechanism for the NTN UE to ensure conformance.

Another mechanism to ensure conformance to Protection and Additional protection requirements is avoidance of e.g. specific spectrum and/or the possibility to terminate transmissions based on either UE or Network reporting. The UE reporting can be location, transmission orientation, detected available terrestrial networks etc. and the BS reporting can be allowed operational band based on location and other elements now covered under the NS framework. The decision to avoid/terminate transmissions can be triggered both by the network and the UE. Currently the UEs choice to avoid/terminate transmissions to ensure conformance is not specified. It would be beneficial for RAN4 to investigate if some occasions of this UE autonomous behavior need to be specified.

RAN4 shall discuss how the Network Signalling (NS) framework can be extended to cover the needed aspects for NTN operation and if UE autonomous behavior needs to be specified.

NTN UE satellite beam tracking

During RAN4 #106, the Way-Forward agreed for UE RF [4] states that RAN4 shall discuss whether and how to define the requirements for beam tracking for an NTN UE operating operation above 10 GHz.

As a result, Nokia have provided further discussion on UE satellite beam tracking in [5].

Conclusion

In the paper, the following Observations and Proposals were made:

Observation 1: VSAT type devices could be compared to a NR FR2 fixed-wireless-access (FWA) device.

Observation 2: VSAT type devices could be compared to a NR FR2 Vehicular UE.

Observation 3: ESIM is also in the WID referred to as a “mobile VSAT”.

Observation 4: None of the in the WID targeted UE types for an NTN band above 10 GHz is assumed to be a handheld device.

Observation 5: At least in Rel-18 handheld FR2-NTN UEs are out of scope

Proposal 1: FR2-NTN UEs shall be separated into Fixed and Mobile types

Observation 6: Relaying on the current Network Signalling (NS) framework for a NTN UE is not enough to ensure conformance to Protection and Additional protection requirements.

Proposal 2: RAN4 shall discuss how the Network Signalling (NS) framework can be extended to cover the needed aspects for NTN operation and if UE autonomous behavior needs to be specified.

- References

- RP-223534, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2302998, WF for system parameters of above 10 GHz NTN band, Thales (Moderator)

- R4-2305925, WF for system parameters of above 10 GHz NTN band, Thales (Moderator)

- R4-2303533, WF on NTN UE RF requirements for Ka band, ZTE

- R4-2305065, On Beam Tracking Capabilities, Nokia

- R4-2302878, WF for above 10GHz NTN-TN co-existence study, Samsung