3GPP TSG-RAN WG4 Meeting # 107		R4-2307322

# Incheon, Korea, May 22 – May 26, 2023

Agenda item:	8.9.2.3

Source:	Apple

Title:	On other potential enhancement for FR2 SCell activation

Document for:	Discussion

# Introduction

As discussed in last RAN4#106 meeting, the WF of eFeRRM has been agreed in [1]. In this paper, we focus on the issue 3-1-1 in [1].

# Discussion on other enhancement for FR2 SCell activation

Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation

The options we discussed in last meeting were:

Issue 3-1-1: Scheduling enhancement for FR2 unknown SCell activation FFS: - Option 1 (Apple, QC, OPPO, HW, Ericsson, ZTE): the early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed. - Option 1a(ZTE): - For FR2, the initial scheduling can be decided by the NW after receiving the beam info and signal quality info after L1-RSRP report. For FR1, once the signal quality info derived by NW, the initial scheduling is possible. - Option 2 (Nokia): - The potential to enable earlier data transmission within the activation period shall be studied. - The UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting. - Starting from receiving the TCI activation command, the UE shall report CSI, instead of OoR, if the UE has available uplink resources to report CQI for the SCell.

As we commented in last meeting, when the UE has DL timing and beam/TCI information, UE is able to be scheduled, and such early scheduling is not precluded in current spec. That means, before valid CQI reporting, if network wants to schedule this UE, it’s possible and allowed, but whether or not this UE will response or react correctly to this scheduling is up to the status at UE; and we believe even in R15, if network schedules this UE before valid CQI reporting, as long as UE is ready on timing and beam, UE will receive such scheduled data to increase the throughput and reduce latency. Bullet 1/2/3 in option 2 is up to network implementation and may not need to capture in the spec. in order to move forward, we propose to add one more bullet to reflect the early scheduling possibility for network implementation.

Proposal 1:

The early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

Whether UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting is up to network implementation.

# Conclusion

In this paper, we further discuss the other potential enhancement to shorten the FR2 SCell activation delay.

Proposal 1:

The early scheduling is network implementation and it’s not precluded in existing SCell activation requirement. No enhancement is needed.

Whether UE shall be scheduled by the network immediately after L3 or L1-RSRP reporting is up to network implementation.

# References

[1] R4-2306315, WF on R18 RRM enhancement - FR2 SCell activation delay reduction, Apple, RAN4 #106-bis-e