3GPP TSG-RAN WG4 Meeting #107												R4-2307348

Incheon, KR, May 22 – May 26, 2023

Agenda item:	8.8.3.3

Source:	Apple

Title:   	On RLM and BFD/CBD for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#106bis, some options related to RLM and BFD/CBD were included in the WF [1], copied below.

### Issue 2-2-2: Whether to enhance TRP specific CBD procedure

<Way forward >:

- Proposals

  - Option 1:

    - It is proposed to clarify if the issues as in R4-2304133 should be addressed to ensure system performance. Accordingly, the necessary requirements can be defined to ensure the expected UE behavior

  - Option 2:

    - RAN4 to discuss the motivation of CBD first when beam failure happens during simultaneous reception.

      - If UE fall back to single TRP, no CBD is needed.

      - If UE fall back to TDM based mTRP, legacy TRP specific requirement can be used as baseline. FFS any update.

      - If UE will continue simultaneous reception after CBD, the new TX beam needs to guarantee that the beam pair quality is good, i.e. will not broke the link of another panel. Requirement update is needed.

  - Option 3:

    - RAN4 to discuss what to do for TRP specific CBD procedure to avoid newly detected candidate beam from TRP#1 turns out cannot form a good beam pair with existed beam from TRP#2.

  - Option 4

    - RAN4 to not consider simultaneously formed multiple UE Rx beam based RLM and BFD/CBD on multiple resources from two TRPs.

In this contribution, we continue to discuss those issues.

# 2	Discussion

In R17, TRP-specific BFR is based on SCell BFR framework (using BFR MAC CE)

- Two beam failure detection (BFD) RS sets are configured

- Two candidate beam detection (CBD) RS sets are configured

In our understanding, the two BFD/CBD sets are independently operated by the UE. For example, when BFD takes place for one TRP, say TRP1, the UE will start CBD process and try to identify a new beam for TRP1. As such, there are some issues identified:

- The UE cannot simultaneously receive any two DL beams. When the UE starts to search for a new beam for TRP1 without regard to the existing beam for TRP2, a newly identified beam may not work with the existing beam for TRP2. To ensure the new beam can coexist with the existing beam for TRP2 from UE perspective, i.e., the UE can receive both simultaneously, should we revise the existing CBD beam selection criterion to consider beam coexistence? Also, to maximize the chance for the network to configure a new beam pair for the multi-RX capable UE to fully utilize its capability, early search of beam pair in case of beam failure instance is preferred. With such early search, by the time BFD for one TRP happens, the network can quickly schedule another suitable beam pair for the UE through TCI state switching.

- In case of single DCI (sDCI) is configured where DCI is transmitted from a single TRP, there is a motivation to inform the network of BFD immediately because 1) there is likely large delay (>100 slots) from the time that BFD is detected to the time CBD is completed, as shown in the CBD evaluation period below. 2) If DCI cannot be correctly received, the network cannot schedule the UE for DL reception or UL transmission although the link of TRP2 is still good. It is necessary to clearly specify UE behavior in case the UE cannot find a new beam through CBD for one TRP that can coexist with the existing beam for the other TRP.

As the issue has been discussed in RAN4 for multiple meeting, it is proposed that a decision is made at this meeting.

Proposal 1: It is proposed to make a decision if the above issues should be addressed to ensure system performance, and if the necessary requirements can be defined to ensure the expected UE behavior.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to make a decision if the above issues should be addressed to ensure system performance, and if the necessary requirements can be defined to ensure the expected UE behavior.

# 4	References

R4-2306318, " WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 1)," vivo