3GPP TSG-RAN WG4 Meeting #107                                                                                  R4-2307381

Incheon, KR, May 22 – May 26, 2023

Source: 	CATT

Title: 	Further discussion on remaining issues about simulation assumptions for above 10GHz NTN co-existence study

Agenda item:	8.27.2

Document for:	Approval

- Introduction

In RAN4#106-bis-e meeting, A WF [1] on simulation assumptions for above 10GHz NTN co-existence study was agreed. Some remaining issues in WF need further discussion in next meeting.

This contribution provides our views on remaining issues about simulation assumptions for above 10GHz NTN co-existence study.

- Discussion

2.1 System parameters

NTN SAN UL parameters – NF

As per the WF [1], the remaining open issue concerning NTN SAN UL NF is shown as below:

Issue 3-2-1: NTN SAN UL parameters – NF - Proposals - Option 1: 3.5 - Option 2: 5.9 - Option 3: Other values - Recommended WF - Pause the discussion by this meeting - Companies are encouraged to bring more technical analysis for this issue

Referring to recent state-of-the-art LNA-only noise figure publications in [91 to 96] of TR38.808[2], figure 2-1 presents the LNA-only noise figure for different semi-conductor technologies over frequency based on published data.

Figure 2-1: LNA-only NF trend over frequency

Considering the pre- and post-LNA losses from routing and other sources, apparently the noise figure should be greater than the LNA-only noise figure stated in Figure 2-1. For NTN SAN UL receiving, the frequency band is 27GHz, and referring to the trend curve in Figure 2-1, the LNA-only noise figure @30GHz reaches 3.5dB.  Therefore, 3.5dB(Option 1) is too small and ignores the pre- and post-LNA losses.

Referring to ETSI technical report TR 101 854 [3] on fixed radio point-to-point systems, the information on typical NF and associated industrial margins (IMF) values across wide range of frequencies is shown in Table 2-1. The  Noise Figure Industrial Margin (IMF) was carefully developed considering various performance variations of system elements, such as temperature extremes, voltage variations, or aging by capturing production spread of RF circuits.

Table 2-1: Typical Noise Figures (NF) and associated Industrial Margins (IMF)

From Table 2-1, the typical NF for frequency band 26 to 28GHz is about 7dB. Hence the 5.9dB (Option 2) is also too small. Therefore, we prefer that the NF for NTN SAN UL  to be 7dB.

Proposal 1: Adopt NTN SAN NF 7dB.

NTN UE parameters – NF

As per the WF [1], the remaining open issue concerning NTN UE NF is shown as below:

Issue 3-4-1: NTN UE parameters - NF - Proposals: - Option 1: 2.1 dB (baseline assumption) - Option 2: 5.9 dB - Option 3: Other values - Recommended WF - Further discuss the value for co-existence simulation.

With the same results as discussed above, for NTN UE DL receiving, the frequency band is 17GHz.  Referring to the trend curve in Figure 2-1, the LNA-only noise figure @20GHz reaches 2dB,  2.1dB(Option 1) is too small and ignores the pre- and post-LNA losses as well.

From Table 2-1, the typical NF for 18 to 23GHz is about 6dB, so we believe that the option 2(5.9dB) is more reasonable.

Proposal 2: Adopt option 2(5.9dB) for NTNUE NF.

- Conclusion

This contribution provides our general consideration on remaining issues about simulation assumptions for above 10GHz NTN co-existence study. The following proposals are concluded as follows:

Proposal 1: Adopt NTN SAN NF 7dB.

Proposal 2: Adopt option 2(5.9dB) for NTNUE NF.

- References

- R4-2306002, WF on [311] NR_NTN_enh_Part3, Samsung, RAN4#106-bis-e

- 3GPP TR 38.808 V17.0.0, Study on supporting NR from 52.6 GHz to 71 GHz.

- ETSI TR 101 854 V2.1.1 (2019-04), Fixed Radio Systems; Point-to-point equipment; Derivation of receiver interference parameters useful for planning fixed service point-to-point systems operating different equipment classes and/or capacities