3GPP TSG-RAN WG4 Meeting #107                                                                                   R4-2307382

Incheon, KR, May 22 – Mat 26, 2023

Source: 	CATT

Title: 	Further discussion on SAN RF requirements for above 10GHz bands

Agenda item:	8.27.3

Document for:	Approval

- Introduction

In RAN4#106 and 106-bis meeting, WFs [1, 3] on NTN enhancements SAN RF requirement were agreed. Some remaining issues in WF need further discussion in next meeting.

This contribution provides our views on remaining issues for SAN RF requirements for above 10GHz bands.

- Discussion

OTA reference sensitivity level

As per the WF [1], the remaining open issues concerning OTA Reference sensitivity level for SAN type 2-O are shown as below:

Issue 2-3-2: SAN OTA Rx Requirements - Agreement: OTA Rx requirement; Applicability notes. OTA Reference sensitivity level; While the overall requirement framework can be reused from the BS type 2-O in TS 38.104, the range of EIS values applicable to the NTN SAN deployment in FR2 requires further study..

It is agreed to reuse same approach for BS type 2-O from TN. EISREFSENS levels are derived from a single declared basis level EISREFSENS_50M, which is based on a reference measurement channel with 50 MHz SAN channel bandwidth. Referring to TR 38.817-2, EISREFSENS_50M can be calculated by the following,

EISREFSENS_50M = -174dBm+10*log10(BW) +IM+SNR+NF –G_Rx                                                                  (Equation 1)

Where,

BW = 66*12*60*1000.

IM =2.

SNR=-1.1dB.

NF and G for Ka-band NTN are discussed in the following.

So, using above parameters, equation 1 can be expressed as the following equation 2.

EISREFSENS_50M = -96.33dBm +NF –G_Rx                                                                                                          (Equation 2)

Referrng to our contribution[4],  we think that 7dB NF NF for GEO, LEO 600 and LE1200 for Ka-band is more reasonable.

Proposal 1:  Adopt NF in Table 2-1 for calculating Ka-band OTA REFSENS range.

Table 2-1: NTN NF for Ka-band

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| NF (dB) (30GHz) | [7] | [7] | [7] |

Referring to Set-1 satellite parameters for system level simulator calibration in Table 6.1.1.1-1 of TS38.821 as below:

Table 6.1.1.1-1: Set-1 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Referring to Set-1 satellite parameters for system level simulator calibration in Table 6.1.1.1-2 of TS38.821 as below:

Table 6.1.1.1-2: Set-2 satellite parameters for system level simulator calibration

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

The typical Satellite Rx max Gains range for GEO, LEO-1200, and LEO-600 are 50.5-58.5dBi, 30.5-38.5dBi, and 30.5-38.5dBi, respectively. Considering 10dB extension for above them, so the possible Satellite Rx max Gains range for GEO, LEO-1200, and LEO-600 are 40.5-68.5dBi, 20.5-48.5dBi, and 20.5-48.5dBi, respectively. Considering 3dB off peak margin, the G_Rx assumption for calculating Ka-band OTA REFSENS range for GEO, LEO-1200, and LEO-600 are 37.5-65.5dBi, 17.5-45.5dBi, and 17.5-45.5dBi, respectively as shown in Table 2-2.

Proposal 2: Adopt G_Rx assumption in Table 2-2 for calculating Ka-band OTA REFSENS range.

Table 2-2: G_Rx assumption for calculating Ka-band OTA REFSENS range

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G_Rx (dBi) | [37.5-65.5] | [17.5-45.5] | [17.5-45.5] |

Using NF proposed in Table 2-1 and G_Rx proposed in Table 2-2 and equation 2, the OTA reference sensitivity level for Ka-band for EISREFSENS_50M declaratiion can be claculated as following:

OTA reference sensitivity level for Ka-band for EISREFSENS_50M declaration is:

-	1 dB step within the range [-126 to -155] dBm dB for GEO

-	1 dB step within the range [-106 to -135] dBm for LEO 600

-	1 dB step within the range [-106 to -135] dBm dB for LEO 1200

Proposal 3: OTA reference sensitivity level for Ka-band for EISREFSENS_50M declaration is:

-	1 dB step within the range [-126 to -155] dBm dB for GEO

-	1 dB step within the range [-106 to -135] dBm for LEO 600

-	1 dB step within the range [-106 to -135] dBm dB for LEO 1200

OTA out-of-band blocking

As per the WF [3], the remaining open issues concerning OTA out-of-band blocking for SAN type 2-O are shown as below:

- Out-of-band blocking Way Forward - FFS how to specify OOB and ΔfOOB.

In RAN4#106 meeting [2], the NR operating bands in Ka-band are agreed as below:

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |

For above n512, 511, and n510 band, their FUL_high – FUL_low is less than 4000MHz, and their UL frequency range is in FR2-1 for BS type 2-O in TS 38.104, so as shown in Table 2-3, the ΔfOOB =1500MHz for FR2-1 in TS 38.104 can be reused for SAN type 2-O in Ka-band.

Proposal 4: Adopt ΔfOOB offset in Table 2-3 for satellite operating bands in Ka-band.

Table 2-3: ΔfOOB offset for satellite operating bands in Ka-band

| SAN type | Operating band characteristics (MHz) | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL_high – FUL_low ≤ 4000 | 1500 |

In TS 38.108, the -44 dBm was specified as interfering signal mean power (dBm)  for FR1 conducted out-of-band blocking, and 0.0029 V/m was specified as interfering signal RMS field-strength (V/m) for FR1 OTA out-of-band blocking. Referring to TR38.863, the aggressors are SpaceOps (Space Operations) TC (TeleCommand) stations on ground with band between 2025 and 2110.  The -44 dBm and 0.0029 V/m are derived from the worst case that SpaceOps TC station is at satellite Nadir (ground). For a LEO satellite at 600 km (LEO@600km), the free space losses between LEO@600km and SpaceOps TC station are 154 dB, and the Satellite Rx max Gain of LEO@600km is assumed as 30dBi, In TR 38.863, the maximum EIRPs of SpaceOps TC stations are 80dBW, we think that it is typo and should be 80dBm. So interfering signal mean power (dBm) received by LEO@600km is calculated as 80dBm-154dB+30dBi = -44dBm, the interfering signal RMS field-strength (V/m) is calculated as following.

E=30EIRP(W)d=30*10EIRPdBW10d=30*10EIRPdBm-3010d=30*1080-3010600*1000=0.0029V/m

For Ka-band of NTN, if SpaceOps TC stations within Ka-band are aggressors to Ka-band SAN, the maximum EIRP of Ka-band SpaceOps TC stations can be assumed as [60dBm], which refers to 4dBW/MHz with 200MHz CBW for LEO@600km in Ka-band to meet same distance transmission for Ka-band SpaceOps TC stations. So the interfering signal RMS field-strength (V/m) for Ka-band is calculated as following.

E=30EIRP(W)d=30*10EIRPdBW10d=30*10EIRPdBm-3010d=30*1060-3010600*1000=0.00029V/m

Referring to OTA out-of-band blocking for FR2-1 in TS 38.104, the 0.0029 V/m can be reused for 30 to 12750 MHz since the FR1 Ka-band SpaceOps TC stations is also existing as aggressor to Ka-band SAN. And the 0.00029 V/m can be used for 12750 to FUL,low – 1500, and FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band. So the OTA out-of-band blocking performance requirement for Ka-band is shown in Table 2-4.

Proposal 5: Adopt OTA out-of-band blocking performance requirement in Table 2-4 for Ka-band.

Table 2-4: OTA out-of-band blocking performance requirement for Ka-band

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of .interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.0029 | CW |
| 12750 to FUL,low – 1500 | EISREFSENS + 6 dB | [0.00029] | CW |
| FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | [0.00029] | CW |

- Conclusion

This contribution provides our general consideration on SAN RF requirement for above 10GHz bands. The following observations and proposals are concluded as follows:

Proposal 1:  Adopt NF in Table 2-1 for calculating Ka-band OTA REFSENS range.

Table 2-1: NTN NF for Ka-band

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| NF (dB) (30GHz) | [7] | [7] | [7] |

Proposal 2: Adopt G_Rx assumption in Table 2-2 for calculating Ka-band OTA REFSENS range.

Table 2-2: G_Rx assumption for calculating Ka-band OTA REFSENS range

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G_Rx (dBi) | [37.5-65.5] | [17.5-45.5] | [17.5-45.5] |

Proposal 3: OTA reference sensitivity level for Ka-band for EISREFSENS_50M declaration is:

-	1 dB step within the range [-126 to -155] dBm dB for GEO

-	1 dB step within the range [-106 to -135] dBm for LEO 600

-	1 dB step within the range [-106 to -135] dBm dB for LEO 1200

Proposal 4: Adopt ΔfOOB offset in Table 2-3 for satellite operating bands in Ka-band.

Table 2-3: ΔfOOB offset for satellite operating bands in Ka-band

| SAN type | Operating band characteristics (MHz) | ΔfOOB (MHz) |
| --- | --- | --- |
| SAN type 2-O | FUL_high – FUL_low ≤ 4000 | 1500 |

Proposal 5: Adopt OTA out-of-band blocking performance requirement in Table 2-4 for Ka-band.

Table 2-4: OTA out-of-band blocking performance requirement for Ka-band

| Frequency range of interfering signal (MHz) | Wanted signal mean power (dBm) | Interferer RMS field-strength (V/m) | Type of .interfering signal |
| --- | --- | --- | --- |
| 30 to 12750 | EISREFSENS + 6 dB | 0.0029 | CW |
| 12750 to FUL,low – 1500 | EISREFSENS + 6 dB | [0.00029] | CW |
| FUL,high + 1500 to 2nd harmonic of the upper frequency edge of the operating band | EISREFSENS + 6 dB | [0.00029] | CW |

- References

- R4-2302877, WF for SAN RF requirements on above 10 GHz, Moderator (THALES), RAN4#106

- R4-2302998, “WF for system parameters of above 10GHz NTN band”, RAN4#106, Moderator (THALES)

- R4-2305927, WF on SAN RF requirements – NTN Enhancements, Ericsson, RAN4#106-bis-e

- R4-2307381, Further discussion on remaining issues about simulation assumptions for above 10GHz NTN co-existence study, CATT, RAN4#107