3GPP TSG-RAN WG4 Meeting # 107                                                       R4- 2307388

# Incheon, KR, May 22 – May 26, 2023

Title: 	TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”

Source: 	CATT

Agenda item:	8.20.2.2.1

Document for:	Approval

Background

In WF[1], The TR section “10.2 Feasibility of FR1 Wide Area BS aspects” has been further broken-down, the same TR section break-down can be applied to Medium Range FR1 BS class, and FFS it can be applicable to Local Area FR1 BS. Template for capturing co-site inter-sector co-channel interference impact has been provided in WF[2]. For LA BS, we think there may not be so many different analysis.

Reference

[1] R4-2305916, “WF on implementation feasibility of SBFD”, RAN4#106bis-e, Samsung

[2] R4-2305917, “WF on implementation feasibility of SBFD: Co-channel Co-site/inter-site interference”, RAN4#106bis-e, Ericsson

[3] R4-2304431, TP for TR 38.858:“Feasibility of FR1 Local Area BS aspects”, RAN4#106bis-e, CATT

TP for TR 38.858

## 10.4 Feasibility of FR1 Local Area BS aspects

### 10.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

Using the analysis framework in 10.2.1, the following analysis can be made.

Table 10.4.1.2-1: SI feasibility study for FR1 Local Area BS

| Parameters |  |  |  | Calculation formula | Value/Comment |
| --- | --- | --- | --- | --- | --- |
| BS TX Power dB(P_TX ) = ① dBm |  |  |  |  | 24 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc |  |  | 45 |
|  |  | Frequency isolation techniques used |  |  | CFR、[DPD] |
|  |  | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc |  |  | 70 |
|  |  | Spatial isolation techniques used |  |  | TX/RX panel separation、 isolation structures、isolation material、cross polarization |
|  | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc |  |  |  | 0 |
|  |  |  |  |  | TX Beam nulling is not assumed due to the array size |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBm(P_(SI_leakage) ) (Note 1) |  |  | ①-②- ③- ⑨-⑦-10*log10(40/20) | -104.01 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band sub-band dB(α_(SI-RFIC) ) = ⑤ dBc |  |  | 0 |
|  |  | RF IC capability and other tech. in RX sub-band sub-band dB(α_(SI-RFIC) ) = ⑧ dBc |  |  | 0 |
|  |  | RF IC techniques and other tech.(before LNA) |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  | 0 |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBm(P_(SI_TXSB) ) (Note 1) |  |  | ①-③-④ | -46 |
|  | Blocker Suppression at RX | Frequency isolation capability dB(α_(CSSI-frequency,RX) )=⑥dBc |  |  | 55 |
|  |  | Frequency isolation techniques |  |  | Digital filter for ACS |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  | -16 |
|  |  |  | Rx IM3 contribution (dBm) | (①-③-④)-2*(IIP3-(①-③-④)) | -106 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | No other significant impacts |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBm(P_(SI_Selectivity) )(Note 1, 2) |  |  | ①-③-④-⑥ | -101 |
|  | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc |  |  |  | 0 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |
|  | Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  |  |  | 10 |
| Overall RSIC capability dB(RSIC) (Note 1) |  |  |  | ①-(P_(SI_leakage)+Rx IM3+Other RX +P_(SI_Selectivity)) calculate with power linear value | 122.33 |
| Noise floor ⑩dBm |  |  |  | -174+ 10*LOG10(20*10^6)+NF | -87.99 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  |  | -93.99 |
| Required RSIC budget (①-⑪dBc) |  |  |  |  | 117.99 |
| SBFD configuration |  |  |  |  | 40-20-40 |
| Guardband assumption (if exist) |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. |  |  |  |  |  |
| Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. |  |  |  |  |  |
| Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |

For FR1 LA BS SI analysis, the following assumptions are used.

Table 10.4.1.2.1-1: SI analysis assumptions for Local area BS

| Parameters | Local Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width | 40MHz*2 |
| UL subband width | 20MHz |
| Tx output power over whole channel | 24dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 13dB |

In SI capability analysis, the following techniques are used,

- Tx antennas panel and Rx antennas panel are separate, there are also some isolation material between them, and cross polarization is also used.

- Rx signals include wanted signal and interference signals. Interference signals far from Rx operating frequency range can be rejected by Rx RF and IF filters.DL-subband and UL-subband are adjacent, RF filters except metal cavity filter are difficult to reject the adjacent interference signals. At the same time, the filter also needs to be changed when subband configuration changes. Tx RF filters have the same performance as Rx RF filter.

- Digital filter is used to resolve the adjacent subband( i.e. TX subband )interference issue.

From the analysis in Table 10.4.1.2-1, the following capabilities are needed,

- Frequency isolation capability of frequency isolation at TX is 45dB

- Spatial isolation is 70dB

- ACS is 55dBc

- IIP3 is -16dBm

The above capabilities except frequency isolation are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

### 10.4.2	Co-channel inter-sub-band inter-site interference analysis

Co-channel inter-subband inter-site interference analysis is similar with SI analysis, analysis assumptions, interference leakage in gNB RX subband due to non-ideal TX and Rx path contribution are same with SI, but antenna isolation is replaced by spatial isolation and Digital IC for improving Tx non-ideal leakage is impossible for inter-site BS.

Table 10.4. 2-1: Co-channel inter-subband inter-site interference feasibility study for Local Area BS

| Item |  | Parameters and calculations | example |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) ① | 24 |
|  |  | SBFD configuration | DUD |
|  |  | UL subband width (MHz) ② | 20 |
|  |  | DL subband width (MHz) ③ (Note1) | 40 |
|  |  | Guard band assumption |  |
|  |  | BS NF (dB) ④ | 13 |
| Tx leakage contribution |  | Tx Frequency isolation capability (dB) ⑤ | 45 |
|  |  | DL subband RF filter suppression capability (dB) ⑥ | 0 |
|  |  | Spatial isolation capability (dB) ⑦ | 71 |
|  |  | Tx Beam nulling/isolation capability (dB) ⑧ | NA |
|  |  | DL EIRP impact due to beam nulling in TX sub-band |  |
|  |  | Digital IC for Tx leakage capability (dB) ⑨ | 0 |
|  |  | Overall RSIC for Tx leakage (dB) ⑩ |  |
|  |  | ⑩=⑤+⑥+⑦+⑧+⑨ | 116 |
|  |  | Tx leakage contribution to inter-site BS(dBm/UL subband) ⑪ |  |
|  |  | ⑪=①-⑩-10*log10(③/ ②) | -95.01 |
| Rx path noise contribution | Blocking analysis | Rx beam nulling/isolation capability (dB) ⑫ | 0 |
|  |  | RX sensitivity degradation caused by RX beam nulling |  |
|  |  | UL subband RF filter suppression capability (dB) ⑬ | 0 |
|  |  | Overall RSIC for blocking capability (dB) ⑭ |  |
|  |  | ⑭=⑦+⑫+⑬ | 71 |
|  |  | Tx SB interference signal level at Rx path (dBm) ⑮ |  |
|  |  | ⑮=①-⑭ | -47 |
|  | Rx IMD | Rx IIP3 capability (dBm) ⑯ | -16 |
|  |  | Rx IM3 contribution (dBm) ⑰ |  |
|  |  | ⑰=⑮-2*(⑯-⑮) | -109 |
|  | Rx ACS | ACS capability (dB) ⑱ | 55 |
|  |  | Rx ACS contribution (dBm) ⑲ |  |
|  |  | ⑲=⑮-⑱ | -102 |
|  |  | Any other RX impacts if significant (dBm) ⑳(Note 2) |  |
|  | Rx path overall noise contribution (dBm) ㉑ ㉑=10*log10(10⑰10+10⑲10+10⑳10) |  | -101.20 |
| Total inter-site contribution (dBm) ㉒ ㉒=10*log10(10⑪10+10㉑10) |  |  | -94.07 |
| Noise floor (dBm/UL subband) ㉓ ㉓=-174+10*log10②*106+④ |  |  | -87.99 |
| Overall REFSENS degradation (dB) ㉔ ㉔=10*log1010㉒10+10㉓10-㉓ |  |  | 0.96 |
| Note 1：③ is single DL subband width |  |  |  |

In the analysis in Table 10.4. 2-1, spatial isolation is 71dB which is corresponding to FSPL of the 21 meters It’s near the typical ISD of the indoor deployment scenario. Therefore, it’s feasible for inter-sited co-channel inter-subband scenario.

### 10.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.