3GPP TSG-RAN WG4 Meeting  # 107	R4-2307394

# Incheon, KR, May 22th – May 26th, 2023

Title: 	Discussion on mobility requirements for Rel-18 ATG

Source: 	CATT

Agenda item:	8.14.4.2

Document for:	Discussion

- Introduction

In RAN4#106 meeting, a WF on NR ATG RRM core requirements was approved and many issues were discussed and the current states have been summarized in it [1].

This document will further discuss these issues for ATG mobility requirements for ATG RRM requirements and present our views and proposals.

- Discussion

The conclusions on ATG mobility requirements in the last meeting are extracted as following, and we further provide our discussion and proposals.

  - Mobility in RRC_IDLE/INACTIVE

Issue 2-1-1-1: Cell re-selection mechanism

- Option 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

  - If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

  - If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

- Option 4:

  - If ATG UE altitude higher than certain threshold, ATG UE perform measurement

  - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement

- Option 5: No need to specify distance, altitude or speed-based enhancements to cell re-selection mechanism during Release 18.

  - If needed, such mechanisms may be discussed in future releases.

Agreements:

Candidate options for FFS:

- Option 1

- Option 1+4

- Option 5

Considering additional enhanced cell re-selection mechanisms will increase UE complexity and the workload, we can support not to support specify distance, altitude or speed-based enhancements to cell re-selection mechanism at least during Release 18, if needed, such mechanisms may be discussed in future releases.

In order to help UE to trigger neighbour cell measurement in time, we also can agree to introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement, and we think the mechanism as NTN could be reused, both RSRP and distance should be considered.

Proposal 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

- If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

- If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

Issue 2-1-1-4: Cell re-selection measurement requirements

Agreements:

- Keep previous agreements for number of carriers to be monitored.

- For intra-frequency measurement, apply legacy R15 cell-reselection requirement

- For inter-frequency measurement,

  - Define two set of requirements for ATG

    - Set 1: legacy R15 cell-reselection requirement

    - Set 2: R16 HST cell-reselection requirement

    - Details of signalling, associated UE capabilities and how to switch between the two sets of requirements are FFS

    - For set 2, FFS whether to use only R17 HST cell detection delays or measurement and evaluation delays as well.

Issue 2-1-1-5: Signalling for Cell re-selection measurement requirements

- FFS: Whether optional UE capability for enhanced requirements should be introduced for ATG network

For set 2, we prefer to use all of R17 HST cell-reselection requirement included cell detection delay, cell measurement delay and cell evaluation delay not only R17 HST cell detection delays or measurement and evaluation delays [2].

For UE capability, we agree to introduce an optional UE capability of numbers of monitored carrier. The Rel-15 NR requirements on intra-frequency and inter-frequency measurement can be reused for ATG. Whether to switch to another set of cell re-selection requirements is related to the capability.

Proposal 2: For inter-frequency measurement,

  - Define two set of requirements for ATG

    - Set 1: legacy R15 cell-reselection requirement

    - Set 2: R17 HST cell-reselection requirement

Proposal 3: Optional UE capability for enhanced requirements should be introduced for ATG network.

Issue 2-1-1-6: How to switch between the two sets of cell re-selection requirements

- Option 1: If UE is configured with [NW configuration flag for enhanced requirements], and the UE supports [capability for enhanced requirements], the R16 HST cell-reselection requirement should be applied. For other cases, the legacy R15 cell-reselection requirement should be applied.

- Option 2: network indicates which requirements apply to a particular cell via system information

  - For a particular cell, network can indicate Set 1(Legacy R15 cell-reselection requirement) or set 2 (R16 HST cell-reselection requirement) based on the ISD information and DRX configuration.

For the method on switching between the two sets of cell re-selection requirements, we support option 2. We think this issue is about the case of more faster measurement is needed. It is reasonable for NW to indicate via system information according the known ISD information and DRX configuration to the UE in IDLE/INACTIVE mode.

Proposal 4: Network indicates which requirements apply to a particular cell via system information

- For a particular cell, network can indicate Set 1(Legacy R15 cell-reselection requirement) or set 2 (R16 HST cell-reselection requirement) based on the ISD information and DRX configuration.

  - Mobility in RRC_CONNECTED

Issue 2-2-1: location-based CHO mechanism

- Option 1: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition)

- Option 2: Location based CHO is supported for A2G in Rel-18, and details of requirements are FFS

- Option 3: Using BS location information and UE location information, CHO can be used for UE located on the blind area of ATG BS.

- Option 4: Not to specify location-based CHO, such mechanism can be specified in future releases

If location-based CHO for ATG is agreed to introduce, we support option 1 and believe that the procedure and signalling from R17 NTN (combines RSRP and location based triggering condition) could be reused.

In this mechanism, locations refer to the reference location of the serving cell and the candidate target cell and the UE location. RAN4 have already agreed that ATG UE is capable of GNSS, and ATG BS could send the reference location by PositionVelocity-r17 as defined in TS 38.331 [3]. The UE will specifically trigger CHO based on the relationship between its distance from the reference location of the serving cell and the candidate target cell and the corresponding threshold.

Proposal 5: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition).

- Conclusion

In this paper, we provide our views on ATG mobility requirements. From this discussion we have derived the following proposals:

Proposal 1: Introduce distance-based triggering as an additional condition for intra- and inter-frequency cell measurement as NTN, whether and how to configure the feature is up to network implementation.

- If Srxlev > S(non)IntraSearchP and Squal > S(non)IntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter/intra-frequency layers of higher priority.

- If Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter/intra-frequency layers of higher, equal or lower priority in preparation for possible reselection.

Proposal 2: For inter-frequency measurement,

  - Define two set of requirements for ATG

    - Set 1: legacy R15 cell-reselection requirement

    - Set 2: R17 HST cell-reselection requirement

Proposal 3: Optional UE capability for enhanced requirements should be introduced for ATG network.

Proposal 4: Network indicates which requirements apply to a particular cell via system information

- For a particular cell, network can indicate Set 1(Legacy R15 cell-reselection requirement) or set 2 (R16 HST cell-reselection requirement) based on the ISD information and DRX configuration.

Proposal 5: For the location-based CHO, reusing the procedure in R17 NTN for R18 ATG (combines RSRP and location based triggering condition).

- Reference

- R4-2306395 WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2306249 Topic summary for [106-bis-e][218] NR_Mob_enh2_part1, MediaTek Inc.