3GPP TSG-RAN WG4 Meeting #107                                                       R4-2307438
Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.33.2

Source:	Vodafone

# Title:	System parameters for the new NR FDD bands (1400+700)

Document for:	Approval

# Introduction

The new WID on FDD Bands using the uplink from n28 and the downlink of n75 and n76was approved in RAN#99 in [1] to specify the following new NR FDD frequency bands:

a)	Band nA: 703 – 748 MHz UL / 1432 – 1517 MHz DL

b)	Band nB: 703 – 748 MHz UL / 1427 – 1432 MHz DL

In this contribution we initiate discussion on RAN4 requirements for these new NR FDD frequency bands.

# Discussion

In this section we discuss channel bandwidth and subcarrier spacings for the new FDD bands to be specified under the new work item in [1].

The DL frequencies of the new FDD bands are same as the SDL bands n75 and n76. The UL frequencies of the new FDD bands are same as the UL frequencies of the existing FDD band n28. Therefore, possible channel bandwidths and subcarrier spacings for the DL signals on these new FDD bands could be assumed to be the same as the ones for the existing NR SDL bands n75 and n76 or their subsets. Similarly, for the UL signals the channel bandwidth and subcarrier options could be assumed be the same as n28. Table 1 below shows the existing channel bandwidths (CBW) and subcarrier spacing (SCS) for the NR bands n75, n76, and n28.

Table 1: UE CBW and SCS for n75, n76 and n28

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n75 (1432 MHz – 1517 MHz) | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
| n76 (1427 MHz – 1432 MHz) | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| n28 (UL: 703 MHz – 748 MHz) | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

From the table above we can see that channel bandwidth and subcarrier spacing options do not fully match between the DL and UL frequencies.

In the following table we have combined what channel bandwidth and subcarrier spacings would be supported for the DL or UL of these new bands if the current CBW and SCS were directly copied from the existing bands.  ‘YES’ in the table indicates that both DL and UL support the given CBW and SCS. ‘DL’ or ‘UL’ is used when only DL or UL support the given CBW and SCS.

Table 2: Combined UE CBW and SCSs for the new FDD bands based on the n75, n76 and n28 CBW and SCS support.

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 60 |  | DL | DL | DL | DL | DL | DL | DL |  |  |  |  |
| Band nB: 1427 – 1432 MHz DL / 703 – 748 MHz UL | 15 | Yes | UL | UL | UL | UL | UL |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

After excluding different SCS cases, the CBW and SCS table for the new FDD bands looks as follows:

Table 3: CBWs and SCSs for the new FDD bands

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| Band nB: 1427 – 1432 MHz DL / 703 – 748 MHz UL | 15 | Yes | UL | UL | UL | UL | UL |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

The remaining open item is to decide if UL and DL CBW options in Table 3 should all be supported for the new FDD bands.

Proposal 1:

- Confirm CBW and SCSs marked with ‘YES’ for the new FDD bands based on the Table 3 in this document.

- Decide if UL and DL CBW options in Table 3 should all be supported for the new FDD bands.

# Conclusions

In this contribution we have discussed channel bandwidth, subcarrier spacings and variable duplex spacing for the following new NR FDD bands:

a)	Band nA: 703 – 748 MHz UL / 1432 – 1517 MHz DL

b)	Band nB: 703 – 748 MHz UL / 1427 – 1432 MHz DL

Based on our analyses we make the following proposal:

Proposal 1:

- Confirm CBW and SCSs marked with ‘YES’ for the new FDD bands based on the Table 3 in this document.

- Decide if UL and DL CBW options in Table 3 should all be supported for the new FDD bands.

Table 3: CBWs and SCSs for the new FDD bands

|  | NR band / SCS / UE CBW |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Band nA: 1432 – 1517 MHz DL / 703 – 748 MHz UL | 15 | Yes | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | DL | DL |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |
| Band nB: 1427 – 1432 MHz DL / 703 – 748 MHz UL | 15 | Yes | UL | UL | UL | UL | UL |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |

# References

- RP-230791, new FDD Bands using the uplink from n28 and the downlink of n75 and n76, RAN#99 meeting, March 2023.