3GPP TSG-RAN WG4 Meeting #107                                                       R4-2307439
Incheon, KR, May 22 – May 26, 2023

Agenda item:	7.33.3

Source:	Vodafone

# Title:	UE RF requirements analysis for the new NR FDD bands (1400+700)

Document for:	Approval

# Introduction

The new WID on FDD Bands using the uplink from n28 and the downlink of n75 and n76 was approved in RAN#99 in [1] to specify the following new NR FDD frequency bands:

a)	Band nA: 703 – 748 MHz UL / 1432 – 1517 MHz DL

b)	Band nB: 703 – 748 MHz UL / 1427 – 1432 MHz DL

In this contribution we initiate discussion on RAN4 UE requirements for these new NR FDD frequency bands. TX-RX separation and asymmetric channel bandwidth are discussed in this contribution.

# Discussion

TX-RX frequency separation

Independently licensed UL and DL spectrum needs to be taken into account for these new bands. This requires specifying new Tx-Rx separation for each band. The Tx-Rx separation approach for n91, n92, n93 and n94 bands can be adopted here. From Table 5.4.4-1 in [2], The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL).

Following such approach, TX-RX separation can be defined for the new bands as given in the following table.

| NR Operating Band | UL Frequency Range (MHz) | DL Frequency Range (MHz) | TX – RX carrier centre frequency separation |
| --- | --- | --- | --- |
| nA | 703 – 748 | 1432 - 1517 | 689 MHz - 809 MHz (μ = 0) 694 MHz - 804 MHz (μ = 1) |
| nB | 703 – 748 | 1427 - 1432 | 684 MHz – 724 MHz |

Proposal 1: TX-RX frequency separation for the proposed bands is the following:

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| nA | 689 MHz - 809 MHz (μ = 0) 694 MHz - 804 MHz (μ = 1) |
| nB | 684 MHz – 724 MHz |

The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL).

Asymmetric Channel BW

Proposal 2: Asymmetric channel BW operation should be supported with the below combinations:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| nA | 5 | 10, 15, 20, 25, 30, 40, 50 |
|  | 10 | 15, 20, 25, 30, 40, 50 |
|  | 15 | 20, 25, 30, 40, 50 |
|  | 20 | 25, 30, 40, 50 |
|  | 25 | 30, 40, 50 |
|  | 30 | 40, 50 |
| nB | 10 | 5 |
|  | 15 | 5 |
|  | 20 | 5 |
|  | 25 | 5 |
|  | 30 | 5 |

# Conclusion

This contribution initiates the discussion on the UE requirements for the new bands with the below proposals.

Proposal 1: TX-RX frequency separation for the proposed bands is the following:

| NR Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| nA | 689 MHz - 809 MHz (μ = 0) 694 MHz - 804 MHz (μ = 1) |
| nB | 684 MHz – 724 MHz |

The range of TX-RX frequency separation given paired UL and DL channel bandwidths BWUL and BWDL is given by the respective lower and upper limit FDL_low – FUL_high + 0.5(BWDL + BWUL) and FDL_high – FUL_low – 0.5(BWDL + BWUL).

Proposal 2: Asymmetric channel BW operation should be supported with below combinations:

| NR Operating Band | Channel BWs for UL (MHz) | Channel BWs for DL (MHz) |
| --- | --- | --- |
| nA | 5 | 10, 15, 20, 25, 30, 40, 50 |
|  | 10 | 5, 15, 20, 25, 30, 40, 50 |
|  | 15 | 5, 10, 20, 25, 30, 40, 50 |
|  | 20 | 5, 10, 15, 25, 30, 40, 50 |
|  | 25 | 5, 10, 15, 20, 30, 40, 50 |
|  | 30 | 5, 10, 15, 20, 25, 40, 50 |
| nB | 10 | 5 |
|  | 15 | 5 |
|  | 20 | 5 |

# References

- RP-230791, new FDD Bands using the uplink from n28 and the downlink of n75 and n76, RAN#99 meeting, March 2023.

- TS 38.101-1