3GPP TSG-RAN WG4 Meeting #107	R4-2307456

# Incheon, KR, May 22-26, 2023

Title: 	Discussion on reply LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

Source: 	vivo

Agenda item:	10.1.2

Document for:	Discussion

Introduction

RAN4 received LS [1] from RAN2 on clarifications on monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs based on specification text in RAN2 and relevant sections in RAN1 and RAN4 and the detailed LS is copied below.

Current RAN2 specifications do not explicitly specify what happens for UEs in half duplex mode if a paging occasion conflicts with a CG-SDT occasion. It is RAN2’s understanding that although information pertaining to this can be found in e.g., 38.213, clause 17.2 or in 38.133, clause 5.1B.2.6, the UE is only required to monitor paging for SI change indication in any paging occasion at least once per modification period during SDT if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. Similar to connected mode behaviour, since the UE is only required to monitor the paging in any paging occasion at least once per modification period, there should be other paging occasions available (within the modification period) to monitor the paging for SI change even if some of them overlap with the CG-SDT occasion(s). Hence, RAN2 would like to ask RAN1 and RAN4 to take the above understanding into account and discuss possible amendment on misalignment between RAN2 specifications and RAN1 and/or RAN4 specifications.

In this contribution we provide our views on the issue in RAN2’s LS.

Discussion

- The related part of RAN4 specification is copied below:

------------------------

- TS38.133 section 5.1B.2.6 Maximum interruption in paging reception

The requirements in clause 4.2B.2.6 shall apply for RedCap UEs.

For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.

------------------------

The related part of RAN1 specification is:

------------------------------

TS 38.213 section 17.2

A HD-UE does not expect to receive both a Type-0/0A/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols.

The RAN2 status to our understanding:

During SDT, the UE doesn't monitor normal paging, it only monitors paging for SI change notification (for ETWS/CMAS). This is only done in any paging occasion once per modification period.

Based on our understanding on RAN2’s status the scenario where a paging occasion overlaps with CG-SDT transmission will not appear hence in order to align RAN1/2/4 specification, the corresponding part “For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.” should be deleted.

Proposal 1: To align RAN1/2/4 specification, delete the corresponding part “For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.”

Conclusions

In this paper we provided our views on the question in the LS [1] and have the following proposal:

Proposal 1: To align RAN1/2/4 specification, delete the corresponding part “For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission.”

Reference

[1] R2-2304562, LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs, RAN2

- Annex: draft reply LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs

3GPP TSG-RAN WG4 Meeting #107 R4-22xxxxx Incheon, KR, May 22-26, 2023 Title: Reply LS on Monitoring of paging occasions for CG-SDT with HD-FDD Redcap UEs Response to: R2-2304562 Release: Release 17 Work Item: NR_SmallData_INACTIVE-Core Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: xusheng wei - E-mail Address: xusheng.wei@vivo.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the questions in LS R2-2304562. Based on discussions in RAN4#107, RAN4 would like to inform that in order to align RAN1/2/4 specification, RAN4 decides to delete the following part “For RedCap UE in HD-FDD mode, if a paging occasion overlaps with CG-SDT transmission then the UE shall monitor the paging during the paging occasion. In this case the UE is allowed to drop the CG-SDT transmission” from section 5.1B.2.6 of TS38.133 RAN4 respectfully asks RAN2 to take the above information into account. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #108 Aug 21 – Aug 25, 2023 Toulouse, France RAN WG4 Meeting #108bis Oct.09 – Oct. 13, 2023 Xiamen, China