3GPP TSG-RAN WG4 Meeting #107 	R4-2307468

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Advanced Receivers - Test parameters

Agenda item:	8.19.1.2

Document for:	Discussion

Introduction

In RAN4#106bis several agreements were made on the topic of configuration settings for advanced receivers Phase I simulation (see WF [1]).

In this contribution we will discuss the remaining open issues and make new proposals if needed.

Discussion

The number of co-scheduled UEs

In RAN4#106bis it was discussed if the phase I study should cover 2 co-scheduled UEs. The following was captured in the WF [1]:

Issue 2-1-1: The number of co-scheduled UEs Candidate options: - Option 1: 1 co-scheduled UE as higher priority - Option 2: Cover 2 co-scheduled UEs at least in phase I study Way forward - Keep the previous meeting agreement, and continue discussion in the next meeting.

When UE is performing demodulation with interference, it is the number of layers with interference which are important. Seen from a genie perspective, UE can do R-ML or E-IRC in the layers where the required parameters for the selected type of receiver is known. For layers where no/not enough information is available we assume the UE will select standard IRC receiver.

We do not see the need in phase I to include covering 2 co-scheduled UEs.

Cover 1 co-scheduled UE as high priority.

Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs

In RAN4#106bis the rank allocation for target and co-scheduled UEs with 2 co-scheduled UEs were discussed (see [1]):

Issue 2-1-3: Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs Candidate options: - Option 1: Rank 1+1+1 Way forward - Maintain the last meeting agreement: Option 1 for interested companies to bring evaluation results in initial study stage

As already indicated in section 2.1 we do not see a need to include 2 co-scheduled UEs in the Phase I study. In case it is decided to include 2 co-scheduled UEs in Phase I study, we are fine with using option 1: Rank 1+1+1.

If 2 co-scheduled UEs are introduced in Phase I study, we see Option 1: Rank 1+1+1 to be a valid configuration.

MCS for the target UE

In RAN4#106 it was agreed to cover MCS13 and MCS18 for initial simulations (see WF [1]). The question of in addition covering MCS4 for rank 1 was kept open.

Issue 2-1-4: MCS for the target UE Way forward - Cover MCS 13 for rank 1 and rank 2 for initial simulation - Cover MCS 17 for rank 2 for initial simulation - Further discuss whether to cover MCS 4 for rank 1 in the next meeting - The assumption can be updated later based on available results.

In our simulation results for target UE at MCS4, we see both R-ML and E-IRC performs better than baseline [2]. In addition, R-ML clearly performs better than E-IRC (2.6dB). As the advanced receiver is also required to work with low SNR conditions, including MCS4 would be a good choice. We also see setting the target UE at MCS4 and interference UE at QPSK represents configuration at cell edge and should be included in phase I study.

Using MCS4 for target UE will secure requirement definitions for low SNR which is a valid scenario that represent both target UE and interference UE at cell edge.

Cover MCS4 for rank 1 for initial simulation.

PDSCH resource allocation for the target and co-scheduled UE

In RAN4#106bis it was discussed if partial transmission bandwidth configuration should be included in the Phase I study. The following was captured in the WF [1]:

Issue 2-1-9: PDSCH resource allocation for the target and co-scheduled UE Candidate options: - Candidate scenarios: - Scenario 1: Maximum transmission bandwidth configuration for all UEs. - Scenario 2: Maximum transmission bandwidth configuration for the target UE and partial transmission bandwidth configuration for the co-scheduled UEs. - Full OFDM symbol allocation for both scenarios. - Option 1: Cover both scenario 1 and scenario 2 in the phase I study - Option 1A: Consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information - Target UE with frequency domain full PRB allocation (52PRBs) - Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission. - Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission. - Full OFDM symbol allocation. - Option 1B: Allocation; Layer 1; Layer 2; Layer 3; Layer 4. PRB 0 ~ PRB19; Target UE; UE3 64QAM; UE1 16QAM. PRB20 ~ PRB39; None. PRB40 ~ PRB51; UE2 QPSK. - Option 1C - Target UE with frequency domain full PRB allocation (52PRBs) - Co-scheduled UE1 with PRB0~26 allocation. - Option 2: Only consider scenario 1 - Option 3: FFS based on the conclusion for Issue 1-2-2-8 Way forward - For initial simulation in phase I, cover scenario 1 - FFS whether to cover scenario 2, based on the discussion outcome in the NWA part.

In our contribution for receiver assumption in [3] Issue 1-2-2-8, we have shown, that it is possible to perform aided blind detection of FDRA for scenario 2 (option 1C). 
Option 1A and Option 1B includes more than 1 co-scheduled UE and it is our proposal (see section 2.1) to prioritize only 1 UE. As Option 1A and 1B from above requires 2 co-scheduled UE, we do not see option 1A and 1B to have priority.

Aided blind detection of FDRA can be done with good performance when using option 1C: Target UE with frequency domain full PRB allocation (52PRBs) and Co-scheduled UE1 with PRB0~26 allocation.

Option 1A and option 1B are related to more than one co-scheduled UE and we do not see more than one co-scheduled UE as a priority configuration for Phase I.

Cover “Maximum transmission bandwidth configuration for all UEs” (Scenario 1). Cover “Target UE with frequency domain full PRB allocation (52PRBs) and Co-scheduled UE1 with PRB0~26 allocation.” (Scenario 2, option 1C)

Conclusion

This paper presents Nokia's views on various open issues with relation to test parameters for advanced receivers

In the paper, the following Observations and Proposals were made:

- We do not see the need in phase I to include covering 2 co-scheduled UEs.

- Cover 1 co-scheduled UE as high priority.

If 2 co-scheduled UEs are introduced in Phase I study, we see Option 1: Rank 1+1+1 to be a valid configuration.

Using MCS4 for target UE will secure requirement definitions for low SNR which is a valid scenario that represent both target UE and interference UE at cell edge.

Cover MCS4 for rank 1 for initial simulation.

Aided blind detection of FDRA can be done with good performance when using option 1C: Target UE with frequency domain full PRB allocation (52PRBs) and Co-scheduled UE1 with PRB0~26 allocation.

Option 1A and option 1B are related to more than one co-scheduled UE and we do not see more than one co-scheduled UE as a priority configuration for Phase I.

Cover “Maximum transmission bandwidth configuration for all UEs” (Scenario 1). Cover “Target UE with frequency domain full PRB allocation (52PRBs) and Co-scheduled UE1 with PRB0~26 allocation.” (Scenario 2, option 1C)

- References

- R4-2305914 WF on advanced receiver for MU-MIMO scenario

- R4-2307469- Advanced Receivers - Simulation results

- R4-2307466- On Advanced Receivers - Receiver assumption