3GPP TSG-RAN WG4 Meeting #107 	R4-2307471

Incheon, Korea, May 22 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Absolute physical layer throughput requirements with link adaptation

Agenda item:	8.19.2

Document for:	Discussion

Introduction

In RAN4#106bis several agreements were made on the topic of absolute physical layer throughput requirements with link adaptation (see WF [1]).

In this contribution we will discuss the remaining open issues and make new proposals if needed.

Discussion

Phy Layer TP requirement specification

In RAN4#106bis the simulation results from companies were analyzed. Based on the analysis and requirement for Gspan <=2.5dB a number of candidate testpoints were collected. It was not yet considered if removal of outlier results could provide opportunity to include additional candidate testpoints to the list. Also, the additional margin to add to impaired results were not decided (see WF [1]):

Issue 2-1-1: Test point T (%) selection - Candidate test pool based on alignment simulation results in R4-2304257 Test Case; Candidate test point 1: T % (SNR, Gspan); Candidate test point 2: T % (SNR, Gspan). FR1 FDD 2x2; 10% (3.2 dB, 1.6 dB) 15% (6.5 dB, 2.1 dB); 40% (18.5 dB, 2.4 dB). FR1 TDD 2x2; 10% (3.6 dB, 1.9 dB) 15% (7.1 dB, 1.8 dB); 35% (17.5 dB, 2.4 dB). FR1 FDD 2x4; 15% (3.3 dB, 1.9 dB); 50% (15.2 dB, 2.5 dB). FR1 TDD 2x4; 15% (3.6 dB, 2.2 dB); 50% (15.4 dB, 2.8 dB) 35% (10.7 dB, 2.3 dB). FR2-1 (TDD 2x2); 15% (4.4 dB, 2.1 dB); 40% (15.1 dB, 1.8 dB). - Issue A. # of test point(s) - Option 1. Two test points for 2Rx and 4 Rx. - Option 2. Two test points for 2Rx and 1 test point for 4 Rx. - Option 3. Single test point for 2Rx and 4 Rx - Issue B. T(%) and outlier removal for Gspan <= 2.5 dB Issue 2-1-2: X dB margin - Option 1: Apply X dB margin per modulation order to impairment results X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM Note that the modulation order would be based on median value over companies’ own median statistics on CQI. - Option 2: Apply single X dB margin to impairment results (e.g. X=0.5 dB or else) WF for Issue 2-1-1 and 2-1-2 - Discuss in the next meeting with the following aspect based on the updated results if any. - SNR options considering uniqueness of test SNR coverage - T (%) based on alignment results considering Gspan - Final SNR with X dB margin is within range of [0 20] for FR1 and [0 16] for FR2. It does not preclude the possibility of T(%) adjustment with [+- 5% steps] or removal of outlier results from Gspan perspective.

Test point T (%) selection

Based on the simulation results provided by companies in RAN4#106bis-e we have good alignment inside a span of 2.5dB for all cases enabling definition of requirements. We do see, that for one case (FR1 TDD 2x4, testpoint 2 from above) only 35% throughput can be achieved with <=2.5dB span.
As 35% throughput is not high for an FR1 ATP test, it shall be considered to remove outliers which would enable 55% throughput requirements for FR1 2x4 both TDD and FDD (testpoint 2). We do not see an issue with removing outliers, as there are 9 companies providing results and it cannot be expected to reach <=2.5dB alignment with that high number of companies providing results.

A high number (9) companies have provided simulation results. With so many companies providing results outlier results can be removed until <= 2.5dB is reached.

When considering the candidate test points, remove outlier results until <=2.5dB span is achieved.

Assuming removing outlier results to reach <=2.5dB leads us to the following proposal:

Define ATP requirements with the following testpoints:

| Test Case | test point 1: T % (SNR, Gspan) | test point 2: T % (SNR, Gspan) |
| --- | --- | --- |
| FR1 FDD 2x2 | 15% (6.5 dB, 2.1 dB) | 35% (16.5 dB, 2.5dB) [one outlier removed] |
| FR1 TDD 2x2 | 15% (7.1 dB, 1.8 dB) | 35% (17.5 dB, 2.4 dB) |
| FR1 FDD 2x4 | 15% (3.3 dB, 1.9 dB) | 55% (16.5dB, 2.3dB) (one outlier removed) |
| FR1 TDD 2x4 | 15% (3.6 dB, 2.2 dB) | 55% (16.8dB, 2.0dB) (two outliers removed) |
| FR2-1 (TDD 2x2) | 15% (4.4 dB, 2.1 dB) | 40% (15.1 dB, 1.8 dB) |

X dB margin

We see option 1 is using the margins normally used by RAN4 for requirement definition. This leads us to the following proposal:

Apply X dB margin per modulation order to impairment results; X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM ; X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM

Applicability and release independence

In RAN4#106bis it was discussed if ATP requirements should only apply for Rel-18 UEs. No decision was made and the following was captured in the WF [1]:

Issue 2-2: Applicability and release independence - Option 1: The requirement with link adaptation should be applicable for all NR UEs without any new applicability rules, and the requirement should be release independent from Rel-15 - Option 1.a: Proposal considering declaration - Optional for Rel-15 and Rel-16 UEs based on declaration - Mandatory for all Rel-17 and forward UEs.. - Option 2: The absolute physical layer throughput requirement with link adaptation should be applicable from Rel-18 and not release independent from Rel-15.

Assuming companies’ results are Rel-17 compliant, it is possible for Rel-17 UEs to comply to the RAN4 defined ATP requirements, hence we do not see a reason to not make the requirements mandatory for all Rel-17 and forward UEs. In addition, we also expect most Rel-15/Rel-16 UEs would be capable of passing the requirements, hence there is no need to explicitly remove Rel-15/Rel-16 UEs from declaring applicability.

We see Rel-17 UEs as being capable of supporting the requirements defined in this WID. In addition, we understand that some Rel-15/Rel-16 UEs might not support the requirements. We see option 1.a as a good compromise enabling Rel-15/Rel-16 UEs to declare compliance and having compliance mandatory from Rel-17 and onwards.

For applicability and release independence we support option 1.a:
	- Optional for Rel-15 and Rel-16 UEs based on declaration 
	- Mandatory for all Rel-17 and forward UEs.

DraftCR for FR2-1 ATP requirements

We have provided a DraftCR for FR2-1 ATP (see [2]) requirements in this meeting. During the drafting, we have the following observations and proposals

Void sections

According to drafting rules, it is not allowed to introduce void sections.

RAN4 to decide if void sections should be introduced to keep same structure as for existing sections.

FRC section

We have identified that there is a need for a new FRC section as none of the existing FRC sections support link adaptation. We have provided our proposal for a new FRC section for FR2-1 ATP requirements in our DraftCR.

Existing FRC sections in 38.101-4 are defined with fixed MCS. This is not useable for link adaptation.

Introduce new FRC sections to support link adaptation.

Re-transmission

Regarding how to handle re-transmission, there is a number of agreements in RAN4#106bis (see [1]) which shall be captured in the configuration information for the tests. We see two places to add such information:

- Provide the information as notes in the parameter section

- Provide the information as notes in the FRC section.

The requirement specification needs to contain the directives on how to handle the re-transmission cases.

Include the information on how the test shall handle the re-transmission as notes in new FRC section.

Conclusion

In this contribution we have provided Nokia's view on the remaining open issues for Application Layer Throughput with link adaptation.

In the paper, the following Observations and Proposals were made:

Test point T (%) selection

- A high number (9) companies have provided simulation results. With so many companies providing results outlier results can be removed until <= 2.5dB is reached

- When considering the candidate test points, remove outlier results until <=2.5dB span is achieved.

- Define ATP requirements with the following testpoints:

| Test Case | test point 1: T % (SNR, Gspan) | test point 2: T % (SNR, Gspan) |
| --- | --- | --- |
| FR1 FDD 2x2 | 15% (6.5 dB, 2.1 dB) | 35% (16.5 dB, 2.5dB) [one outlier removed] |
| FR1 TDD 2x2 | 15% (7.1 dB, 1.8 dB) | 35% (17.5 dB, 2.4 dB) |
| FR1 FDD 2x4 | 15% (3.3 dB, 1.9 dB) | 55% (16.5dB, 2.3dB) (one outlier removed) |
| FR1 TDD 2x4 | 15% (3.6 dB, 2.2 dB) | 55% (16.8dB, 2.0dB) (two outliers removed) |
| FR2-1 (TDD 2x2) | 15% (4.4 dB, 2.1 dB) | 40% (15.1 dB, 1.8 dB) |

X dB margin

Apply X dB margin per modulation order to impairment results; X = [0.5] dB for QPSK, X = [0.5] dB for 16QAM ; X = [0.8] dB for 64QAM, X = [0.8] dB for 256QAM

Applicability and release independence

We see Rel-17 UEs as being capable of supporting the requirements defined in this WID. In addition, we understand that some Rel-15/Rel-16 UEs might not support the requirements. We see option 1.a as a good compromise enabling Rel-15/Rel-16 UEs to declare compliance and having compliance mandatory from Rel-17 and onwards.

For applicability and release independence we support option 1.a:
	- Optional for Rel-15 and Rel-16 UEs based on declaration 
	- Mandatory for all Rel-17 and forward UEs.

DraftCR for FR2-1 ATP requirements

According to drafting rules, it is not allowed to introduce void sections.

RAN4 to decide if void sections should be introduced to keep same structure as for existing sections.

Existing FRC sections in 38.101-4 are defined with fixed MCS. This is not useable for link adaptation.

Introduce new FRC sections to support link adaptation.

The requirement specification needs to contain the directives on how to handle the re-transmission cases.

Include the information on how the test shall handle the re-transmission as notes in new FRC section.

- References

- R4-2305915 - WF on absolute physical layer throughput requirement

- R4-2307941 - DraftCR on ATP Requirements for FR2-1