3GPP TSG-RAN WG4 Meeting #107	R4-2307493

# Incheon, South Korea, 22 – 26 May, 2023

Source: 	NEC

Title: 	TP to TR 38.877: Corrections to NR mmWave MB-BS TR

Agenda Item:	8.2.2

Document for:	Approval

Introduction.

This contribution provides text proposals to correct the text in TR38.877.

Discussion

In the last meetings, RAN4 achieved good progress to create the technical report for study on NR mmWave MB-BS. However, we still see some issues to be corrected.

Issues include,

- In the previous meeting the term “fractional bandwidth” in the TR was replace by “percentage bandwidth” [1] to avoid confusion because the term is defined differently in other documents. However, we still see “fractional bandwidth” in the current TR because it was included in the approved TP in the previous meeting. We need to replace “fractional bandwidth” again.

- A table of the percentage bandwidth of FR2-1 operating band(s) has numerical errors

- Some editorial corrections, including table and figure number corrections are required

Text proposal is provided below. TP is on [2] provided by the rapporteur.

References

[1] R4-2305881, TP to TR 38.877: Fractional bandwidth and percentage bandwidth, NEC

[2] R4-230xxxx, TR 38.877 v0.4.0

Text proposal

--------------Start of text proposal-------------

### 4.2.1	Architecture options

Figure 4.2.1-1 depicts potential architectures based on the implementation options available for multi-band support in FR2-1 BS. The main components included are the PA, antenna array, and diplexers (wherever needed). Note that there are no active RF components before the RF input in any of the options in the figure.

Figure 4.2.1-1:  High-level architecture options to support multi-band operation in FR2-1

From the options captured in the figure, Option #2 and Option #4 are the target scenarios of this study item, while no study is needed for Option #5. For Option #1 and Option #3, if the PA is capable of being operated in multiple bands at the same time, then it supports multi-band operation. However, if the PA can only be configured and operated in a single-band, then it is considered a single-band RIB.

Considerations for architecture options

- Antenna array

As detailed in Clause 5.3.1, either a multi-band antenna array (broadband or with multi-resonances) or separate antenna arrays with lower percentage bandwidth can be used. In comparison to separate antenna arrays, a consolidated multi-band design is more compact and reduces costs, but trades performance aspects that can be optimized for in separate designs. On the other hand, having separate designs enables dedicated optimizations, yielding better performance. This comes at the cost of significantly larger circuit area being used for two designs and additional integration losses from lines and transitions.

- PA and component placement

Choosing where to place components is an important part of a multi-band solution as it impacts the overall performance. For instance, placing the diplexer before the PA is preferred from a design loss and power dissipation perspective. This is because the insertion loss of the diplexer occurs at a lower power level and can be accommodated by a simple increase in driver gain. Conversely, if the diplexer is placed after the PA, the insertion loss of the diplexer directly reduces the power available to the antenna array and leads to more thermal dissipation. As shown in Option #4, this diplexer can be avoided by using a multi-band antenna, such as a single input stacked patch design.

Dedicated PAs for the desired frequency ranges may be adopted from a design loss and power dissipation perspective, as the narrower band designs will have better efficiency and the thermal dissipation will be spread out over a larger area.

- Feasibility

Option #1 and Option #3 are feasible with current technology if the ranges covered by each path have a percentage bandwidth up to 19.5%. Per agreements in sub-Clause 5.2.1.4, Option #2 and Option #4 should also be feasible for a percentage bandwidth up to 19.5%. Note that even within the same frequency group, the performance attained would yield a lower Tx power compared to a single-band design. However, given that this requirement is a manufacturer declaration, it would not impact the requirement applicability.

5	Feasibility study

5.1	General

According to the definition in section 6.1, a multi-band FR2-1 BS is a BS that transmits in two or more FR2-1 bands using common active components. In this section, the feasibility of an FR2-1 multi-band BS is considered. The feasibility is examined taking into account technology challenges and emerging solutions. It is possible that not all parts of the BS are multi-band but that some advantage can be gained by implementing some parts as multi-band. For example one potential advantage of a multiband PA might be that power can be distributed between the supported bands offering more flexibility to optimize the power available for each, this requires only a multi-band PA.

The evaluation considers whether the proposed set of FR2-1 requirements is likely to be achievable with present or future implementation possibilities. The study does not aim to conclude on whether a multi-band FR2 solution is more optimal or effective (considering complexity, power, weight etc.) than other possibilities, for example mounting individual radio panels for each band within a BS enclosure.

When discussing the feasibility of wideband components/systems the multi-band bandwidth as a percentage of the carrier frequency should be considered.

Note that this is generally called fractional BW but in [2]the term fractional BW is already defined to mean something specific so is best avoided to prevent confusion.

Fractional bandwidth (FBW) is defined for a single operating band as shown below:

Fractional bandwidth (FBW) is defined in [2] as FBW=200∙FFBWhigh-FFBWlowFFBWhigh+FFBWlow%,

where FFBWhigh is highest supported frequency within supported operating band, for which fractional bandwidth support was declared, and FFBWlow is lowest supported frequency within supported operating band, for which fractional bandwidth support was declared.

Multiple operating bands are discussed in this study so the existing definition of fractional bandwidth is not suitable, the term percentage bandwidth is therefore used and defined as follows:

Percentage bandwidth (PBW) is defined in this study as PBW=200∙FPBWhigh-FPBWlowFPBWhigh+FPBWlow%,

where FPBWhigh is highest supported frequency, for which percentage bandwidth support was declared, and FPBWlow is lowest supported frequency, for which percentage bandwidth support was declared.

Table 5.1-1 shows the percentage bandwidth for the FR2-1 bands and various potential multi-band groups.

Table 5.1-1:Percentage band width of FR2-1 operating band (s)

| Frequency group, NR operating band pairing (lowest, highest) | Frequency range | Percentage bandwidth |
| --- | --- | --- |
| 26 GHz (n258) | 24250 MHz – 27500 MHz | 12.6% |
| 28 GHz (n257) | 26500 MHz – 29500 MHz | 10.7% |
| 28 GHz (n261) | 27500 MHz – 28350 MHz | 3% |
| 39 GHz (n260) | 37000 MHz – 40000 MHz | 7.8% |
| 40 GHz (n259) | 39500 MHz – 43500 MHz | 9.6% |
| 48 GHz (n262) | 47200 MHz – 48200 MHz | 2.1% |
| 24-29 GHz (n257/n258/n261) | 24250 MHz –29500 MHz | 19.5% |
| 37-48 GHz (n260/n259/n262) | 37000 MHz – 48200 MHz | 26.3% |
| 26+28 GHz (n258, n261) | 24250 MHz – 28350 MHz | 15.6% |
| 28+39 GHz (n257/n261, n260) | 26500 MHz – 40000 MHz | 40.6% |
| 26+40 GHz (n258, n259/n262) | 24250 MHz – 48200 MHz | 66.1% |
| 28+40 GHz (n257/n261, n259/n262) | 26500 MHz – 48200 MHz | 58.1% |

5.2	Wideband RF architectures

5.2.1	RF Front end

5.2.1.1	General

Feasibility for RF front ends can be considered by looking at existing component capabilities which could be used for multi-band FR2 products now but also by studying literature for the direction of future capabilities.

5.2.1.2	Beam former and PA

There are a number of components available today which offer sufficient band widths to cover at least some of the FR2 multi-band options. In [24] a wide band beamformer and PA was presented which covered 24 to 29.5 GHz (bands n257/n258/n261). The performance of which can be seen in table 5.2.1.2-1 and figure 5.2.1.2-1.

Table 5.2.1.2-1: beamformer IC in 24-29.5GHz

| Parameter | Beamformer |
| --- | --- |
| Frequency Range | 24-29.5 GHz |
| Tx Op1dB/OIP3 | 21 / 25 dBm |
| Tx Pdiss/Ch @ 12dBm Pout | 300 mW |
| Pout @3% EVM w/ 64QAM | 12 dBm |
| Rx Single Channel NF | 4 dB |
| Rx Pdiss | 130 mW |
| Instantaneous Bandwidth | 1600 MHz |

Figure 5.2.1.2-1: PA bias optimized for P1dB for 24GHz (left), and 28GHz (right)

It is considered feasible that multi-band beamformer IC covering 24-29.5GHz and associated bands can be achieved.

Looking further ahead the results of academic studies in [25] a 24-42 GHZ wideband PA was presented which exhibited flat P1dB of 17.8 to 19.6dBm, within 1.6dB from Psat, and flat PAEP1dB of 36.6 to 44.3% over 24 to 40GHz, verifying the truly wideband large-signal matching.

In [26] the hybrid N/PMOS allowed the 26-39GHz PA deep Class-AB biasing and device cascade, substantially increasing PA Pout and efficiency.

These PAs across different frequency groups are summarized in the following Table 5.2.1.2-2. It can be seen that some recent study shows PA covering 26-39GHz, 24-42GHz with average output power around 10dBm,PAEP1dB larger than 30%.

Table 5.2.1.2-2: PAs performance from literatures

| Frequency | [25] 24 -42GHz PA | [26] 26 -39GHz PA |
| --- | --- | --- |
| Authors and year of publication | Fei Wang,2020 | J Park,2022 |
| Technology | 45nm SOI CMOS | 45nm SOI CMOS |
| Gain | 20.5 | 18.9 |
| S21BW-3dB（GHz） | 25.8-43.4 | 25.3-42.0 |
| P1dBBW-1dB（GHz） | 22.0-37.0 | 25.0-37.0 |
| PAEp1dB BW-1dB（GHz） | 24.0-41.2 | 29.9-33.6 |
| P1dB（dBm） | 17.8-19.6 | 16.3-18.4 |
| Pavg（dBm） | 8.4-11.3 | 10.8-12.3 |
| PAEP1dB（%） | 36.6-44.3 | 29.9-34.9 |

These papers indicate that PAs covering frequency ranges from 26 to 40GHz or more are at least technically feasible in the research environment and may become available in the longer term.

It should be noted PAs with wide percentage bandwidths may not be capable of operating with signals that broad. For instance, while the PA may cover 50% bandwidth, its instantaneous bandwidth, i.e., maximum signal bandwidth, is usually restricted to a few hundred MHz due to limitations with bias networks and memory effects.

--------------Next text proposal-------------

5.2.3	Phase shifter

Analog phase shifters in mmWave BS are used to control the phase of signals in order to steer the beam. In some architectures, attenuators or variable-gain amplifiers are also placed along with the phase shifter to control the gain of beamformed signals to achieve desirable beamforming performance.

Current phase shifter designs are wideband and as such it would prove difficult to apply different phase shifts to different bands. Hence it is not possible to independently steer beams to different directions in the different bands, and it may even not be feasible to steer the beams in the same directions for to 2 separate bands if the bands are sufficiently far apart in frequency that the beamforming weights would need to be different for each band.

Examples of beam steering results when wide-band phase shifters and wideband phased array are used in a multiband transmitter are shown below. The phase shifters only control the beam weights of the lower frequency band. Two steering angles are considered, i.e., 0 and 20 degrees. Figure 5.2.3-1 illustrates the beam patterns when the element separation for lower band and upper band is 0.5λ and 0.6λ, respectively, which corresponds to the case of, for example, the combination n258+n261. Figure 5.2.3-2 presents the case when the operating bands are further apart, i.e., the element separation for lower band and upper band is 0.35λ and 0.65λ, respectively, which can be the case of combinations across different frequency groups. As can be seen, except when UEs of different bands are at the boresight of the array, otherwise the transmitter cannot steer the beam to multiple UEs independently. Even if different-band UEs are located at same direction (but not boresight), the beam for upper-band UEs is not pointed to its desired direction and a larger frequency separation of the bands results in bigger error of the steering angle.

Figure 5.2.3-1. Beam pattern of different band signals when using wideband phase shifter and antenna array. Phase shifters apply beam weights for the lower band. Array separation for lower band 0.5λ, and upper band 0.6 λ: (Left) 0-degree steering angle; (Right) 20-degree steering angle

Figure 5.2.3-2. Beam pattern of different band signals when using wideband phase shifter and antenna array. Phase shifters apply beam weights for the lower band. Array separation for lower band 0.35λ, and upper band 0.65λ: (Left) 0-degree steering angle; (Right) 20-degree steering angle.

Thus, it is not clear such a wide band phase shifter as shown in Figure 5.2.3-3 would be of use. Within the time frame of this SI it seems commercially available multi-band frequency selective phase shifters that can apply a different phase shift per band are unlikely to be available. However, any specification should not preclude a potential future architecture based on frequency dependent phase shifters. If and when frequency dependent phase shifters become viable, it is possible that RAN4 requirements created in Rel-18 would need re-visiting.

Figure 5.2.3-3. Example of multi-band phase shifter prior to a multi-band PA

In order to apply independent phase shift and hence independent steering to each band, phase shifters need to applied differently to each band.  Using technology available today it may be possible to use multiple single band phase shifters to provide the beam steering to each band independently whilst feeding into a multi-band PA shown in Figure 5.2.3-4, although this may have some performance penalty.

Note that Figures 5.2.3-3 and 5.2.3-4 show example architectures for a multi-band BS. Depending on the implementation, filter may or may not be placed after the multi-band PAs.

Figure 5.2.3-4. Example of single band phase shifter prior to a multi-band PA to steer each band separately

--------------Next text proposal-------------

6.1	Definition of FR2 multi-band BS

The existing definitions of single-band RIB and multi-band RIB for FR1 defined in TS 38.104 are shown as below:

multi-band RIB: operating band specific RIB associated with a transmitter or receiver that is characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band than the other carrier(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band.

single-band RIB: operating band specific RIB supporting operation either in a single operating band only, or in multiple operating bands but does not meet the conditions for a multi-band RIB.

From definition of multi-band RIB for FR1, the transmitter or receiver of multi-band RIB dependent BS have the capability to process two or more carriers from the different bands in common active RF components simultaneously. For FR2-1, the existing multi-band RIB definition for FR1 can be reused.

As the same with FR1, the existing explanations on BS capable of supporting operation in multiple operating bands in 38.104 [xx] for FR1 are sufficient. The same descriptions can be applied to FR2-1, i.e.

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands.

--------------End of text proposal-------------