3GPP TSG-RAN WG4 Meeting # 107		R4-2307592

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.14.4.5

Source: 	CMCC

Title: 			 Discussion on RRM measurement requirements for ATG

Document for:	Discussion

- Introduction

In RAN4#106bis-e meeting, ATG measurement procedure and measurement requirements were discussed, the WF has been approved in [1]. In this contribution, we continue the discussion about RRM measurement impaction on ATG.

- Measurement

Issue 5-1: Measurement mechanism

- Option 1:

  - If ATG UE altitude higher than certain threshold, ATG UE perform measurement and measurement report

  - If ATG UE altitude lower than certain threshold, ATG UE may not perform measurement and/or does not perform measurement report

- Option 2: Wait for the progress in RAN2 for the height-dependent configuration for measurements in connected mode for reusing the framework in ATG.

- Option 3: Do not introduce height-dependent requirements for measurement procedures for ATG UES

  - Option 3-1: All requirements are applicable for the ATG CPE which is not lower than 3km

- Option 4: De-prioritize new measurement mechanism

We don’t think altitude-based measurement mechanism is necessary for ATG scenario. Based on our acknowledge, there will be some administration requirements for the aircraft communication equipment, the aircraft communication equipment should be power off automatically when the altitude lower than certain threshold (e.g., 3km). Ericsson also provide similar observation in R4-2216399 that almost all airlines state 10,000ft≈3km as the altitude above which WiFi is activated according to various regulations.

Observation 1: the aircraft communication equipment should be power off automatically when the altitude lower than certain threshold (e.g., 3km).

Proposal 1: No need to introduce height-dependent requirements for measurement procedures for ATG UE.

Issue 5-2: Measurement requirement

- Option 1: Further discuss whether the assumption of a directional antenna affects the measurement requirements.

There are two main different features between ATG FR1 directional antenna and legacy FR2 directional antenna. First, the beam of FR1 directional antenna is wider than FR2, which is 25°. Second, UE could acquire BS location which could help UE to position the beam direction. UE could also use the flight route information to position the Rx beam direction of neighboring cell.

In summary, we think the beam management is up to UE implementation, the measurement delay will not be impacted by FR1 directional antenna.

Proposal 2: For measurement requirement and beam management, no need to consider the FR1 directional antenna impaction.

Issue 5-3: Scheduling restriction

- Option 1:

  - The legacy scheduling restriction requirement can be reused for ATG.

    - When it is enabled, the restriction is on SSB symbols and 1 data symbol before and after.

    - When it is disabled, the restriction is on all symbols in SMTC.

- Option 2:

  - When NW enables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SSB symbols to be measured. The dedicated symbol value can be indicated by NW.

  - When NW disables the deriveSSB-IndexFromCell, the UE is allowed to not transmit or receive data more than 1 symbols before and after SMTC to be measured. The dedicated symbol value can be indicated by NW.

- Option 3:

  - Legacy scheduling restriction to measure neighbor cell is not suitable, so RAN4 needs to decide how to define scheduling restriction considering

    - Alt 1: define scheduling restriction based on maximum ISD in ATG

    - Alt 2: define scheduling restriction based on the difference of propagation delay from serving cell and neighbor cells

For intra-frequency measurement, in the current specification, the scheduling restriction for ground-based network is as below:

If deriveSSB-IndexFromCell is enabled, then the scheduling restriction window is 1 data symbol before and after the consecutive SSB symbols to be measured within SMTC window duration.

If deriveSSB-IndexFromCell is not enabled, then the scheduling restriction window is whole SMTC window.

In last meeting, RAN4 has agreed that it is up to network to decide whether to enable ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not based on DL SCS and ISD for ATG network.

In ATG network:

If network enable ‘deriveSSB-IndexFromCell’, it means that network has the confidence that the frame of boundary alignment between serving cell and target cell could fulfill the requirement of min(2 SSB symbols, 1 PDSCH symbol). Therefore, the scheduling restriction window can also be set as the consecutive SSB symbols to be measured within SMTC window duration and 1 data symbol before and after.

If network not enable ‘deriveSSB-IndexFromCell’, then as we analysis in R4-2300782 [2], since the flight route is relatively fixed, the SSB of target cell to be measured will be always later than or before than the SSB of serving cell. The largest total time for SSB pattern and propagation time difference is 4.67ms, there is still 0.33ms margin (> 4 symbols) in SMTC duration. The current requirement is enough. Therefore, the scheduling restriction window can also be set as the whole SMTC window duration.

Figure 1. Total time for SSB pattern and propagation time difference.

Proposal 3: For ATG intra-frequency measurement scheduling restriction:

- When ‘deriveSSB-IndexFromCell’ is enabled, the scheduling restriction is on the consecutive SSB symbols/RSSI symbols to be measured  within SMTC window duration and 1 data symbol before and after.

- When ‘deriveSSB-IndexFromCell’ is not enabled, the scheduling restriction is on all symbols in SMTC.

For inter-frequency measurement without gap, the requirements can be applied provided that the timing of SSBs across serving cell and inter-frequency neighbor cells are aligned, therefore, for scheduling restriction, the legacy requirements can be reused.

Proposal 4: For ATG inter-frequency measurement without gap, the legacy scheduling restriction can be reused.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: the aircraft communication equipment should be power off automatically when the altitude lower than certain threshold (e.g., 3km).

Proposal 1: No need to introduce height-dependent requirements for measurement procedures for ATG UE

Proposal 2: For measurement requirement and beam management, no need to consider the FR1 directional antenna impaction.

Proposal 3: For ATG intra-frequency measurement scheduling restriction:

- When ‘deriveSSB-IndexFromCell’ is enabled, the scheduling restriction is on the consecutive SSB symbols/RSSI symbols to be measured  within SMTC window duration and 1 data symbol before and after.

- When ‘deriveSSB-IndexFromCell’ is not enabled, the scheduling restriction is on all symbols in SMTC.

Proposal 4: For ATG inter-frequency measurement without gap, the legacy scheduling restriction can be reused.

- Reference

[1] R4-2306344, WF on NR ATG RRM requirements, CMCC

[2] R4-2300782, Discussion on RRM measurement requirements for ATG, CMCC