3GPP TSG-RAN WG4 Meeting # 107		R4-2307593

Incheon, KR, May 22 – May 26, 2023

Agenda Item:	8.14.5.1

Source: 	CMCC

Title: 			 Discussion on general demodulation assumptions for ATG

Document for:	Discussion

- Introduction

In last meeting, the agreement for ATG demod scenario, UE pre-compensation assumption, and channel model have been achieved, the WF was approved in [1]. The agreements are as follows for information.

Issue 1-1: Scenario Agreement: - Focus on ATG deployment scenario which aligned with co-existence session, other scenarios (e.g., the scenario with large Doppler jumping in an ATG cell) can be considered in the future enhancement when needed. Issue 1-2: UE assumption Tentative agreements: - Take ATG UE pre-compensation on UL frequency shift and UL timing shift for ATG demodulation as baseline. - Take ATG UE pre-compensation on DL frequency shift for ATG demodulation as baseline. Issue 1-3-1: Channel model Agreement: - At least single path AWGN channel with doppler

In this meeting, we further provide our views on the doppler shift assumption.

- Channel model

Issue 1-3-2: Doppler shift assumption:

Agreement:

- Align with RF session agreement on frequency error requirements with below values as starting point

  - For FDD channel model, set Doppler as [220Hz]

  - For TDD channel model, set Doppler as [400/500Hz].

FFS:

- For FDD channel model,

  - Option 1: set Doppler as 220Hz

  - Option 2: Set Doppler as 200Hz for UL, 220Hz for DL

- For TDD channel model:

  - Option 1: set Doppler as 400Hz.

  - Option 2: set Doppler as 500Hz

In previous meetings, RAN4 has agreed to consider the UE frequency pre-compensation in both DL and UL, and 0.1ppm doppler assumption after pre-compensation was agreed.

We think the practical operating bands and worst cases should be considered. The operating FDD bands include n1and n3, TDD bands include n34, n41, n78 and n79. The highest frequency for FDD UL is 1980MHz, FDD DL is 2170MHz. Therefore, we support to set doppler as 200Hz for FDD UL, 220Hz for FDD DL. The highest frequency for TDD is 5000MHz. Therefore, we propose to consider doppler as 500Hz for TDD.

Proposal 1: For FDD channel model, set Doppler as 200Hz for UL, 220Hz for DL.

Proposal 2: For TDD channel model, set Doppler as 500Hz.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: For FDD channel model, set Doppler as 200Hz for UL, 220Hz for DL.

Proposal 2: For TDD channel model, set Doppler as 500Hz.

- Reference

[1] R4-2305899, WF for ATG demodulation requirements, CMCC