3GPP TSG-RAN WG4 Meeting # 107	R4-2307616

Incheon, KR, May 22 – May 26, 2023

Agenda Item: 8.31.3.3

Source: MediaTek Inc.

Title: Discussion on RRM impacts for LTE SL and NR SL coexistence

Document for: Discussion

- Introduction

RAN4#106bis-e had some discussion on the RRM requirements for LTE SL and NR SL coexistence [1]. There is still some open issue. In this contribution, we will share our views on the remaining issue.

- Discussion

In last meeting, RAN4 had some discussion on whether the existing RRM requirements can be reused for LTE SL and NR SL coexistence.

Issue 2-1-2: Whether to reuse existing RRM requirements for Co-channel coexistence for LTE SL and NR SL Candidate options: - Option 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning - Option 2: Wait for RAN1’s conclusion on co-channel coexistence solutions, e.g., for dynamic resource pool.

In R18 semi-static resource pool partitioning and dynamic resource sharing between LTE SL and NR SL are supported. Since LTE and NR SL UEs have different resource pools in semi-static resource sharing, R16/17 RAN4 requirements apply to semi-static resource sharing. For dynamic resource sharing, RAN1 assumed device type A (Device type A contains both LTE SL and NR SL modules), and the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission as shown in RAN1 working assumption and agreement. We see no impact on NR SL RRM core part requirements yet. It is also fine to us to wait for more progress in RAN1 to make the final conclusion in RAN4. As RAN1 is designing NR SL resource (re)selection procedure for dynamic resource sharing, RAN4 may need to specify new test cases on NR SL resource (re)selection procedure for dynamic resource sharing.

RAN1#110 Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. RAN1#110bis Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer.

Proposal 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning

Proposal 2: LTE SL and NR SL coexistence has no impact on RRM core part requirements yet. Also fine to wait for more progress in RAN1 to make the final conclusion in RAN4.

- Summary

In this paper, we have some discussion on LTE SL and NR SL coexistence. We have the following proposals:

Proposal 1: The existing RRM requirements are applicable for the co-channel coexistence solution of TDM-based semi-static resource pool partitioning

Proposal 2: LTE SL and NR SL coexistence has no impact on RRM core part requirements yet. Also fine to wait for more progress in RAN1 to make the final conclusion in RAN4.

4 Reference

- R4-2306364, WF on NR_SL_enh2_part2, OPPO, RAN4#106bis-e, April 17 – April 26, 2023.