3GPP TSG-RAN WG4 Meeting #107	R4-2307631

Incheon, Korea, 22th– 26th May 2023

Source:	China Telecom

Title:	Phase I study for the advanced receiver for MU-MIMO: simulation results

Agenda Item:	8.19.1.2

Document for:	Information

- Introduction

RAN4 #106bis-e meeting we discuss the advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. The WF is approved in [1] as the discussion outcome.

In this paper, our simulation results are provided based on the agreed assumptions in [1-2].

- Simulation Results

| Co-scheduled UE number | Rank for the target UE | Rank for the Co-scheduled UE | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | MIMO | Channel model | Precoder selection for the Co-scheduled UE | FDRA of the Co-UE | China Telecom |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | R-ML | E-IRC | IRC (baseline) | Gain of R-ML | Gain of E-IRC |
| 1 | 1 | 1 | 13 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | random | Full CHBW allocation (52PRBs) | 18.1 | 20.7 | 25.5 | 7.4 | 4.8 |
|  | 2 | 2 |  | 64QAM | 4Tx 4Rx ULA Low | TDLA30-10 | orthogonal |  | 12.5 | 12.9 | 13.5 | 1.0 | 0.6 |
|  |  |  |  | QPSK |  |  |  |  | 11.3 | 12.8 | 13.5 | 2.2 | 0.7 |
|  |  |  | 17 | 64QAM |  |  |  |  | 17.2 | 17.3 | 17.9 | 0.7 | 0.6 |
|  |  |  |  | QPSK |  |  |  |  |  | 17.3 | 17.8 |  | 0.5 |
| 2 | 1 | 1 for each Co-scheduled UE | 13 | 64QAM and QPSK | 4Tx 4Rx XPL medium | TDLC300-100 | random |  | 12.6 | 14.4 | 16.1 | 3.5 | 1.7 |
|  |  |  |  | 64QAM and 16QAM |  |  |  |  | 13.6 | 14.4 | 16 | 2.4 | 1.6 |

- References

- [1] R4-2305914, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106bis-e, April, 2023

- [2] R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106, March, 2023